| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K323 |
| --- | --- |

| Title: | CE 10: Signal Adaptive Diffusion Filters For Video Coding (Test 10.4.1-10.4.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jennifer Rasch, Jonathan Pfaff, Michael Schäfer, Anastasia Henkel, Heiko Schwarz, Mischa Siekmann, Martin Winken, Philipp Helle, Detlev Marpe, Thomas Wiegand Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany | Tel: Email: | +49 30 31002-727 Jennifer.Rasch@hhi.fraunhofer.de, Jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer Institute for Telecommunications – Heinrich Hertz Institute. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results on the tool under test in CE10 (JVET-J1030) for studying the performance of the signal adaptive diffusion filters proposed in JVET-J0038 on top of VTM and BMS. 
The diffusion filters are filters which may be applied to the prediction signal generated by a hybrid video codec. Two types of diffusion filters are proposed: Uniform and signal dependent diffusion filters. The uniform diffusion filters convolves the extended prediction signal n times using a symmetric filter mask. The signal dependent diffusion filters use the input prediction signal to identify structures of the underlying signal and diffuse along edges rather than perpendicular to them.

# Proposed method: Decoder algorithm description

Signal Adaptive Diffusion Filters For Video Coding have been integrated to the test model according to the description of sub-tests CE10.4.1-CE10.4.5 [1]. These filters were proposed in the document [2].

## General algorithm description

It is proposed that the diffusion filters are applied to the prediction signal pred generated by the underlying codec. The filter may be applied for intra as well as inter predictions. Two types of diffusion filters are proposed: Uniform and signal dependent diffusion filters.

### Uniform Diffusion Filter

Mathematically, the uniform diffusion filters are motivated by the following partial differential equation

∂∂tux,t=div∇ ux,t,

ux,0=pred(x),

where u denotes the filtered prediction and parameter t time which corresponds to iterations in a discrete setting. The uniform diffusion filters are implemented as a symmetric filter mask

h =    14 010101010

Note that whenever possible, all 2D filter masks are implemented as 1D filters. The filtered prediction un is calculated by correlating n times the prediction signal pred  with the filter mask h, i.e., by calculating

un=h*…*hn times*pred

Here * denotes the application of the filter mask to the image, also called correlation. At each correlation step, at the right and lower boundary symmetric boundary conditions are assumed while at the top and left boundary equality with the reconstructed samples is assumed as shown in Figure 1. If in inter case, it is not desired to use the reconstructed samples of the current frame, the boundary condition can be replaced by using reflected samples here as well (corresponds to the first test CE10-4.5, section 3.5).

prediction block

reconstructed samples

reflected samples

Figure 1 Extended prediction signal: the blue boundary denotes the boundary of reconstructed samples, the white block is the given prediction signal, the red boundary are the reflected samples.

It is proposed that the number n of iterations is equal to 5 or 15 iteration steps, in case of Inter and to 5 or 35 in case of Intra prediction signals as depicted in Table 3 below. The more iteration steps are carried out, the smoother the modified prediction will be. In the end, the filtered prediction is used as the modified prediction block.

### Signal dependent Diffusion Filter

While the uniform diffusion filters smooth edges and smoother areas alike, the idea of the signal dependent diffusion filters is to locate edges using the given prediction signal and reduce the magnitude of diffusion at those locations. The implementation of the signal dependent diffusion filters is motivated by the following signal dependent differential equation

∂∂tu=divfJρ∇predx ∇u

with initial and boundary conditions as described above. If in inter case, it is not desired to use the reconstructed samples of the current frame, the boundary condition can be replaced by using reflected samples here as well (corresponds to the first test CE10-4.5, section 3.5). Here, edges and their directions are detected  in terms of the gradient of the initial prediction signal. Therefore, the proposed signal dependent diffusion filter performs n correlations of the prediction signal pred with a mask h(x) that is different at each position x ϵR2 and is computed out of the prediction signal pred  as follows.

First, in order to identify edges, a Gaussian-smoothed diffusion tensor

Jρ∇pred  :=K* ∇pred  ∇pred T

is constructed, with a Gaussian kernel K. The discrete gradient ∇pred=EW, NST   is calculated using finite differences, namely by

EW≔ -000101000*pred   and   NS≔ 0100000-10*pred

using boundary conditions as described above. That means the gradient components EW and NS are given by simple differences of two sample values.

Then, for each sample location x ϵR2 inside the prediction block area, the 2x2 diffusion tensor is given by

4 ∇pred (x) ∇pred (x)T=EW(x)∙EWxEW(x)∙NSxEW(x)∙NSxNS(x)∙NSx ,

where the multiplication dot is understood to be pointwise. The Gaussian kernel is chosen as

K =    14 010121010

The Gaussian kernel is applied on the gradient matrices EW∙EW, EW∙NS and NS∙NS. This way, we receive a symmetric 2x2 matrix

Jρ∇pred (x)=14 gxxgxygxygyy(x)=14 ((K*(EW∙EW))x((K*(EW∙NS))x((K*(EW∙NS))x((K*(NS∙NS))x

for every x ϵR2 inside the prediction block area.

Secondly, the matrices Jρ∇pred (x) are diagonalized such that

λ1(x)00λ2(x)=cos⁡(θ)-sin⁡(θ)sin⁡(θ)cos⁡(θ) Jρ∇pred(x) cos⁡(θ)sin⁡(θ)-sin⁡(θ)cos⁡(θ),

where λ1(x), λ2(x) denote the eigenvalues of Jρ∇pred (x) and θ represents the associated rotation angle in the coordinate system. Using trigonometric formulas, it can be shown that

λ1(x)=14(gxx(x)-gxy(x)tanθ)

λ2(x)=14(gyy(x)-gxy(x)tanθ),

where 
tan⁡(2θ) =2gxy(x)gyy(x)-gxx(x)

The determination of the eigenvalues λ1,  λ2 is implemented using integer look-up tables. For that purpose, with a and b representing the rounded integers

a =256⋅2gxygyy-gxx       and      b =256⋅gyy-gxx2gxy

the following expressions are tabulated for a = 0..256 and b = 0..256

tab1 a =256⋅cos 0.5⋅atana256 2

tab2 a =256⋅cos 0.5⋅atana256 ⋅sin 0.5⋅atana256

tab3 a =256⋅sin 0.5⋅atana256 2

tab4 b =256⋅cos 0.5⋅atan256b 2

tab5 b =256⋅cos 0.5⋅atan256b ⋅sin 0.5⋅atan256b

tab6 b =256⋅sin 0.5⋅atan256b 2

where the square brackets [ ] specify rounding to the next integer.

Next, a function f  is applied to the eigenvalues λ1, λ2 of Jρ∇pred which serves as an edge detector: if eigenvalues λ1, λ2 are large, i.e. an edge is indicated, function f will be small. Function f is chosen in case of intra as an approximation of fs=exp⁡(-sμ), and in case of inter as fs=1 / (1+sμ) with parameter μ>0. In the implementation, the factor 14  can be incorporated into function f. The Parameter μ is a fixed value given by

μ=200,  &for intra prediction64,  &for inter prediction and QP<33550,  &for inter prediction and QP≥33

The two functions fs are tabulated using integer tables.

After applying the function f, the matrix is retransformed

fJρ∇predx:=cosθsinθ-sinθcosθfλ100f(λ2)cosθ-sinθsinθcosθ=: axxaxyaxyayy  (x)

Using this sample-wise definition, three functions axx, axy,ayy are defined with an entry for each sample location x ϵR2 inside the prediction block area. Similarly, as described above the rotation of the 2×2 matrix is implemented using the look-up tables given above.

Given the arrays axx(x), axy(x),ayy(x) for each sample location xϵR2, the following 8 arrays are derived using a simple averaging of two neighboring samples:

wN≔12010010000*ayy

wS≔12000010010*ayy

wW≔12000110000*axx

wE≔12000011000*axx

wO1≔14010001000* axy

wO2≔14000001010* axy

wO3≔14010100000* axy

wO4≔14000100010* axy

The integer weighting matrices wN, wS, wW, wE, wO1, wO2, wO3, wO4 are calculated at the beginning of the signal dependent diffusion filter process. These matrices are used for the actual filtering in the iteration steps. For summarizing the approach, the integer weighting matrices are derived by the following ordered steps:

- Calculate discrete samples differences EW(x) and NS(x)

- Calculate products (EW⋅EW)(x), EW⋅NSx , (NS⋅NS)(x)

- Filter the three products with a 5-tap filter kernel for obtaining the gradient arrays gxx(x), gxy(x), and gyy(x)

- Determine the eigenvalues λ1(x), λ2(x) using look-up tables

- Apply a function f(s) to the eigenvalues using pre-defined look-up tables

- Derive the integer arrays axx(x), axy(x), and ayy(x) using look-up tables

- Derive the integer weighting arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 using sample averages.

Given the arrays wN, wS, wW, wE, wO1, wO2, wO3, and wO4 for the sample locations xϵR2, the iterative filtering proceeds as follows. Let u0=pred represent the initial luma prediction signal. In each iteration step, the signal u is modified according to

ut+1=ut+14(wO1∙001000000*ut-wO2∙000000001*ut-wO3∙100000000*ut+wO4∙000000100*ut+wN∙0100-10000*ut+wW∙0001-10000*ut+wE∙0000-11000*ut+wS∙(0000-10010*ut))

Figure 2 Left: Original Intra Prediction, Middle: Uniformly Filtered Intra Prediction, Right: Signal dependent Filtered Intra Prediction

where the multiplication dot and the summation is to be understood pointwise. Mapping ut+1denotes the filtered prediction after t+1 iteration steps. This way, diffusion along edges is preferred over diffusion perpendicular to them. The signal dependent filter is applied to the prediction for two different amounts of iteration steps which corresponds to two signal dependent diffusion filter modes from which the encoder can choose. In case of Intra, it is proposed that the encoder can choose between 5 and 20 iteration steps and for Inter between 4 and 8 as depicted in Table 3 below. In the end, the filtered prediction u is used as the modified prediction block.

Figure 2 shows a comparison for the application of the uniform and the signal dependent filter on an angular intra prediction signal. It is visible that while the uniform filter blurres the edges of the underlying prediction, the signal dependent filter preserves them.

### Bitstream structure

For every CU, one flag is sent to indicate if the diffusion filter is enabled or not. If it is enabled, the index of the diffusion filter mode (in range 0 to 4) is sent. These bits are written into the bitstream after sending the intra prediction modes, reference indexes and motion vector information, respectively.

In the skip case, the diffusion filter is not allowed, thus no bits are sent. Additionally, the diffusion filter is not allowed in case the current CU lies on the left or top picture border. In case the intra prediction mode is DC or Planar, the diffusion is not allowed either. In case of merge, the diffusion filter can be merged according to the chosen candidate or tested on top of the merged prediction block. In the latter case, the diffusion filter information has to be sent additionally.
There are several options to constrain the usage of diffusion filters in order to reduce encoder run time and signaling:

- Restriction to larger blocks: The Diffusion filters are not allowed, if the area of CU is smaller or equal than 32.

- Restriction to larger blocks: The Diffusion filters are not allowed, if at least one side of CU is equal to 4 or 128 (except for 128 x 128).

- Restriction to larger blocks: certain filters are not allowed if at least one side of the CU is equal to 8.

The implemented restrictions a) – c) depending on block width and height are depicted in the table below. For each case the index number of the tested filters is given. Index 0 stands for the diffusion filter being disabled. The Indices 1 and 2 stand for the uniform filter options, indices 3 and 4 for the signal dependent filter options.

| BlkSizes | 4 | 8 | 16 | 32 | 64 | 128 |
| --- | --- | --- | --- | --- | --- | --- |
| 4 | 0 | 0 | 0 | 0 | 0 | 0 |
| 8 | 0 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0,1,3,4 | 0 |
| 16 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 32 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 64 | 0 | 0,1,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0,1,2,3,4 | 0 |
| 128 | 0 | 0 | 0 | 0 | 0 | 0,1,2,3,4 |

Table 1 Restricted number of tested Filters ordered by block sizes, width versus height

- Temporal layer restriction: In inter case the amount of possible diffusion filters for non-reference frames (more specifically for temporal layer 4) is reduced.

- In case of Intra in Inter frames, only uniform diffusion is allowed

- Restriction to larger intra blocks: The diffusion filters are not allowed, if the area of CU is smaller or equal than 64.

or

- Restriction to larger intra blocks in inter frames: The diffusion filters are not allowed, if the area of CU is smaller or equal than 64.

- Restriction to certain Inter modes

### Entropy decoding

There are several modes of the diffusion filter tool that are send in the bitstream as decribed in the table below.

| DiffFilterMode | Description |
| --- | --- |
| 0 | No diffusion filters |
| 1 | Diffusion filters are enabled, in merge case diffusion parameters are merged |
| 4 | Diffusion filters are enabled, in merge case diffusion parameters are tested and send additionally |
| 6 | Diffusion filters are enabled, their application in inter is restricted to certain cases |

Table 2 Binarization of the diffusion filter index

If the diffusion filter is enabled, the diffusion filter index in the range within 0 and 4 is binarized sending one bit to distinguish between uniform and signal dependent filtering and another bit to determine which one of the two filter modes was chosen. This is depicted in the table below.

| diffFilterIdx | Bits |
| --- | --- |
| 0 | 0 |
| 1 | 100 |
| 2 | 101 |
| 3 | 110 |
| 4 | 111 |

Table 3 Binarization of the diffusion filter index

The syntax element diffFilterIdx indicates the filtering mode as well as the number of applied iterations as specified in the table below.

| cu_diffusion_filter_idx | filter type | number of iterations n |  |
| --- | --- | --- | --- |
|  |  | intra | inter |
| 0 | no filter | - | - |
| 1 | uniform | 5 | 5 |
| 2 | uniform | 15 | 35 |
| 3 | signal dependent | 5 | 4 |
| 4 | signal dependent | 20 | 8 |

Table 4 Interpretation of the syntax element diffFilterIdx

The detailed restriction settings for the scheduled tests can be seen in the table below.

| FastDiffusion | Restrictions |  |
| --- | --- | --- |
| 0 | no restrictions |  |
| 1 | restrictions a), e), f) |  |
| 7 (used as default) | restrictions a)- f) |  |
| 9 | restrictions a),e),g) |  |

Table 5 Binarization of the implemented restrictions

### Conversion process from entropy decoded data to symbols

There is one context for the enabling flag of the diffusion filter in case the CU is intra, is inter or inter-merge. The same distinction is done for the contexts used to determine if the filter is uniform or signal dependent and which one of the filters it is. The latter bit has an additional set of contexts (intra, inter and inter-merge) for the uniform and signal dependent filter case.

#### Intra prediction parameters

The diffusion filter is indicated by a CU parameter called diffFilterIdx. Its range is within 0 and 4, where diffFilterIdx >0 indicates that the filter is enabled and which one of the four modes was chosen.

#### Motion parameters

The diffusion filter is indicated by a CU parameter called diffFilterIdx. Its range is within 0 and 4, where diffFilterIdx >0 indicates that the filter is enabled and which one of the four modes was chosen.

# Proposed method: Encoder algorithm description

The diffusion filter is tested in the Intra case on top of the angular intra prediction modes that were chosen for a full RD cost check. All possible combinations of the intra prediction modes and diffusion filter modes are tested using a fast cost check and then those with minimal costs are chosen for a full RD cost check. In case that PbIntraFast is enabled, the list of options for the best diffusion filter combinations is reduced in the same way as the intra mode list. A full RD cost check is performed for the best option with uniform and for the best option with signal dependent filter (if applicable). Only if one of these two is better than the best non-diffused option, further diffusion filter combinations are checked.

In case the non-diffused option resulted in a cbf equal to zero, there are no diffusion filter modes checked.

In case of inter, the diffusion filter is applied on the motion compensated prediction. In case of merge, it can be chosen between merging the diffusion filter candidate or testing it on top of the merged prediction and sending additional filter information. If the config parameter FastInterDiffusionFilter is enabled, there is a fast cost check performed and only the best diffusion filter mode is chosen for a full RD cost check. When performing the fast cost check, first only one uniform and one signal dependent filter is tried. Only if the first filter of either kind performs better, the other one is tested.

# Results

According to the CE10 description in JVET-J1030 [1] and the common test conditions in JVET-J1010 [3], the proposed method is evaluated for intra-only, random-access and low delay configuration with BMS-1.0 software in VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler. The code was made available to all CE10 participants in the following branches:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-4.1

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-4.2

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-4.3

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-4.4

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE10/CE10-4.5

In the following table an overview over the scheduled CE tests with their corresponding config parameters is shown.

|  | Test |  | Section |
| --- | --- | --- | --- |
| CE10.4.1 | All Intra, VTM +BMS - Fast Encoder Decisions and restrictions for All Intra | #Diffusion filtering DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.1 |
| CE10.4.2 | Random Access and Low Delay, VTM +BMS - Fast Encoder Decisions and Restrictions - Merging diffusion parameters | #Diffusion filtering DiffusionFilterMode : 1 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.2 |
| CE10.4.3 | Random Access and Low Delay, VTM +BMS - Fast Encoder Decisions and restrictions - Additionally sending diffusion parameters in merge case | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.3 |
| CE10.4.4 | All Intra, Random Access and Low Delay, VTM +BMS - More Extensive Search - Additionally sending diffusion parameters in merge case | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 9 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.4 |
| CE10.4.5 | Random Access and Low Delay, VTM +BMS - Fast Encoder Decisions and restrictions - Additionally sending diffusion parameters in merge case - No neighboring block samples used | #Diffusion filtering DiffusionFilterMode : 4 FastDiffusionFilter : 7 NMBDRYDiffusion : 1 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.5 |
|  | Random Access and Low Delay, only BMS - Fast Encoder Decisions and restrictions - Additionally sending diffusion parameters in merge case - Restrict application of diffusion filter in inter mode | Use CE10.4.5 Branch and config settings #Diffusion filtering DiffusionFilterMode : 6 FastDiffusionFilter : 7 NMBDRYDiffusion : 0 NumDiffusionFiltersIntra : 4 NumDiffusionFiltersInter : 4 | 3.6 |

## CE10-4.1, All Intra, With Default Restrictions (FastDiffusion=7)

In the table below, the results for All Intra with default restrictions as described above (restriction d) has no impact on Intra) are depicted.

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.65% | -1.83% |  | -1.41% |  |
| Class A2 | -1.03% | -1.22% |  | -0.84% |  |
| Class B | -0.92% | -1.15% |  | -1.43% |  |
| Class C | -0.36% | -0.50% |  | -0.70% |  |
| Class E | -0.82% | -1.29% |  | -1.56% |  |
| Overall | -0.92% | -1.15% |  | -1.19% |  |
| Enc Time [%] | 157% |  |  |  |  |
| Dec Time [%] | 116% |  |  |  |  |
| Class D | -0.27% |  | -0.27% |  | -0.30% |

|  | BMS, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.33% | -3.21% |  | -2.72% |  |
| Class A2 | -1.00% | -1.65% |  | -1.13% |  |
| Class B | -0.74% | -1.56% |  | -1.87% |  |
| Class C | -0.27% | -0.63% |  | -0.56% |  |
| Class E | -0.54% | -1.50% |  | -1.64% |  |
| Overall | -0.74% | -1.63% |  | -1.56% |  |
| Enc Time [%] | 140% |  |  |  |  |
| Dec Time [%] | 104% |  |  |  |  |
| Class D | -0.18% |  | -0.37% |  | -0.38% |

## CE10-4.2, Random Access and Low Delay, No Additional Test In Merge Case, With Default Restrictions (FastDiffusion=7)

The results in the table below in the merge case, the diffusion filter is merged with the merge candidate. Default Restrictions are employed.

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.46% | -2.18% |  | -1.75% |  |
| Class A2 | -1.22% | -1.56% |  | -1.14% |  |
| Class B | -1.45% | -1.75% |  | -2.13% |  |
| Class C | -0.40% | -0.63% |  | -0.77% |  |
| Overall | -1.13% | -1.50% |  | -1.49% |  |
| Enc Time [%] | 112% |  |  |  |  |
| Dec Time [%] | 109% |  |  |  |  |
| Class D | -0.14% |  | -0.47% |  | -0.58% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.80% | -0.14% |  | -0.14% |  |
| Class C | -0.28% | 0.12% |  | 0.24% |  |
| Class E | -0.61% | 0.10% |  | -0.70% |  |
| Overall | -0.58% | 0.01% |  | -0.15% |  |
| Enc Time [%] | 113% |  |  |  |  |
| Dec Time [%] | 110% |  |  |  |  |
| Class D | -0.20% |  | 0.44% |  | -0.17% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.91% | -2.11% |  | -1.31% |  |
| Class A2 | -0.79% | -1.53% |  | -1.03% |  |
| Class B | -0.83% | -1.49% |  | -1.85% |  |
| Class C | -0.20% | -0.40% |  | -0.85% |  |
| Overall | -0.67% | -1.33% |  | -1.31% |  |
| Enc Time [%] | 120% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.06% |  | -0.07% |  | -0.39% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.47% | -0.36% |  | -0.12% |  |
| Class C | -0.25% | 0.06% |  | 0.02% |  |
| Class E | -0.37% | -0.13% |  | -0.53% |  |
| Overall | -0.37% | -0.16% |  | -0.18% |  |
| Enc Time [%] | 116% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.09% |  | 0.35% |  | -0.19% |

## CE10-4.3, Random Access and Low Delay, Additional Test In Merge Case, With Default Restrictions (FastDiffusion=7)

The results in the table below in the merge case, the diffusion filter is tested on top of the chosen merge candidate. Default Restrictions are employed.

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.59% | -2.39% |  | -1.80% |  |
| Class A2 | -1.34% | -1.68% |  | -1.32% |  |
| Class B | -1.64% | -1.92% |  | -2.39% |  |
| Class C | -0.50% | -0.81% |  | -1.02% |  |
| Overall | -1.26% | -1.67% |  | -1.69% |  |
| Enc Time [%] | 126% |  |  |  |  |
| Dec Time [%] | 107% |  |  |  |  |
| Class D | -0.26% |  | -0.49% |  | -0.68% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -1.00% | 0.19% |  | 0.26% |  |
| Class C | -0.40% | 0.44% |  | 0.43% |  |
| Class E | -0.51% | 1.26% |  | -0.10% |  |
| Overall | -0.67% | 0.54% |  | 0.23% |  |
| Enc Time [%] | 138% |  |  |  |  |
| Dec Time [%] | 109% |  |  |  |  |
| Class D | -0.28% |  | 0.46% |  | -0.12% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.97% | -2.24% |  | -1.48% |  |
| Class A2 | -0.90% | -1.72% |  | -1.15% |  |
| Class B | -1.00% | -1.65% |  | -2.16% |  |
| Class C | -0.30% | -0.42% |  | -0.99% |  |
| Overall | -0.79% | -1.46% |  | -1.51% |  |
| Enc Time [%] | 130% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.15% |  | -0.20% |  | -0.53% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.66% | -0.04% |  | 0.17% |  |
| Class C | -0.36% | 0.33% |  | 0.41% |  |
| Class E | -0.46% | -0.72% |  | -0.27% |  |
| Overall | -0.51% | -0.09% |  | 0.14% |  |
| Enc Time [%] | 136% |  |  |  |  |
| Dec Time [%] | 99% |  |  |  |  |
| Class D | -0.15% |  | 0.92% |  | 0.09% |

## CE10-4.4, All Intra, Random Access and Low Delay, Additional Test In Merge Case, With Less Restrictions (FastDiffusion=9)

In the table below results are depicted that are also tested additionally in the merge case. Here, less restrictions are employed.

|  | VTM, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.68% | -1.83% |  | -1.38% |  |
| Class A2 | -1.13% | -1.40% |  | -0.99% |  |
| Class B | -1.05% | -1.25% |  | -1.53% |  |
| Class C | -0.47% | -0.70% |  | -0.94% |  |
| Class E | -0.99% | -1.52% |  | -1.69% |  |
| Overall | -1.03% | -1.29% |  | -1.31% |  |
| Enc Time [%] | 200% |  |  |  |  |
| Dec Time [%] | 118% |  |  |  |  |
| Class D | -0.39% |  | -0.56% |  | -0.51% |

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.70% | -2.57% |  | -1.89% |  |
| Class A2 | -1.47% | -2.00% |  | -1.49% |  |
| Class B | -1.83% | -2.17% |  | -2.61% |  |
| Class C | -0.63% | -0.95% |  | -1.21% |  |
| Overall | -1.41% | -1.89% |  | -1.87% |  |
| Enc Time [%] | 142% |  |  |  |  |
| Dec Time [%] | 109% |  |  |  |  |
| Class D | -0.37% |  | -0.40% |  | -0.73% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -1.07% | -0.16% |  | -0.01% |  |
| Class C | -0.41% | 0.34% |  | 0.41% |  |
| Class E | -0.42% | -0.51% |  | 0.16% |  |
| Overall | -0.69% | -0.08% |  | 0.17% |  |
| Enc Time [%] | 148% |  |  |  |  |
| Dec Time [%] | 109% |  |  |  |  |
| Class D | -0.27% |  | 0.66% |  | 0.04% |

|  | BMS, All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.27% | -3.15% |  | -2.68% |  |
| Class A2 | -1.01% | -1.71% |  | -1.20% |  |
| Class B | -0.75% | -1.60% |  | -1.98% |  |
| Class C | -0.28% | -0.67% |  | -0.70% |  |
| Class E | -0.60% | -1.63% |  | -1.70% |  |
| Overall | -0.75% | -1.68% |  | -1.64% |  |
| Enc Time [%] | 162% |  |  |  |  |
| Dec Time [%] | 104% |  |  |  |  |
| Class D | -0.19% |  | -0.48% |  | -0.32% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.99% | -2.44% |  | -1.60% |  |
| Class A2 | -0.97% | -1.81% |  | -1.27% |  |
| Class B | -1.08% | -1.79% |  | -2.28% |  |
| Class C | -0.30% | -0.65% |  | -1.06% |  |
| Overall | -0.83% | -1.62% |  | -1.62% |  |
| Enc Time [%] | 140% |  |  |  |  |
| Dec Time [%] | 101% |  |  |  |  |
| Class D | -0.18% |  | -0.34% |  | -0.51% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.64% | 0.17% |  | 0.30% |  |
| Class C | -0.30% | 0.52% |  | 0.35% |  |
| Class E | -0.54% | -0.03% |  | -0.70% |  |
| Overall | -0.50% | 0.23% |  | 0.07% |  |
| Enc Time [%] | 141% |  |  |  |  |
| Dec Time [%] | 97% |  |  |  |  |
| Class D | -0.13% |  | 1.65% |  | 0.19% |

## CE10-4.5, Random Access and Low Delay, Additional Test In Merge Case, With Default Restrictions (FastDiffusion=7), Reflected Boundary Condition in Inter Case

The results in the table below in the merge case, the diffusion filter is tested on top of the chosen merge candidate. Default Restrictions are employed. In the inter case there are no reconstructed samples used.

|  | VTM, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -1.50% | -1.99% |  | -1.66% |  |
| Class A2 | -1.22% | -1.38% |  | -1.04% |  |
| Class B | -1.43% | -1.68% |  | -2.09% |  |
| Class C | -0.40% | -0.73% |  | -0.90% |  |
| Overall | -1.13% | -1.43% |  | -1.48% |  |
| Enc Time [%] | 126% |  |  |  |  |
| Dec Time [%] | 106% |  |  |  |  |
| Class D | -0.23% |  | -0.35% |  | -0.62% |

|  | VTM, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.69% | -0.05% |  | 0.41% |  |
| Class C | -0.23% | 0.28% |  | 0.33% |  |
| Class E | -0.46% | 0.58% |  | -0.59% |  |
| Overall | -0.48% | 0.22% |  | 0.13% |  |
| Enc Time [%] | 136% |  |  |  |  |
| Dec Time [%] | 107% |  |  |  |  |
| Class D | -0.14% |  | 0.78% |  | -0.07% |

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.87% | -2.32% |  | -1.68% |  |
| Class A2 | -0.78% | -1.59% |  | -0.99% |  |
| Class B | -0.83% | -1.68% |  | -2.07% |  |
| Class C | -0.22% | -0.47% |  | -1.02% |  |
| Overall | -0.67% | -1.47% |  | -1.50% |  |
| Enc Time [%] | 130% |  |  |  |  |
| Dec Time [%] | 99% |  |  |  |  |
| Class D | -0.07% |  | -0.02% |  | -0.53% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.37% | -0.28% |  | 0.02% |  |
| Class C | -0.18% | 0.17% |  | 0.28% |  |
| Class E | -0.33% | -0.06% |  | -0.37% |  |
| Overall | -0.30% | -0.08% |  | 0.01% |  |
| Enc Time [%] | 137% |  |  |  |  |
| Dec Time [%] | 98% |  |  |  |  |
| Class D | -0.09% |  | 0.98% |  | -0.45% |

## CE10-4.5, Random Access and Low Delay, Additional Test In Merge Case, With Default Restrictions (FastDiffusion=7), Restriction to specific Inter Case (DiffFilterMode=6)

The results in the table below in the merge case, the diffusion filter is tested on top of the chosen merge candidate. Default Restrictions and restriction h) are employed. Here, only BMS is tested, since IMV has to be turned on in order to for the restriction to specific inter cases to be applied properly.

|  | BMS, Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class A1 | -0.94% | -2.13% |  | -1.40% |  |
| Class A2 | -0.75% | -1.76% |  | -1.04% |  |
| Class B | -0.92% | -1.63% |  | -2.01% |  |
| Class C | -0.29% | -0.54% |  | -0.98% |  |
| Overall | -0.72% | -1.47% |  | -1.42% |  |
| Enc Time [%] | 127% |  |  |  |  |
| Dec Time [%] | 100% |  |  |  |  |
| Class D | -0.16% |  | -0.18% |  | -0.63% |

|  | BMS, Low Delay Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate |  | V BD-rate |  |
| Class B | -0.64% | -0.12% |  | 0.18% |  |
| Class C | -0.39% | 0.33% |  | 0.13% |  |
| Class E | -0.52% | -0.50% |  | -0.11% |  |
| Overall | -0.53% | -0.07% |  | 0.09% |  |
| Enc Time [%] | 130% |  |  |  |  |
| Dec Time [%] | 97% |  |  |  |  |
| Class D | -0.21% |  | 0.66% |  | -0.03% |

# Cross checks

The cross-checks for the current document were performed by Ericsson, [4].

# References

- C.W. Hsu, M. Winken, X. Xiu, “Description of Core Experiment 10 (CE10): Combined and multi-hypothesis prediction,” JVET-J1030, San Diego, USA, April 2018.

- J. Pfaff et al., “Signal Adaptive Diffusion Filters For Video Coding,” JVET-J0038, San Diego, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- …, “Cross-check report …”

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).