| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0326-v1 |
| --- | --- |

| Title: | CE 1: Context modelling for QT/BT/TT decision tree (Test 1.0.9) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Tel: Email: | +1-858-651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE1 test 1.0.9. This tool proposes changes to context modelling used to signal QT/BT/TT decision tree. For VTM, the results show -0.01%, 0.04%, 0.06% luma gain in AI, RA and LDB with similar encoding and decoding time. For BMS, the results show 0.00%, 0.05% and -0.02% luma gain in AI, RA and LDB with similar encoding and decoding time.

# Introduction

In current VTM software [1] and VVC draft, QTBT+TT is used as the decision tree for a coding tree unit (CTU). An alternative context modelling for QTBT+TT decision tree is studied in CE1 test 1.0.9 and this document reports its coding performance.

# Proposed method

In general, QTBT+TT signaling is same as that signaling in VTM with exception of the context derivation. For BT/TT signaling, the first bin indicates whether the BT/TT is further split or not. The context for this bin is calculated based on the observation that the likelihood of further split is highly correlated to the relative size of the current block to its neighbors. Let W, H, and A be the width, the height, and the area size of the current block, respectively. The context model Ctx is calculated by comparing the size of the current block relative to those of the neighboring blocks:

- Ctx=0;

- Ctx += H > HL;

- Ctx += W > WT;

- Ctx += A > ATL;

- Ctx += A > ATR;

where HL, WT, ATL, ATR represent the height of the left block, width of the top block, area size of the top-left block, and the area size of the top-right block, respectively. As illustrated in Figure 1, the reference positions for the top and left blocks are located with offsets of H/2 and W/2 from the top-left corner of the block, respectively.

Figure 1: Reference positions for context modeling of BT/TT split flag and BT/TT direction flag.

If BT/TT is further split, the second bin indicates whether it is a horizontal partitioning or vertical partitioning. The presence of the center-sided ternary trees increases the occurrence of “tall” or “wide” blocks and hence, the context model exploits this property to signal this flag more efficiently:

Ctx= (W==H ? 0 : (W > H ? 1 : 2));

if (W > 2H) Ctx = 3;

if (H > 2W) Ctx = 4;

The third bin indicates the tree-type of the partition, i.e., whether it is a BT or TT.

# Experimental Results

The software is implemented based on BMS-1.0. The unmodified BMS-1.0 with VTM configuration used as VTM anchor. Under CTC[2], the software was tested and the results are summarized in following tables. More detailed results can be found in the accompanied Excel tables.

| AI | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.10% | -0.05% | 99% | 101% |
| Class A2 | 0.00% | 0.01% | 0.00% | 99% | 101% |
| Class B | -0.01% | -0.03% | -0.02% | 99% | 100% |
| Class C | -0.01% | -0.08% | -0.06% | 100% | 100% |
| Class E | -0.01% | -0.13% | -0.14% | 99% | 100% |
| Overall | -0.01% | -0.06% | -0.05% | 99% | 100% |
| Class D | 0.00% | -0.01% | 0.04% | 101% | 102% |

Table 1. AI result in VTM

| RA | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.08% | -0.03% | 100% | 101% |
| Class A2 | 0.05% | -0.03% | -0.04% | 100% | 101% |
| Class B | 0.05% | -0.04% | -0.12% | 100% | 101% |
| Class C | 0.00% | -0.17% | -0.14% | 107% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | -0.08% | -0.09% | 102% | 102% |
| Class D | 0.05% | 0.19% | -0.01% | 100% | 102% |

Table 2. RA result in VTM

| LDB | Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.18% | 0.20% | 100% | 101% |
| Class C | 0.02% | 0.11% | 0.10% | 100% | 100% |
| Class E | 0.10% | 0.41% | 0.11% | 99% | 98% |
| Overall | 0.06% | 0.21% | 0.14% | 100% | 100% |
| Class D | 0.01% | 0.04% | -0.71% | 98% | 96% |

Table 3. LDB result in VTM

| AI | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.17% | -0.13% | 99% | 97% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.01% | -0.03% | -0.08% | 99% | 99% |
| Class C | -0.01% | -0.09% | -0.02% | 99% | 100% |
| Class E | 0.02% | -0.15% | -0.28% | 99% | 100% |
| Overall | 0.00% | -0.08% | -0.10% | 99% | 99% |
| Class D | -0.02% | 0.12% | -0.05% | 99% | 98% |

Table 4. AI result in BMS

| RA | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.10% | -0.07% | 100% | 99% |
| Class A2 | 0.06% | -0.09% | 0.09% | 100% | 100% |
| Class B | 0.06% | -0.15% | -0.22% | 99% | 99% |
| Class C | 0.01% | 0.17% | -0.15% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | -0.04% | -0.11% | 99% | 99% |
| Class D | 0.07% | -0.08% | 0.17% | 100% | 100% |

Table 5. RA result in BMS

| LDB | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.19% | 0.27% | 100% | 98% |
| Class C | -0.03% | -0.02% | -0.20% | 99% | 99% |
| Class E | -0.09% | -0.23% | -0.17% | 99% | 99% |
| Overall | -0.02% | -0.14% | 0.00% | 99% | 99% |
| Class D | -0.02% | 0.11% | -0.13% | 100% | 98% |

Table 6. LDB result in BMS

# References

[1] J. Chen, E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, April 2018.

[2] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).