| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0327-v1 |
| --- | --- |

| Title: | CE2: Adaptive Loop Filter Simplification (Test 2.4.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Rahul Vanam, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121 USA | Tel: Email: | +1-858-210-4800 rahul.vanam@interdigital.com yuwen.he@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Technologies related to the Adaptive Loop Filter (ALF) are currently being studied in the Core Experiment on in-loop filters (CE2).  In this proposal, the testing results of the ALF simplification in CE2 Test 2.4.1.2 is reported.  In this simplification, derivative calculation for pictures belonging to the highest temporal layer of RA configuration is simplified by computing derivatives only at the top-left sample position of each 2×2 luma block. It is reported that applying this simplification to the BMS-1.0 and VTM-1.0+ALF anchors yields an average decoding speedup of 3% with a small loss of 0.09% for the RA configuration, and no impact for other configurations.  Applying this simplified ALF to the VTM-1.0 anchor yields average luma BD-rate gain of -3.30%, -5.26%, -4.55% for AI, RA, and LB, respectively.  Similarly, use of the simplified ALF in the BMS-1.0 software yields gain over BMS-1.0 without ALF, with average BD-rate gains of -2.74%, -4.98%, -4.24%, for AI, RA, and LB, respectively.

# Introduction

The adaptive loop filter (ALF) is an in-loop filter applied at both the encoder and decoder in the BMS-1.0 software. ALF involves block classification and filtering, and they increase the computational complexity of the decoder.  Block classification is performed on each 2×2 luma reconstructed block, and it involves gradient computation along horizontal, vertical, diagonal and anti-diagonal directions.  CE2 [1] was established in April meeting for ALF related testing. A scheme to simplify ALF is in CE2 Test 2.4.1.2.

# Proposed ALF simplification

The ALF in this proposal is based on ALF in the BMS.  The derivative calculation of 2×2 block for those pictures at the highest temporal layer is simplified by calculating the derivative only at top-left sample position (the gray circles in Figure 1) of each 2×2 block as follows

|  | (1) |
| --- | --- |

where dir is the direction such as horizontal, vertical, diagonal and anti-diagonal, and  is the derivative at top-left sample position of each 2×2 luma block.

Figure 1. Block classification for a 2×2 block.  Gray circles represent pixels for which derivatives are calculated.

# Experimental results

BMS-1.0 is used as the software basis in our experiments as specified by [1], and we use test sequences, configurations, and test conditions specified by JVET-J1010 [2].  Following four tests are conducted as described in [1]:

- VTM-1.0 with simplified ALF vs. VTM-1.0. (Tool on)

- VTM-1.0 with simplified ALF vs. VTM-1.0 + ALF. (Tool on)

- BMS-1.0 – ALF + simplified ALF vs. BMS-1.0 – ALF. (Tool off)

- BMS-1.0 – ALF + simplified ALF vs. BMS-1.0. (Tool off)

Results for Test 1 and 2 are summarized in Table 1 for RA configuration. Similarly, results for Test 3 and 4 are summarized in Table 2.  Results for AI and LB configurations for all tests are provided in the accompanying Excel sheets.

Table 1. RA results of VTM with simplified ALF compared to VTM-1.0 and VTM-1.0+ALF anchors.

|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0 + ALF |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.42% | -2.66% | -1.55% | 107% | 226% | 0.13% | 0.01% | 0.00% | 100% | 96% |
| Class A2 | -6.39% | -1.44% | -0.33% | 109% | 227% | 0.08% | -0.04% | -0.05% | 100% | 96% |
| Class B | -5.72% | -3.42% | -2.29% | 111% | 201% | 0.09% | -0.03% | 0.01% | 100% | 98% |
| Class C | -4.47% | -1.02% | -2.00% | 109% | 167% | 0.08% | -0.03% | -0.01% | 100% | 97% |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | -5.26% | -2.23% | -1.67% | 109% | 200% | 0.09% | -0.02% | -0.01% | 100% | 97% |
| Class D | -5.35% | -2.13% | -2.54% | 114% | 168% | 0.10% | 0.00% | 0.03% | 100% | 97% |

Table 2. RA results of BMS-1.0 with simplified ALF compared to BMS-1.0 – ALF and BMS-1.0 anchors.

|  | Over BMS-1.0 – ALF |  |  |  |  | Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.15% | -2.79% | -0.39% | 101% | 202% | 0.13% | -0.02% | 0.02% | 100% | 95% |
| Class A2 | -6.34% | -0.79% | -0.42% | 102% | 200% | 0.06% | -0.06% | -0.06% | 100% | 97% |
| Class B | -5.31% | -3.60% | -2.22% | 102% | 160% | 0.09% | -0.04% | -0.01% | 99% | 98% |
| Class C | -4.16% | -1.21% | -1.65% | 102% | 139% | 0.08% | -0.04% | 0.00% | 100% | 98% |
| Class E | - | - | - | - | - | - | - | - | - | - |
| Overall | -4.98% | -2.24% | -1.34% | 102% | 169% | 0.09% | -0.04% | -0.01% | 100% | 97% |
| Class D | -5.20% | -2.12% | -2.18% | 102% | 139% | 0.12% | -0.01% | 0.00% | 100% | 96% |

# Conclusion

This contribution proposes a simplification approach for the adaptive loop filter.  The derivative calculation for pictures belonging to the highest temporal layer of RA configuration is simplified by computing derivatives only at the top-left sample position of each 2×2 luma block.  Applying this simplification to the BMS-1.0 and VTM-1.0+ALF anchors yields an average decoding speedup of 3% with a small loss of 0.09% for the RA configuration.

# References

- L. Zhang and K. Andersson, “Description of Core Experiment (CE2) on In-Loop Filters,” JVET-J1022, 10th Meeting, April 2018, San Diego, USA.

- J. Boyce et. al., “JVET common test conditions and software reference configurations”, JVET-J1010, 10th Meeting, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).