| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0328 |
| --- | --- |

| Title: | CE13: Cubemap projection (Tests 2.1 and 2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 tests 2.1 and 2.2. The cubemap projection (CMP) is one of the anchors in the 360° video CTC. Results based on end-to-end WS-PSNR show that CMP provides luma BD-rate savings over PERP of -3.49% and -3.39% for VTM and BMS configurations, respectively. Guard bands of 4 luma samples yield a luma BD-rate loss of about 0.7%.

# Introduction

Projection formats are used to map 360° video to a normal 2D video such that it can be encoded with existing video codecs. The (padded) ERP and CMP projection formats are supported by HEVC SEI messages and the omnidirectional media format (OMAF). CMP is studied in CE13 tests 2.1 and 2.2 [1] and this contribution reports the coding performance of CMP.

# Technical description

The CMP is defined in [2]. When padding is used, guard bands of 4 luma samples are added around each group of continuous faces, as it has been suggested in JVET-G0071 [3]. No blending is used.

# Simulation results

The software is implemented based on BMS-1.0 and 360Lib-6.0. The encoding was done under 360° video CTC [4]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanied Excel spreadsheets.

## Test 2.1: CMP w/o padding

Results for CMP without padding over PERP are reported in Table 1.

Table 1. BD-rate for CMP w/o padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -8.77% | -11.69% | -11.25% | -9.74% | -11.84% | -11.43% |
|  | ChairLift | -19.01% | -14.52% | -15.17% | -14.93% | -6.49% | -11.93% |
|  | KiteFlite | -4.65% | 0.33% | -2.22% | -4.83% | 0.36% | -4.61% |
|  | Harbor | 0.69% | -1.84% | -0.97% | 0.31% | -1.18% | -1.51% |
|  | Trolley | -2.87% | 3.96% | -0.95% | -3.61% | 5.66% | -0.29% |
|  | GasLamp | 1.72% | 6.95% | 14.21% | 0.93% | 6.33% | 13.52% |
| Class S2 | Balboa | -5.16% | -0.32% | -1.54% | -3.27% | 1.75% | 0.22% |
|  | Broadway | -0.08% | -0.77% | 0.37% | 1.46% | 1.28% | 2.11% |
|  | Landing2 | 5.05% | 5.75% | 4.48% | 3.97% | 5.98% | 4.31% |
|  | BranCastle2 | -1.80% | 0.44% | 0.97% | -4.16% | -1.06% | -0.49% |
| Class S1 |  | -5.48% | -2.80% | -2.72% | -5.31% | -1.19% | -2.71% |
| Class S2 |  | -0.50% | 1.28% | 1.07% | -0.50% | 1.99% | 1.54% |
| Overall |  | -3.49% | -1.17% | -1.21% | -3.39% | 0.08% | -1.01% |
| Enc Time[%] |  | 106% |  |  | 106% |  |  |
| Dec Time[%] |  | 102% |  |  | 100% |  |  |

Simulation results show overall coding performance of CMP is consistent between VTM and BMS configurations. BD-rate savings over PERP are around 3.5%. Large variations are observed between different sequences, with values ranging from a loss of about 5% on Landing2 to a gain of 19% on ChairLift. The encoding complexity of CMP is slightly increased compared to PERP, but the decoding complexity is similar.

## Test 2.2: CMP w/ padding

Results for CMP with padding over PERP are reported in Table 2.

Table 2. BD-rate for CMP w/ padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -8.34% | -11.77% | -11.64% | -9.22% | -12.12% | -11.81% |
|  | ChairLift | -18.60% | -13.91% | -15.05% | -14.48% | -6.36% | -11.34% |
|  | KiteFlite | -4.07% | 0.71% | -2.13% | -4.23% | 0.49% | -4.58% |
|  | Harbor | 1.31% | -2.50% | -1.67% | 0.96% | -1.94% | -1.83% |
|  | Trolley | -2.08% | 4.59% | -0.52% | -2.79% | 5.56% | 0.38% |
|  | GasLamp | 2.68% | 7.08% | 14.66% | 1.78% | 6.16% | 13.37% |
| Class S2 | Balboa | -4.74% | 0.44% | -1.08% | -2.79% | 2.00% | 1.10% |
|  | Broadway | 0.35% | -0.50% | 0.67% | 1.94% | 1.30% | 2.38% |
|  | Landing2 | 5.66% | 5.40% | 4.05% | 4.64% | 5.55% | 4.21% |
|  | BranCastle2 | -0.81% | 1.10% | 1.59% | -3.14% | -0.32% | 0.08% |
| Class S1 |  | -4.85% | -2.63% | -2.73% | -4.66% | -1.37% | -2.63% |
| Class S2 |  | 0.12% | 1.61% | 1.31% | 0.16% | 2.13% | 1.94% |
| Overall |  | -2.86% | -0.94% | -1.11% | -2.73% | 0.03% | -0.80% |
| Enc Time[%] |  | 107% |  |  | 106% |  |  |
| Dec Time[%] |  | 107% |  |  | 101% |  |  |

Simulation results show similar trends as for CMP without padding. BD-rate savings over PERP are around 2.8%.

Results for CMP with padding over CMP without padding are reported in Table 3.

Table 3. BD-rate for CMP w/ padding over CMP w/o padding.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.48% | -0.08% | -0.44% | 0.59% | -0.29% | -0.43% |
|  | ChairLift | 0.52% | 0.71% | 0.14% | 0.55% | 0.15% | 0.67% |
|  | KiteFlite | 0.64% | 0.38% | 0.11% | 0.66% | 0.13% | 0.04% |
|  | Harbor | 0.64% | -0.68% | -0.72% | 0.67% | -0.78% | -0.35% |
|  | Trolley | 0.88% | 0.61% | 0.45% | 0.93% | -0.10% | 0.68% |
|  | GasLamp | 0.96% | 0.12% | 0.37% | 0.87% | -0.16% | -0.12% |
| Class S2 | Balboa | 0.45% | 0.75% | 0.47% | 0.52% | 0.25% | 0.87% |
|  | Broadway | 0.45% | 0.26% | 0.32% | 0.48% | 0.02% | 0.25% |
|  | Landing2 | 0.60% | -0.31% | -0.39% | 0.67% | -0.37% | -0.11% |
|  | BranCastle2 | 1.05% | 0.67% | 0.63% | 1.11% | 0.75% | 0.58% |
| Class S1 |  | 0.69% | 0.18% | -0.01% | 0.71% | -0.18% | 0.08% |
| Class S2 |  | 0.64% | 0.34% | 0.26% | 0.69% | 0.16% | 0.40% |
| Overall |  | 0.67% | 0.24% | 0.09% | 0.71% | -0.04% | 0.21% |
| Enc Time[%] |  | 100% |  |  | 100% |  |  |
| Dec Time[%] |  | 104% |  |  | 100% |  |  |

Simulation results show that padding reduces coding gain by about 0.7%.

# Conclusion

This contribution reports simulation result of CE13 tests 2.1 and 2.2, which evaluates CMP without and with padding, respectively. The results show BD-rate savings over PERP of -3.49% and -3.39% for VTM and BMS configurations, respectively. Padding yields a BD-rate loss of about 0.7%.

# References

- P. Hanhart, J.-L. Lin, “Description of Core Experiment 13 (CE13): Projection formats”, JVET-J1033, Apr. 2018, San Diego, USA.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5)”, JVET-H1004, Oct 2017, Macao, CN.

- G. Van der Auwera, M. Coban, M. Karczewicz, “AHG8: ACP with padding for 360-degree video”, JVET-G0071, July 2017, Torino, IT.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.