| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0329 |
| --- | --- |

| Title: | CE13: Equi-angular cubemap projection (Tests 3.1 and 3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 tests 3.1 and 3.2. The equi-angular cubemap projection (EAC) format is a cube-based projection format that aims at having a uniform sampling rate regardless of sampling location on a cube face. Results based on end-to-end WS-PSNR show that EAC provides luma BD-rate savings over PERP of -11.53% and -11.24% for VTM and BMS configurations, respectively. Compared to CMP, EAC provides luma BD-rate savings of -8.56% and -8.42% for VTM and BMS configurations, respectively. Guard bands of 4 luma samples yield a luma BD-rate loss of 0.58% for both VTM and BMS configurations.

# Introduction

Projection formats are used to map 360° video to a normal 2D video such that it can be encoded with existing video codecs. The (padded) ERP and CMP projection formats are supported by HEVC SEI messages and the omnidirectional media format (OMAF). However, alternative projection formats may provide better coding efficiency. EAC is studied in CE13 tests 3.1 and 3.2 [1] and this contribution reports the coding performance of EAC.

# Technical description

The EAC is defined in [2]. When padding is used, guard bands of 4 luma samples are added around each group of continuous faces, as it has been suggested in JVET-G0071 [3]. No blending is used.

# Simulation results

The software is implemented based on BMS-1.0 and 360Lib-6.0. The encoding was done under 360° video CTC [4]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanied Excel spreadsheets.

## Test 3.1: EAC w/o padding

Results for EAC without padding over PERP and CMP are reported in Table 1 and Table 2, respectively.

Table 1. BD-rate for EAC w/o padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -19.17% | -20.18% | -19.49% | -19.84% | -20.51% | -20.63% |
|  | ChairLift | -26.47% | -19.85% | -20.06% | -22.22% | -12.89% | -16.02% |
|  | KiteFlite | -10.28% | -1.97% | -4.46% | -11.06% | -3.10% | -6.85% |
|  | Harbor | -10.36% | -3.00% | -2.67% | -11.46% | -2.81% | -2.67% |
|  | Trolley | -8.60% | 2.15% | -3.00% | -9.93% | 3.86% | -1.98% |
|  | GasLamp | -5.42% | 3.65% | 10.38% | -6.70% | 2.59% | 9.11% |
| Class S2 | Balboa | -11.61% | -8.59% | -9.92% | -9.21% | -7.64% | -8.34% |
|  | Broadway | -9.33% | -7.70% | -7.02% | -7.74% | -6.63% | -5.55% |
|  | Landing2 | -3.24% | -3.83% | -6.35% | -2.80% | -2.80% | -5.85% |
|  | BranCastle2 | -10.87% | -8.79% | -8.35% | -11.49% | -9.31% | -8.24% |
| Class S1 |  | -13.38% | -6.53% | -6.55% | -13.53% | -5.48% | -6.51% |
| Class S2 |  | -8.76% | -7.23% | -7.91% | -7.81% | -6.59% | -6.99% |
| Overall |  | -11.53% | -6.81% | -7.09% | -11.24% | -5.92% | -6.70% |
| Enc Time[%] |  | 106% |  |  | 106% |  |  |
| Dec Time[%] |  | 103% |  |  | 102% |  |  |

Table 2. BD-rate for EAC w/o padding over CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -11.06% | -9.11% | -8.92% | -10.90% | -9.31% | -9.79% |
|  | ChairLift | -9.41% | -6.22% | -5.81% | -8.77% | -6.84% | -4.69% |
|  | KiteFlite | -6.40% | -2.28% | -2.36% | -7.08% | -3.48% | -2.48% |
|  | Harbor | -11.39% | -1.33% | -1.82% | -12.21% | -1.88% | -1.30% |
|  | Trolley | -6.85% | -1.77% | -2.18% | -7.63% | -1.74% | -1.83% |
|  | GasLamp | -7.38% | -3.15% | -3.32% | -8.00% | -3.61% | -3.81% |
| Class S2 | Balboa | -6.84% | -8.23% | -8.47% | -6.24% | -9.25% | -8.58% |
|  | Broadway | -9.27% | -6.98% | -7.34% | -9.12% | -7.82% | -7.55% |
|  | Landing2 | -7.67% | -8.74% | -9.98% | -6.42% | -8.00% | -9.40% |
|  | BranCastle2 | -9.29% | -9.14% | -9.22% | -7.85% | -8.34% | -7.78% |
| Class S1 |  | -8.75% | -3.98% | -4.07% | -9.10% | -4.48% | -3.98% |
| Class S2 |  | -8.27% | -8.27% | -8.75% | -7.41% | -8.35% | -8.33% |
| Overall |  | -8.56% | -5.69% | -5.94% | -8.42% | -6.03% | -5.72% |
| Enc Time[%] |  | 100% |  |  | 100% |  |  |
| Dec Time[%] |  | 100% |  |  | 101% |  |  |

Simulation results show overall coding performance of EAC is consistent between VTM and BMS configurations. BD-rate savings over PERP and CMP are around 11.5% and 8.5%, respectively. Large variations are observed between different sequences, with gain ranging from about 3% to over 26% compared to PERP. The complexity of EAC is similar to that of CMP.

## Test 3.2: EAC w/ padding

Results for EAC with padding over PERP and CMP are reported in Table 3 and Table 4, respectively.

Table 3. BD-rate for EAC w/ padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -18.88% | -20.65% | -20.49% | -19.72% | -21.31% | -21.58% |
|  | ChairLift | -26.33% | -19.39% | -19.75% | -22.05% | -13.04% | -15.82% |
|  | KiteFlite | -9.67% | -1.65% | -4.11% | -10.48% | -3.00% | -6.78% |
|  | Harbor | -9.91% | -4.05% | -3.63% | -11.07% | -4.45% | -4.47% |
|  | Trolley | -7.80% | 2.07% | -2.80% | -9.16% | 3.42% | -2.04% |
|  | GasLamp | -4.72% | 3.76% | 11.03% | -5.97% | 2.43% | 9.09% |
| Class S2 | Balboa | -11.42% | -8.59% | -9.79% | -8.95% | -7.51% | -8.05% |
|  | Broadway | -9.14% | -7.50% | -6.93% | -7.51% | -7.06% | -4.89% |
|  | Landing2 | -2.72% | -4.79% | -7.26% | -2.25% | -3.87% | -6.67% |
|  | BranCastle2 | -10.03% | -8.20% | -7.58% | -10.62% | -8.68% | -7.53% |
| Class S1 |  | -12.89% | -6.65% | -6.62% | -13.07% | -5.99% | -6.93% |
| Class S2 |  | -8.33% | -7.27% | -7.89% | -7.33% | -6.78% | -6.78% |
| Overall |  | -11.06% | -6.90% | -7.13% | -10.78% | -6.31% | -6.87% |
| Enc Time[%] |  | 107% |  |  | 106% |  |  |
| Dec Time[%] |  | 104% |  |  | 101% |  |  |

Table 4. BD-rate for EAC w/ padding over CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -10.73% | -9.67% | -10.04% | -10.76% | -10.20% | -10.84% |
|  | ChairLift | -9.22% | -5.67% | -5.44% | -8.55% | -7.00% | -4.46% |
|  | KiteFlite | -5.74% | -1.96% | -2.01% | -6.44% | -3.38% | -2.38% |
|  | Harbor | -10.94% | -2.42% | -2.81% | -11.81% | -3.59% | -3.14% |
|  | Trolley | -6.00% | -1.85% | -2.02% | -6.80% | -2.19% | -1.90% |
|  | GasLamp | -6.67% | -3.05% | -2.75% | -7.27% | -3.77% | -3.83% |
| Class S2 | Balboa | -6.62% | -8.21% | -8.32% | -5.95% | -9.13% | -8.29% |
|  | Broadway | -9.06% | -6.78% | -7.25% | -8.87% | -8.25% | -6.94% |
|  | Landing2 | -7.14% | -9.58% | -10.78% | -5.85% | -8.92% | -10.13% |
|  | BranCastle2 | -8.40% | -8.54% | -8.45% | -6.91% | -7.70% | -7.06% |
| Class S1 |  | -8.22% | -4.10% | -4.18% | -8.60% | -5.02% | -4.43% |
| Class S2 |  | -7.81% | -8.28% | -8.70% | -6.90% | -8.50% | -8.11% |
| Overall |  | -8.05% | -5.77% | -5.99% | -7.92% | -6.41% | -5.90% |
| Enc Time[%] |  | 100% |  |  | 100% |  |  |
| Dec Time[%] |  | 102% |  |  | 101% |  |  |

Simulation results show similar trends as for EAC without padding. BD-rate savings over PERP and CMP are around 11.0% and 8.0%, respectively.

Results for EAC with padding over EAC without padding are reported in Table 5.

Table 5. BD-rate for EAC w/ padding over EAC w/o padding.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.42% | -0.60% | -1.21% | 0.22% | -0.97% | -1.17% |
|  | ChairLift | 0.25% | 0.58% | 0.40% | 0.28% | -0.15% | 0.24% |
|  | KiteFlite | 0.73% | 0.34% | 0.36% | 0.72% | 0.10% | 0.10% |
|  | Harbor | 0.56% | -1.10% | -1.01% | 0.52% | -1.77% | -1.88% |
|  | Trolley | 0.96% | -0.08% | 0.16% | 0.96% | -0.47% | -0.08% |
|  | GasLamp | 0.79% | 0.10% | 0.59% | 0.82% | -0.19% | -0.03% |
| Class S2 | Balboa | 0.25% | 0.03% | 0.16% | 0.33% | 0.14% | 0.30% |
|  | Broadway | 0.25% | 0.21% | 0.10% | 0.30% | -0.46% | 0.65% |
|  | Landing2 | 0.58% | -0.94% | -0.89% | 0.61% | -1.01% | -0.81% |
|  | BranCastle2 | 1.00% | 0.67% | 0.85% | 1.04% | 0.71% | 0.79% |
| Class S1 |  | 0.62% | -0.13% | -0.12% | 0.59% | -0.57% | -0.47% |
| Class S2 |  | 0.52% | -0.01% | 0.05% | 0.57% | -0.16% | 0.23% |
| Overall |  | 0.58% | -0.08% | -0.05% | 0.58% | -0.41% | -0.19% |
| Enc Time[%] |  | 100% |  |  | 100% |  |  |
| Dec Time[%] |  | 101% |  |  | 100% |  |  |

Simulation results show that padding reduces coding gain by 0.58%.

# Conclusion

This contribution reports simulation result of CE13 tests 3.1 and 3.2, which evaluates EAC without and with padding, respectively. EAC improves the sampling uniformity, resulting in improved coding efficiency. The results show BD-rate savings over PERP of -11.53% and -11.24% for VTM and BMS configurations, respectively. Compared to CMP, EAC provides BD-rate savings of -8.56% and -8.42% for VTM and BMS configurations, respectively. Padding yields a BD-rate loss of 0.58% for both configurations.

# References

- P. Hanhart, J.-L. Lin, “Description of Core Experiment 13 (CE13): Projection formats”, JVET-J1033, Apr. 2018, San Diego, USA.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5)”, JVET-H1004, Oct 2017, Macao, CN.

- G. Van der Auwera, M. Coban, M. Karczewicz, “AHG8: ACP with padding for 360-degree video”, JVET-G0071, July 2017, Torino, IT.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. does not have any current or pending patent rights relating to the technology described in this contribution.