| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0330 |
| --- | --- |

| Title: | CE13: Hybrid angular cubemap projection (Tests 4.1 and 4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results of CE13 tests 4.1 and 4.2. The hybrid angular cubemap projection (HAC) format is a generalization of the EAC projection format. The horizontal and vertical sampling functions can be adjusted for each face adaptively based on the content. Results based on end-to-end WS-PSNR show that HAC provides luma BD-rate savings over PERP of -11.73% and -11.52% for VTM and BMS configurations, respectively. Compared to CMP, HAC provides luma BD-rate savings of -8.75% and -8.70% for VTM and BMS configurations, respectively. Compared to EAC, HAC provides luma BD-rate savings of -0.24% and -0.33% for VTM and BMS configurations, respectively. Guard bands of 4 luma samples yield a luma BD-rate loss of about 0.6%.

# Introduction

Projection formats are used to map 360° video to a normal 2D video such that it can be encoded with existing video codecs. The (padded) ERP and CMP projection formats are supported by HEVC SEI messages and the omnidirectional media format (OMAF). However, alternative projection formats may provide better coding efficiency. HAC is studied in CE13 tests 4.1 and 4.2 [1] and this contribution reports the coding performance of HAC.

# Technical description

## Hybrid angular cubemap

The HAC format is related to CMP with forward and backward mapping functions. Assuming (x, y) is defined in a CMP face with side length equal to 2, and (x’, y’) is the corresponding point in an HAC face, Equations (1) (2) are the forward mapping functions from CMP to HAC for the horizontal and vertical directions, respectively.

|  | (1) |
| --- | --- |
|  | (2) |

Equations (3) (4) are the backward mapping from HAC to CMP, for the horizontal and vertical directions, respectively.

|  | (3) |
| --- | --- |
|  | (4) |

where a and b are the HAC horizontal and vertical mapping function parameters, and are defined within the range of [0.3, 1.5]. The sampling distribution of HAC (that is, parameters a and b) for each face can be adaptively selected based on the content. As the parameter a or b gets closer to 0, the sampling distribution in that corresponding direction becomes closer to that of CMP.

HAC has 6 faces and can be packed using a 3×2 layout. Figure 1 shows an example for the frame packed picture. Denote Fi as the i-th face of HAC, and (ai, bi) as the parameters defining the mapping functions of face Fi. To keep sampling continuities at face boundaries in between two neighboring faces, the following constraint is applied in HAC.

|  | (5) |
| --- | --- |
|  | (6) |

In other words, the first row faces F4, F0 and F5 will share the same vertical mapping function, and the second row faces F3, F1 and F2 will share the same vertical mapping function.

Figure 1. Frame packed picture in 3×2 layout for Harbor.

In order to be able to combine HAC with adaptive frame packing, further constraints must be applied to ensure that the vertical continuity within each face row holds when shuffling faces. This yields to only three parameters: one for each of the X, Y, and Z axes.

For sequences with large global camera motion, the content may change drastically within a face during the sequence and sampling parameters determined at the beginning may not be optimal through the whole sequence. Thus, when the global motion is large, the adaptiveness of HAC is disabled, i.e., the parameters are set to 1. This detection is performed based on the absolute frame difference computed between the pictures at t=0 and t=0.5s. The EAC format is used in the detection by the encoder.

The HAC parameters are determined at the encoder side for the first frame of the sequence and signaled in the SPS.

When padding is used, guard bands of 4 luma samples are added around each group of continuous faces, as it has been suggested in JVET-G0071 [3]. No blending is used.

## Hybrid angular cubemap parameter determination

Taking into account the constraints in (5)(6), eight parameters need to be determined for the 3×2 HAC projection format, including six parameters in the horizontal direction, and two parameters in the vertical direction.

The HAC parameter derivation process is separated into two sub-processes: (a) deriving parameters for the first face row consisting of F4, F0, and F5; (b) deriving parameters for the second face row consisting of F2, F1, and F3. For each face row, the following iterative parameters searching algorithm for horizontal and vertical mapping functions is applied:

Step 1:	search parameters for the horizontal direction while fixing parameters for the vertical direction, and update the parameters for the horizontal direction with the optimal parameters found in the search; if there is no update, the search process stops; otherwise, go to Step 2;

Step 2:	search parameters for the vertical direction while fixing parameters for the horizontal direction, and update the parameters for the vertical direction with the optimal parameters found in the search; if there is no update, the search process stops; otherwise, go to Step 1.

The search algorithm usually converges after 3~4 iterations. In Step 1 and Step 2, the HAC parameters are selected by minimizing end-to-end weighted conversion-only error calculated as in Equation (7), where wSSE(x, y) is the weighted SSE calculation using sample set x and y, and the weights are the same as those used in WS-PSNR calculation for ERP in 360Lib.

|  | (7) |
| --- | --- |

where  is the portion of source ERP corresponding to face Fi in HAC, and  is the portion of reconstructed ERP corresponding to face Fi in HAC.

For the three parameters configuration, parameter search is performed in a similar way as for the eight parameters configuration: search the parameter for one axis while fixing the parameters for the other two axes, and update the parameter for the searched axis with the optimal parameters found in the search; repeat for the other two axes; if there is no update, the search process stops.

# Simulation results

The software is implemented based on BMS-1.0 and 360Lib-6.0. The encoding was done under 360° video CTC [4]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanied Excel spreadsheets.

## Test 4.1: HAC w/o padding

Results for HAC, 8 parameters configuration, without padding over PERP, CMP, and EAC are reported in Table 1, Table 2, and Table 3, respectively.

Table 1. BD-rate for HAC (8 parameters configuration) w/o padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -20.91% | -22.72% | -21.62% | -22.11% | -23.79% | -23.33% |
|  | ChairLift | -26.32% | -19.75% | -20.09% | -21.97% | -12.43% | -16.08% |
|  | KiteFlite | -10.33% | -2.05% | -4.51% | -11.13% | -3.12% | -6.94% |
|  | Harbor | -10.95% | -3.04% | -2.99% | -12.24% | -2.78% | -3.00% |
|  | Trolley | -8.66% | 2.24% | -3.43% | -10.03% | 4.28% | -2.40% |
|  | GasLamp | -5.29% | 4.16% | 11.12% | -6.62% | 2.84% | 9.85% |
| Class S2 | Balboa | -11.40% | -8.88% | -10.29% | -9.07% | -8.03% | -8.80% |
|  | Broadway | -9.32% | -7.76% | -6.97% | -7.73% | -6.81% | -5.38% |
|  | Landing2 | -3.22% | -3.79% | -6.64% | -2.80% | -2.83% | -6.11% |
|  | BranCastle2 | -10.87% | -8.86% | -8.34% | -11.49% | -9.34% | -8.03% |
| Class S1 |  | -13.74% | -6.86% | -6.92% | -14.02% | -5.83% | -6.98% |
| Class S2 |  | -8.70% | -7.32% | -8.06% | -7.77% | -6.75% | -7.08% |
| Overall |  | -11.73% | -7.04% | -7.38% | -11.52% | -6.20% | -7.02% |
| Enc Time[%] |  | 121% |  |  | 110% |  |  |
| Dec Time[%] |  | 103% |  |  | 102% |  |  |

Table 2. BD-rate for HAC (8 parameters configuration) w/o padding over CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -12.75% | -11.86% | -11.23% | -13.21% | -12.86% | -12.69% |
|  | ChairLift | -9.23% | -6.10% | -5.84% | -8.47% | -6.35% | -4.76% |
|  | KiteFlite | -6.46% | -2.36% | -2.44% | -7.16% | -3.50% | -2.55% |
|  | Harbor | -12.01% | -1.42% | -2.19% | -13.01% | -1.92% | -1.69% |
|  | Trolley | -6.96% | -1.69% | -2.63% | -7.79% | -1.36% | -2.27% |
|  | GasLamp | -7.26% | -2.68% | -2.65% | -7.93% | -3.41% | -3.14% |
| Class S2 | Balboa | -6.60% | -8.50% | -8.83% | -6.09% | -9.64% | -9.03% |
|  | Broadway | -9.26% | -7.04% | -7.30% | -9.11% | -8.00% | -7.40% |
|  | Landing2 | -7.66% | -8.70% | -10.23% | -6.42% | -8.03% | -9.64% |
|  | BranCastle2 | -9.29% | -9.21% | -9.21% | -7.85% | -8.36% | -7.57% |
| Class S1 |  | -9.11% | -4.35% | -4.50% | -9.59% | -4.90% | -4.52% |
| Class S2 |  | -8.20% | -8.36% | -8.89% | -7.37% | -8.51% | -8.41% |
| Overall |  | -8.75% | -5.96% | -6.26% | -8.70% | -6.34% | -6.07% |
| Enc Time[%] |  | 114% |  |  | 104% |  |  |
| Dec Time[%] |  | 101% |  |  | 102% |  |  |

Table 3. BD-rate for HAC (8 parameters configuration) w/o padding over EAC.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -1.90% | -3.14% | -2.63% | -2.55% | -4.00% | -3.33% |
|  | ChairLift | 0.20% | 0.12% | -0.04% | 0.32% | 0.53% | -0.07% |
|  | KiteFlite | -0.09% | -0.06% | -0.07% | -0.10% | -0.02% | -0.08% |
|  | Harbor | -0.80% | -0.10% | -0.38% | -1.01% | -0.06% | -0.41% |
|  | Trolley | -0.17% | 0.08% | -0.45% | -0.23% | 0.38% | -0.45% |
|  | GasLamp | 0.12% | 0.48% | 0.70% | 0.06% | 0.19% | 0.69% |
| Class S2 | Balboa | 0.26% | -0.30% | -0.40% | 0.17% | -0.43% | -0.49% |
|  | Broadway | 0.01% | -0.06% | 0.03% | 0.01% | -0.19% | 0.16% |
|  | Landing2 | 0.01% | 0.04% | -0.27% | 0.00% | -0.03% | -0.25% |
|  | BranCastle2 | 0.00% | -0.08% | 0.01% | 0.00% | -0.03% | 0.23% |
| Class S1 |  | -0.44% | -0.44% | -0.48% | -0.59% | -0.50% | -0.61% |
| Class S2 |  | 0.07% | -0.10% | -0.16% | 0.04% | -0.17% | -0.09% |
| Overall |  | -0.24% | -0.30% | -0.35% | -0.33% | -0.36% | -0.40% |
| Enc Time[%] |  | 114% |  |  | 104% |  |  |
| Dec Time[%] |  | 100% |  |  | 100% |  |  |

Simulation results show overall coding performance of HAC is consistent between VTM and BMS configurations. BD-rate savings over PERP, CMP, and EAC are around 11.6%, 8.7%, and 0.3%, respectively. Large variations are observed between different sequences, with gain ranging from about 3% to over 26% compared to PERP. The decoding complexity of HAC is similar to that of CMP and EAC. However, compared to CMP and EAC, the encoding complexity is increased by 14% and 4% for VTM and BMS configurations, respectively, due to the parameter search process.

Results for HAC, 3 parameters configuration, without padding over PERP, CMP, and EAC are reported in Table 4, Table 5, and Table 6, respectively.

Table 4. BD-rate for HAC (3 parameters configuration) w/o padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -20.79% | -22.70% | -21.58% | -21.90% | -23.76% | -23.56% |
|  | ChairLift | -25.91% | -19.30% | -19.73% | -21.75% | -12.18% | -15.93% |
|  | KiteFlite | -10.34% | -1.94% | -4.51% | -11.15% | -3.05% | -6.80% |
|  | Harbor | -10.72% | -2.48% | -2.44% | -11.96% | -2.42% | -2.63% |
|  | Trolley | -8.60% | 2.12% | -3.07% | -9.96% | 4.15% | -2.36% |
|  | GasLamp | -5.36% | 4.02% | 10.75% | -6.70% | 2.97% | 9.44% |
| Class S2 | Balboa | -11.55% | -8.76% | -10.27% | -9.19% | -7.74% | -8.70% |
|  | Broadway | -9.32% | -7.76% | -6.97% | -7.73% | -6.81% | -5.38% |
|  | Landing2 | -3.22% | -3.79% | -6.64% | -2.80% | -2.83% | -6.11% |
|  | BranCastle2 | -10.87% | -8.86% | -8.34% | -11.49% | -9.34% | -8.03% |
| Class S1 |  | -13.62% | -6.71% | -6.76% | -13.90% | -5.72% | -6.97% |
| Class S2 |  | -8.74% | -7.29% | -8.05% | -7.80% | -6.68% | -7.06% |
| Overall |  | -11.67% | -6.95% | -7.28% | -11.46% | -6.10% | -7.01% |
| Enc Time[%] |  | 123% |  |  | 110% |  |  |
| Dec Time[%] |  | 104% |  |  | 102% |  |  |

Table 5. BD-rate for HAC (3 parameters configuration) w/o padding over CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -12.59% | -11.83% | -11.18% | -12.96% | -12.78% | -12.87% |
|  | ChairLift | -8.72% | -5.57% | -5.42% | -8.22% | -6.08% | -4.59% |
|  | KiteFlite | -6.47% | -2.26% | -2.43% | -7.17% | -3.44% | -2.40% |
|  | Harbor | -11.78% | -0.85% | -1.63% | -12.72% | -1.56% | -1.28% |
|  | Trolley | -6.88% | -1.80% | -2.31% | -7.69% | -1.48% | -2.26% |
|  | GasLamp | -7.32% | -2.81% | -2.97% | -8.01% | -3.26% | -3.53% |
| Class S2 | Balboa | -6.77% | -8.39% | -8.80% | -6.22% | -9.37% | -8.94% |
|  | Broadway | -9.26% | -7.04% | -7.30% | -9.11% | -8.00% | -7.40% |
|  | Landing2 | -7.66% | -8.70% | -10.23% | -6.42% | -8.03% | -9.64% |
|  | BranCastle2 | -9.29% | -9.21% | -9.21% | -7.85% | -8.36% | -7.57% |
| Class S1 |  | -8.96% | -4.19% | -4.32% | -9.46% | -4.77% | -4.49% |
| Class S2 |  | -8.25% | -8.34% | -8.89% | -7.40% | -8.44% | -8.39% |
| Overall |  | -8.67% | -5.85% | -6.15% | -8.64% | -6.24% | -6.05% |
| Enc Time[%] |  | 116% |  |  | 104% |  |  |
| Dec Time[%] |  | 102% |  |  | 102% |  |  |

Table 6. BD-rate for HAC (3 parameters configuration) w/o padding over EAC.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -1.72% | -3.10% | -2.57% | -2.25% | -3.90% | -3.50% |
|  | ChairLift | 0.76% | 0.69% | 0.41% | 0.60% | 0.82% | 0.11% |
|  | KiteFlite | -0.09% | 0.03% | -0.06% | -0.12% | 0.04% | 0.08% |
|  | Harbor | -0.47% | 0.48% | 0.18% | -0.62% | 0.31% | 0.02% |
|  | Trolley | -0.07% | -0.03% | -0.14% | -0.10% | 0.26% | -0.45% |
|  | GasLamp | 0.05% | 0.35% | 0.35% | -0.02% | 0.35% | 0.30% |
| Class S2 | Balboa | 0.08% | -0.18% | -0.36% | 0.03% | -0.14% | -0.40% |
|  | Broadway | 0.01% | -0.06% | 0.03% | 0.01% | -0.19% | 0.16% |
|  | Landing2 | 0.01% | 0.04% | -0.27% | 0.00% | -0.03% | -0.25% |
|  | BranCastle2 | 0.00% | -0.08% | 0.01% | 0.00% | -0.03% | 0.23% |
| Class S1 |  | -0.26% | -0.26% | -0.31% | -0.42% | -0.35% | -0.57% |
| Class S2 |  | 0.02% | -0.07% | -0.15% | 0.01% | -0.10% | -0.07% |
| Overall |  | -0.15% | -0.19% | -0.24% | -0.25% | -0.25% | -0.37% |
| Enc Time[%] |  | 116% |  |  | 104% |  |  |
| Dec Time[%] |  | 101% |  |  | 101% |  |  |

Simulation results show similar trends as for the HAC 8 parameters configurations. BD-rate savings over PERP, CMP, and EAC are around 11.5%, 8.6%, and 0.2%, respectively.

Results comparing the 3 parameters configuration to the 8 parameters configuration are reported in Table 7.

Table 7. BD-rate for HAC 3 parameters configuration over HAC 8 parameters configuration.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.18% | 0.04% | 0.07% | 0.30% | 0.13% | -0.13% |
|  | ChairLift | 0.56% | 0.57% | 0.44% | 0.28% | 0.29% | 0.18% |
|  | KiteFlite | 0.00% | 0.09% | 0.01% | -0.01% | 0.06% | 0.16% |
|  | Harbor | 0.33% | 0.58% | 0.56% | 0.41% | 0.37% | 0.44% |
|  | Trolley | 0.10% | -0.12% | 0.31% | 0.13% | -0.13% | 0.01% |
|  | GasLamp | -0.06% | -0.13% | -0.34% | -0.08% | 0.16% | -0.39% |
| Class S2 | Balboa | -0.18% | 0.12% | 0.04% | -0.14% | 0.29% | 0.09% |
|  | Broadway | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | Landing2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
|  | BranCastle2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% |
| Class S1 |  | 0.19% | 0.17% | 0.18% | 0.17% | 0.15% | 0.04% |
| Class S2 |  | -0.05% | 0.03% | 0.01% | -0.03% | 0.07% | 0.02% |
| Overall |  | 0.09% | 0.12% | 0.11% | 0.09% | 0.12% | 0.04% |
| Enc Time[%] |  | 101% |  |  | 100% |  |  |
| Dec Time[%] |  | 101% |  |  | 100% |  |  |

Simulation results show that introducing more constraints on the parameters reduces the performance by about 0.1%. However, the 3 parameters configuration can be used with adaptive frame packing to further improve coding efficiency.

## Test 4.2: HAC w/ padding

Results for HAC (8 parameters configuration) with padding over PERP, CMP, and EAC are reported in Table 8, Table 9, and Table 10, respectively.

Table 8. BD-rate for HAC (8 parameters configuration) w/ padding over PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -20.87% | -23.21% | -22.65% | -22.00% | -24.72% | -24.61% |
|  | ChairLift | -26.09% | -19.27% | -19.84% | -21.72% | -12.70% | -16.19% |
|  | KiteFlite | -9.75% | -1.55% | -4.15% | -10.54% | -3.01% | -6.48% |
|  | Harbor | -10.63% | -3.95% | -4.17% | -11.86% | -4.36% | -4.48% |
|  | Trolley | -7.91% | 2.54% | -2.76% | -9.26% | 3.71% | -1.91% |
|  | GasLamp | -4.57% | 4.12% | 11.21% | -5.93% | 2.61% | 9.64% |
| Class S2 | Balboa | -10.90% | -8.25% | -9.30% | -8.58% | -7.62% | -7.88% |
|  | Broadway | -9.15% | -7.65% | -6.70% | -7.53% | -6.88% | -5.57% |
|  | Landing2 | -2.72% | -4.93% | -7.27% | -2.25% | -3.81% | -6.76% |
|  | BranCastle2 | -10.03% | -8.21% | -7.71% | -10.63% | -8.72% | -7.51% |
| Class S1 |  | -13.30% | -6.89% | -7.06% | -13.55% | -6.41% | -7.34% |
| Class S2 |  | -8.20% | -7.26% | -7.75% | -7.25% | -6.76% | -6.93% |
| Overall |  | -11.26% | -7.04% | -7.33% | -11.03% | -6.55% | -7.17% |
| Enc Time[%] |  | 122% |  |  | 110% |  |  |
| Dec Time[%] |  | 104% |  |  | 102% |  |  |

Table 9. BD-rate for HAC (8 parameters configuration) w/ padding over CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -12.66% | -12.40% | -12.37% | -13.06% | -13.88% | -14.08% |
|  | ChairLift | -8.93% | -5.53% | -5.56% | -8.16% | -6.63% | -4.88% |
|  | KiteFlite | -5.83% | -1.87% | -2.05% | -6.51% | -3.39% | -2.05% |
|  | Harbor | -11.68% | -2.38% | -3.40% | -12.63% | -3.57% | -3.21% |
|  | Trolley | -6.15% | -1.40% | -1.99% | -6.96% | -1.91% | -1.83% |
|  | GasLamp | -6.54% | -2.72% | -2.56% | -7.24% | -3.61% | -3.35% |
| Class S2 | Balboa | -6.07% | -7.87% | -7.84% | -5.57% | -9.25% | -8.13% |
|  | Broadway | -9.07% | -6.93% | -7.05% | -8.89% | -8.07% | -7.58% |
|  | Landing2 | -7.15% | -9.71% | -10.81% | -5.86% | -8.89% | -10.22% |
|  | BranCastle2 | -8.41% | -8.55% | -8.58% | -6.92% | -7.73% | -7.04% |
| Class S1 |  | -8.63% | -4.38% | -4.65% | -9.09% | -5.50% | -4.90% |
| Class S2 |  | -7.67% | -8.27% | -8.57% | -6.81% | -8.48% | -8.24% |
| Overall |  | -8.25% | -5.94% | -6.22% | -8.18% | -6.69% | -6.23% |
| Enc Time[%] |  | 114% |  |  | 104% |  |  |
| Dec Time[%] |  | 101% |  |  | 101% |  |  |

Table 10. BD-rate for HAC (8 parameters configuration) w/ padding over EAC.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -1.74% | -3.74% | -3.88% | -2.34% | -5.13% | -4.90% |
|  | ChairLift | 0.57% | 0.73% | 0.27% | 0.70% | 0.23% | -0.19% |
|  | KiteFlite | 0.62% | 0.43% | 0.32% | 0.63% | 0.09% | 0.42% |
|  | Harbor | -0.35% | -1.06% | -1.63% | -0.51% | -1.76% | -1.96% |
|  | Trolley | 0.74% | 0.38% | 0.19% | 0.71% | -0.19% | -0.02% |
|  | GasLamp | 0.91% | 0.44% | 0.79% | 0.84% | -0.01% | 0.49% |
| Class S2 | Balboa | 0.85% | 0.39% | 0.69% | 0.74% | 0.00% | 0.48% |
|  | Broadway | 0.24% | 0.05% | 0.30% | 0.27% | -0.27% | -0.02% |
|  | Landing2 | 0.58% | -1.07% | -0.93% | 0.61% | -0.95% | -0.90% |
|  | BranCastle2 | 1.00% | 0.65% | 0.71% | 1.04% | 0.68% | 0.81% |
| Class S1 |  | 0.12% | -0.47% | -0.66% | 0.00% | -1.13% | -1.03% |
| Class S2 |  | 0.67% | 0.00% | 0.19% | 0.66% | -0.14% | 0.09% |
| Overall |  | 0.34% | -0.28% | -0.32% | 0.27% | -0.73% | -0.58% |
| Enc Time[%] |  | 115% |  |  | 104% |  |  |
| Dec Time[%] |  | 101% |  |  | 100% |  |  |

Simulation results show similar trends as for HAC without padding. BD-rate savings over PERP and CMP are around 11.1% and 8.2%, respectively. Compared to EAC, a loss of around 0.3% is reported.

Results for HAC (8 parameters configuration) with padding over HAC (8 parameters configuration) without padding are reported in Table 11.

Table 11. BD-rate for HAC w/ padding over HAC w/o padding.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | 0.18% | -0.61% | -1.26% | 0.22% | -1.17% | -1.60% |
|  | ChairLift | 0.37% | 0.61% | 0.31% | 0.38% | -0.30% | -0.12% |
|  | KiteFlite | 0.71% | 0.50% | 0.40% | 0.73% | 0.11% | 0.50% |
|  | Harbor | 0.46% | -0.97% | -1.25% | 0.52% | -1.70% | -1.55% |
|  | Trolley | 0.92% | 0.29% | 0.64% | 0.95% | -0.57% | 0.43% |
|  | GasLamp | 0.79% | -0.04% | 0.09% | 0.77% | -0.20% | -0.21% |
| Class S2 | Balboa | 0.59% | 0.69% | 1.09% | 0.57% | 0.42% | 0.98% |
|  | Broadway | 0.23% | 0.12% | 0.27% | 0.27% | -0.08% | -0.19% |
|  | Landing2 | 0.57% | -1.11% | -0.66% | 0.61% | -0.92% | -0.64% |
|  | BranCastle2 | 1.00% | 0.73% | 0.70% | 1.03% | 0.70% | 0.58% |
| Class S1 |  | 0.57% | -0.04% | -0.18% | 0.59% | -0.64% | -0.43% |
| Class S2 |  | 0.60% | 0.11% | 0.35% | 0.62% | 0.03% | 0.18% |
| Overall |  | 0.58% | 0.02% | 0.03% | 0.60% | -0.37% | -0.18% |
| Enc Time[%] |  | 100% |  |  | 100% |  |  |
| Dec Time[%] |  | 101% |  |  | 100% |  |  |

Simulation results show that padding reduces coding gain by about 0.6%.

# Conclusion

This contribution reports simulation result of CE13 tests 4.1 and 4.2, which evaluates HAC without and with padding, respectively. HAC adjusts the sampling functions within each face, resulting in improved coding efficiency. The results show BD-rate savings over PERP of -11.73% and -11.52% for VTM and BMS configurations, respectively. Compared to CMP, HAC provides BD-rate savings of -8.75% and -8.70% for VTM and BMS configurations, respectively. Compared to EAC, HAC provides BD-rate savings of -0.24% and -0.33% for VTM and BMS configurations, respectively. Padding yields a BD-rate loss of about 0.6%.

# References

- P. Hanhart, J.-L. Lin, “Description of Core Experiment 13 (CE13): Projection formats”, JVET-J1033, Apr. 2018, San Diego, USA.

- Y. Ye, E. Alshina, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 5)”, JVET-H1004, Oct 2017, Macao, CN.

- G. Van der Auwera, M. Coban, M. Karczewicz, “AHG8: ACP with padding for 360-degree video”, JVET-G0071, July 2017, Torino, IT.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).