| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0332 |
| --- | --- |

| Title: | CE13-related: Adaptive frame packing on top of CMP, MCP, and PAU |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution reports simulation results for adaptive frame packing (AFP, JVET-K0331) on top of CMP, modified cubemap projection (MCP, JVET-K0131), and parallel-to-axis uniform cubemap projection (PAU, JVET-K0182). Results based on end-to-end WS-PSNR show that AFP provides luma BD-rate savings of about XX%, 0.3%, and 0.15% on top of CMP, MCP, and PAU, respectively. There is no significant encoding or decoding complexity increase due to AFP.

# Introduction

AFP is studied in CE13 tests 4.3 and 4.4 for EAC and hybrid angular cubemap projection (HAC), respectively [1]. Results show that AFP provides luma BD-rate savings of about 0.3% on top of both projections with no significant encoding or decoding complexity increase [1]. AFP may be used for any cubemap-like projection format to provide better coding efficiency and reduce the visibility of face seam artifacts. This contribution reports the coding performance of AFP on top of CMP, as well as on top of MCP [2] and PAU [3], two projection formats studied in CE13.

# Simulation results

The software is implemented based on BMS-1.0 and 360Lib-6.0. The encoding was done under 360° video CTC [4]. The following tables only report the end-to-end WS-PSNR results. Results for the other metrics can be found in the accompanied Excel spreadsheets.

## CMP + adaptive frame packing

Results for CMP with AFP over CMP without AFP are reported in Table 2.

Table 1. BD-rate for AFP on top of CMP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot |  |  |  |  |  |  |
|  | ChairLift |  |  |  |  |  |  |
|  | KiteFlite |  |  |  |  |  |  |
|  | Harbor |  |  |  |  |  |  |
|  | Trolley |  |  |  |  |  |  |
|  | GasLamp |  |  |  |  |  |  |
| Class S2 | Balboa |  |  |  |  |  |  |
|  | Broadway |  |  |  |  |  |  |
|  | Landing2 |  |  |  |  |  |  |
|  | BranCastle2 |  |  |  |  |  |  |
| Class S1 |  |  |  |  |  |  |  |
| Class S2 |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Enc Time[%] |  |  |  |  |  |  |  |
| Dec Time[%] |  |  |  |  |  |  |  |

## MCP + adaptive frame packing

Results for MCP with AFP over MCP without AFP are reported in Table 2.

Table 2. BD-rate for AFP on top of MCP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.14% | -0.17% | 0.11% | -0.10% | -0.35% | 0.27% |
|  | ChairLift | -0.19% | -0.20% | -0.71% | -0.29% | -1.97% | -1.40% |
|  | KiteFlite | -0.14% | -1.12% | -1.60% | -0.21% | -1.21% | -1.27% |
|  | Harbor | -0.46% | -2.58% | -3.14% | -0.50% | -2.75% | -3.15% |
|  | Trolley | -0.71% | -2.17% | -2.42% | -0.81% | -2.80% | -2.55% |
|  | GasLamp | -0.90% | -2.62% | -2.07% | -0.80% | -2.75% | -2.54% |
| Class S2 | Balboa | -0.30% | -1.00% | -0.89% | -0.28% | -1.16% | -0.63% |
|  | Broadway | -0.09% | -0.11% | -0.62% | -0.01% | -0.16% | -0.74% |
|  | Landing2 | 0.29% | -0.03% | -0.05% | 0.47% | 0.44% | 0.91% |
|  | BranCastle2 | -0.13% | -0.18% | -0.10% | -0.14% | 0.41% | -0.10% |
| Class S1 |  | -0.42% | -1.48% | -1.64% | -0.45% | -1.97% | -1.77% |
| Class S2 |  | -0.06% | -0.33% | -0.41% | 0.01% | -0.11% | -0.14% |
| Overall |  | -0.28% | -1.02% | -1.15% | -0.27% | -1.23% | -1.12% |
| Enc Time[%] |  | 99% |  |  | 100% |  |  |
| Dec Time[%] |  | 99% |  |  | 100% |  |  |

Simulation results show overall coding performance of MCP with AFP is consistent between VTM and BMS configurations. BD-rate saving for AFP on top of MCP is around 0.27%. The encoding and decoding complexity of MCP with AFP is similar to that of MCP without AFP.

## PAU + adaptive frame packing

Results for PAU with AFP over PAU without AFP are reported in Table 3.

Table 3. BD-rate for AFP on top of PAU.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.08% | -0.19% | 0.01% | -0.06% | -0.20% | 0.80% |
|  | ChairLift | 0.10% | 0.00% | -0.29% | -0.01% | -1.58% | -0.93% |
|  | KiteFlite | -0.10% | -1.07% | -1.39% | -0.14% | -1.10% | -1.38% |
|  | Harbor | -0.42% | -2.34% | -2.74% | -0.41% | -2.30% | -2.54% |
|  | Trolley | -0.55% | -2.10% | -2.16% | -0.59% | -2.76% | -2.45% |
|  | GasLamp | -0.78% | -2.27% | -1.60% | -0.68% | -2.58% | -2.34% |
| Class S2 | Balboa | -0.22% | -0.62% | -0.58% | -0.14% | -0.91% | -0.15% |
|  | Broadway | 0.19% | -0.16% | 0.10% | 0.34% | -0.41% | -0.50% |
|  | Landing2 | 0.03% | -0.47% | -0.42% | 0.12% | -0.13% | 0.58% |
|  | BranCastle2 | 0.14% | 0.18% | 0.13% | 0.14% | 0.92% | 0.31% |
| Class S1 |  | -0.30% | -1.33% | -1.36% | -0.32% | -1.75% | -1.47% |
| Class S2 |  | 0.03% | -0.27% | -0.19% | 0.12% | -0.13% | 0.06% |
| Overall |  | -0.17% | -0.90% | -0.89% | -0.14% | -1.11% | -0.86% |
| Enc Time[%] |  | 101% |  |  | 103% |  |  |
| Dec Time[%] |  | 106% |  |  | 101% |  |  |

Simulation results show overall coding performance of PAU with AFP is consistent between VTM and BMS configurations. BD-rate saving for AFP on top of PAU is around 0.14%. The encoding and decoding complexity of PAU with AFP is similar to that of PAU without AFP.

# Conclusion

This contribution reports simulation result of AFP on top of CMP, MCP, and PAU. The results show BD-rate savings of about XX%, 0.3%, and 0.15% on top of CMP, MCP, and PAU, respectively. There is no significant encoding or decoding complexity increase due to AFP.

# References

- P. Hanhart, Y. He, Y. Ye, “CE13: Adaptive frame packing (Tests 4.3 and 4.4)”, JVET-K0331, July 2018, Ljubljana, SI.

- Y.-H. Lee, J.-L. Lin, S.-K. Chang, C.-C. Ju, “CE13: Modified Cubemap Projection in JVET-J0019 (Test 5)”, JVET-K0131, July 2018, Ljubljana, SI.

- Y. Sun, X. Huangfu, B. Wang, L. Yu, “CE13: Parallel-to-Axis Uniform cubemap projection (PAU) in JVET-J0033 (Test 7)”, JVET-K0182, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).