| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0333 |
| --- | --- |

| Title: | AHG8: Horizontal geometry padding for PERP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Philippe Hanhart, Yuwen He, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com Yan.Ye@interdigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This proposal investigates the performance of horizontal geometry padding (JVET-E0065) over conventional repetitive padding at the left and right boundaries for the padded equirectangular projection (PERP). Similar to geometry padding (JVET-D0075), the horizontal geometry padding scheme considers the intrinsic cyclic property of 360° content in the horizontal direction when a reference sample is outside of the picture in motion compensated prediction, but reuses the repetitive padding method in HEVC in the vertical direction. Experimental results show that horizontal geometry padding achieves average BD-rate reduction of 0.16% and 0.19% for VTM and BMS, respectively, at a similar complexity as repetitive padding. More importantly, it is reported that horizontal geometry padding significantly reduces the seam artifact issue for the Chairlift sequence.

# Introduction

Geometry padding of the reference pictures was previously proposed for motion compensated prediction [1]. When using geometry padding, all boundaries (including the top and bottom boundaries) of a reference ERP picture can be extended in a more meaningful way, which can improve continuity of neighboring samples for areas outside of the reference picture boundaries. For HM-16.12, average BD-rate reduction of 0.3% was previously reported. Similar to geometry padding, unrestricted motion compensation in a wrapped-around fashion in the horizontal direction was proposed for ERP [2]. With the wrapped-around motion compensation method, a reference block outside the left reference picture boundary is replaced with the wrapped-around reference block along the right reference picture boundary, and vice versa. For the vertical direction, the repetitive padding method is still employed. Average BD-rate reduction of 0.2% and 0.3% were reported for HM-16.14 and JEM-4.1, respectively.

The PERP format was shown to effectively reduce the seam artifacts in reconstructed viewports that encompass the left and right boundaries of the ERP picture [3]. Therefore, the PERP format was selected as the format used for the HM and JEM anchors for the 360° category in the joint CfP [4]. However, padding and blending may not be sufficient to completely resolve the seam issue, as a seam artifact is still visible for the Chairlift sequence, as reported in [5]. The seam artifact in Chairlift is particularly visible during video playback, as the video appears to flicker where face seams occur. To solve the seam artifact problem, geometry padding of the reference PERP pictures was applied in [5]. The simulation results using the CfP settings showed that geometry padding achieves average BD-rate reduction of 0.23%. More importantly it was reported that, with geometry padding, no seam artifact was visible for the reconstructed viewports located near the PERP boundaries for the Chairlift sequence.

# Horizontal geometry padding for PERP

The horizontal geometry padding for PERP is the same as for ERP, with the exception that the offset should be based on the unpadded ERP width instead of the picture width to account for the size of the padded regions. Figure 1 depicts the horizontal geometry padding for PERP. If a reference block is outside the left (right) reference picture boundary, it is replaced with the “wrapped-around” reference block shifted to the right (left) by the ERP width. As in [2], the traditional repetitive padding is still employed in the vertical direction. The blending of the left and right padded regions is kept out of loop, as a post-processing operation, unlike [5] where the blending operation was moved to the coding loop (i.e., as an additional in-loop filter).

Compared to the previously proposed geometry padding method, the horizontal geometry padding method is simpler because the vertical direction reuses repetitive padding. This simplification is reasonable, as the top and bottom of an ERP or PERP picture represent the poles, which are oversampled, more likely to be less interesting, and less likely to be viewed by viewers.

Figure 1. Horizontal geometry padding for PERP.

# Simulation results

We implemented the horizontal geometry padding method based on the BMS-1.0 and 360Lib-6.0. The encoding was done in the PERP format using the Random Access configuration under the 360° video CTC [6]. Table 1 reports the BD-rate for horizontal geometry padding compared to repetitive padding for the end-to-end WS-PSNR metric. Results show that horizontal geometry padding can improve the coding performance by providing an average BD-rate savings for the {Y, U, V} components of {-0.16%, -0.29%, -0.32%} and {-0.19%, -0.26%, -0.34%} for VTM and BMS, respectively. The maximum BD-rate savings is observed on sequence Chairlift and reaches -0.52% and -0.72% for VTM and BMS, respectively. The complexity of horizontal geometry padding is similar to that of repetitive padding.

Besides providing about 0.5%-0.7% BD rate saving for Chairlift, the most important highlight of horizontal geometry padding is that it significantly improves the subjective quality of the viewport video. With repetitive padding, a seam artifact is clearly visible when the viewport encompasses the left and right boundaries of the 360° video. Though less visible on still images, the seam artifact is particularly objectionable during video playback, as the video appears to flicker where face seams occur. The horizontal geometry padding method significantly reduces the seam artifact issue for the Chairlift sequence, and the seam artifact becomes barely noticeable to expert viewers.

Table 1. Comparison of horizontal geometry padding and repetitive padding for PERP.

|  |  | VTM |  |  | BMS |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BD-rate(Y) | BD-rate(U) | BD-rate(V) | BD-rate(Y) | BD-rate(U) | BD-rate(V) |
| Class S1 | SkateboardInLot | -0.40% | -0.93% | -1.02% | -0.41% | -0.98% | -1.11% |
|  | ChairLift | -0.52% | -0.81% | -0.67% | -0.72% | -0.92% | -0.96% |
|  | KiteFlite | -0.01% | -0.02% | 0.00% | -0.02% | -0.10% | -0.03% |
|  | Harbor | -0.01% | 0.01% | 0.02% | 0.02% | 0.00% | -0.11% |
|  | Trolley | -0.11% | -0.20% | -0.31% | -0.10% | -0.12% | -0.06% |
|  | GasLamp | 0.01% | 0.01% | 0.03% | -0.02% | -0.01% | 0.00% |
| Class S2 | Balboa | -0.01% | 0.08% | -0.28% | -0.07% | -0.14% | -0.08% |
|  | Broadway | -0.04% | -0.12% | -0.08% | -0.06% | 0.26% | 0.18% |
|  | Landing2 | -0.08% | -0.33% | -0.18% | -0.09% | -0.05% | -0.48% |
|  | BranCastle2 | -0.45% | -0.60% | -0.68% | -0.48% | -0.58% | -0.71% |
| Class S1 |  | -0.17% | -0.32% | -0.33% | -0.21% | -0.36% | -0.38% |
| Class S2 |  | -0.14% | -0.24% | -0.31% | -0.17% | -0.13% | -0.27% |
| Overall |  | -0.16% | -0.29% | -0.32% | -0.19% | -0.26% | -0.34% |
| Enc Time[%] |  | 98% |  |  | 100% |  |  |
| Dec Time[%] |  | 103% |  |  | 100% |  |  |

# Conclusion

The PERP format was proposed to resolve the seam artifact observed in the ERP format. Whereas PERP reduces the seam artifact in some sequences, it is not sufficient to resolve the issue for all sequences, as a seam artifact can still be observed in some sequences, e.g., Chairlift. Based on our findings, horizontal geometry padding can significantly reduce the seam artifact for cases where PERP alone is not sufficient. Therefore, it is suggested that JVET establish a focused group (such as CE) to explore the benefit of horizontal geometry padding for resolving the seam artifact issue. To demonstrate the benefits of horizontal geometry padding in reducing the seam artifact issue, we would like to request for a viewing session.

# References

- Y. He, Y. Ye, P. Hanhart, X. Xiu, “AHG8: Geometry padding for 360 video coding”, JVET-D0075, Oct. 2016, Chengdu, CN.

- M. Zhou, “AHG8: Unrestricted Motion Compensation for 360 Video in ERP Format”, JVET-E0065, Jan. 2017, Geneva, CH.

- J. Boyce, Z. Deng, “EE4: Padded ERP (PERP) projection format”, JVET-G0098, July 2017, Torino, IT.

- A. Segall, V. Baroncini, J. Boyce, J. Chen, T. Suzuki, “Joint Call for Proposals on Video Compression with Capability beyond HEVC”, JVET-H1002, Oct. 2017, Macao, CN.

- P. Hanhart, Y. He, Y. Ye, “AHG8: Geometry padding for PERP”, JVET-J0044, Apr. 2018, San Diego, USA.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-J1012, Apr. 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).