| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10– 18 July 2018 | Document: JVET-K0335 |
| --- | --- |

| Title: | CE4-related: Shape dependent control point selection for affine mode |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yuwen He, Xiaoyu Xiu, Yan Ye 9276 Scranton Rd, #300 San Diego, CA 92121, USA | Tel: Email: | 858-210-4819 yuwen.he@interdigital.com xiaoyu.xiu@interdigital.com yan.ye@interdigital.com |
| Source: | InterDigital Communications Inc. |  |  |

_____________________________

# Abstract

Affine coding mode is a tool in Benchmark Set (BMS). It is a four-parameter motion model which can describe translation, zooming, and rotation. The model parameters are signaled by motion vectors at two control points: top-left and top-right corners of an affine coding block. A shape dependent control point selection method is proposed in this document, where the positions of two control points are determined based on the coding block shape. Three sets of control points are defined for different coding block shape: top-left and top-right corners are selected for the coding block with horizontal shape; top-left and bottom-right corners are selected for the coding block with vertical shape; and top-right and bottom-left corners are selected for the coding block with square shape. Compared to the VTM-1.0 anchor, the proposed method achieves average Y BD-rate reduction of 3.22% for Random Access and 2.26% for Low Delay B. Compared to the VTM-1.0 with affine mode enabled, it achieves average Y BD-rate reduction of 0.24% for Random Access and 0.20% for Low Delay B. Compared to the BMS-1.0 anchors, Y BD-rate reduction of xxx% for Random Access cases and xxx% for Low Delay B cases is reported.

# Problem statement

Four-parameter affine motion model is effective to represent the motion filed for those blocks having non-translational motion such as zoom and rotation. And this affine motion model is included in the BMS. In the existing design of affine coding mode in BMS, the parameters are signaled by two MVs at two control points (CP), as shown in Figure 1. The MV, denoted as (vx, vy), of sub-block centered at (x, y) relative to the top-left of the coding block is derived with the MVs at two control points by Eq. (1)(2).

| vx= v0x+v1x-v0x*xw- v1y-v0y*yw | (1) |
| --- | --- |
| vy= v0y+v1y-v0y*xw+ v1x-v0x*yw | (2) |

where (v0x, v0y) is the MV of the control point at top-left corner, and (v1x, v1y) is the MV of the control point at top-right corner, as shown in Figure 1, and w and h are the coding block width and height.

However, the coding block shape can vary greatly due to QT, BT and TT tree partitions. When the coding block become a tall block where the width is much smaller than the height, the MV interpolation in Eq. (1)(2) become less accurate, especially for those sub-blocks whose vertical position (y) is greater than the width. For example, as shown in Figure 2, those sub-blocks affected by block shape are in the bottom half of coding block if the block height is 2x of the block width.

Figure 1. Control points for affine coding block in existing design

Figure 2. Sub-block MV derivation in a tall affine coding block (h = 2w): motion derivation for the bottom half of block is less accurate

# Affine coding improvement

## Shape dependent control point selection

To solve the MV interpolation problem, a coding block shape dependent control point selection method is proposed for the affine coding mode. If coding block width is greater than or equal to coding block height, the control points are defined by top-left corner (P0) and top-right corner (P1) as shown in Figure 3 (a); if coding block width is smaller than coding block height, the control points are defined by top-left corner (P0) and bottom-left (P2) corner as shown in Figure 3 (b).

(a)                                                               (b)

Figure 3. Shape dependent control point selection: (a) CP in horizontal direction; (b) CP in vertical direction

If control points are defined by P0 and P1, sub-block MV derivation is same as current design. If control points are defined by P0 and P2, the MV for sub-block (x, y) is derived as:

| vx= v0x+v2y-v0y*xh+ v2x-v0x*yh | (3) |
| --- | --- |
| vy= v0y-v2x-v0x*xh+ v2y-v0y*yh | (4) |

If a block is coded as affine merge mode, the MVs at two control points are derived from neighboring affine CU with their MVs at control points. In the current design, the first affine coded CU in the affine merge candidate list is used for the MV derivation of current coding block. The affine merge candidate list is constructed from five neighboring blocks, as in Figure 4. To keep the motion interpolation accuracy, the affine merge candidate in affine merge candidate list having maximum distance of two control points is selected for current CU’s MV derivation at control points. Given P0 and P1 are two control points of neighboring affine coded CU, the distance of control points is calculated as:

d=|P0-P1|2

Figure 4. Affine merge candidates: the candidate availability checking order is N0, N1, N2, N3, N4

## Affine MV prediction

In current affine MV prediction, the affine MV predictor is generated from neighboring blocks of top-left, top-right and bottom-left corners. The predictor has three MVs {MV0, MV1, MV2}. As shown in Figure 5, MV0 is from the neighboring blocks {A, B, C}; MV1 is from the neighboring blocks {D, E}; MV2 is from the neighboring blocks {F, G}.

Figure 5. Affine motion vector predictor construction from MVs in neighboring block {A, B, C}, {D, E} and {F, G}

In BMS-1.0, the affine MV predictor will be discarded if its zoom or rotation parameter is greater than a threshold. This proposal proposes to remove this condition checking and put this kind of predictors towards the back of the predictor list, rather than discarding them.

# Simulation results

The proposed changes are implemented based on BMS-1.0 software [2]. We have three tests. Test1 is for the change of shape dependent control point selection described in Sec. 2.1. Test2 is for affine MVP changes described in Sec. 2.2. Test3 is the combination test. We compare the performance with VTM-1.0 anchor, and VTM-1.0 with affine mode enabled (VTM-1.0+affine) according to common test conditions in [1]. Table 1, Table 2, and Table 3 list the RA and LB coding comparison with the VTM-1.0 anchor and VTM-1.0+affine. Additional tests compared to BMS-1.0 and BMS-1.0 disabling affine are also provided in the attached spreadsheets. Test1 achieves average Y BD-rate reduction by 3.13% for RA and 2.27% for LB compared to VTM-1.0 anchor. Compared to VTM-1.0+affine, it achieves average Y BD-rate reduction by 0.15% for RA and 0.21% for LB. Test2 achieves average Y BD-rate reduction by 3.09% for RA and 2.05% for LB compared to VTM-1.0 anchor. Compared to VTM-1.0+affine, it achieves average Y BD-rate reduction by 0.10% for RA and a marginal loss (0.01%) for LB. Test3 achieves average Y BD-rate reduction by 3.22% for RA and 2.26% for LB compared to VTM-1.0 anchor. Compared to VTM-1.0+affine, it achieves average Y BD-rate reduction by 0.24% for RA and 0.20% for LB.

Table 1. Test1 compared to VTM-1.0 and VTM-1.0+affine

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.01% | -1.66% | -1.65% | 139% | 105% | -0.04% | -0.08% | 0.00% | 100% | 100% |
| Class A2 | -6.05% | -4.83% | -4.63% | 143% | 119% | -0.20% | -0.10% | -0.18% | 100% | 102% |
| Class B | -3.42% | -2.27% | -2.40% | 144% | 105% | -0.16% | -0.11% | -0.07% | 100% | 98% |
| Class C | -1.44% | -0.72% | -0.79% | 142% | 100% | -0.17% | 0.04% | 0.02% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.13% | -2.25% | -2.27% | 142% | 106% | -0.15% | -0.06% | -0.05% | 100% | 100% |
| Class D | -2.01% | -1.16% | -1.29% | 142% | 100% | -0.23% | -0.14% | -0.08% | 101% | 101% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.65% | -1.83% | -1.79% | 160% | 108% | -0.16% | -0.35% | -0.01% | 101% | 100% |
| Class C | -1.57% | -0.65% | -0.52% | 146% | 108% | -0.13% | 0.12% | 0.08% | 101% | 102% |
| Class E | -2.57% | -2.36% | -2.46% | 204% | 114% | -0.40% | -0.54% | -0.14% | 101% | 103% |
| Overall | -2.27% | -1.57% | -1.53% | 165% | 110% | -0.21% | -0.24% | -0.01% | 101% | 101% |
| Class D | -3.15% | -1.97% | -2.13% | 140% | 110% | -0.24% | 0.02% | 0.19% | 101% | 101% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

Table 2. Test2 compared to VTM-1.0 and VTM-1.0+affine

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | -1.71% | -1.72% | 140% | 102% | -0.09% | -0.13% | -0.07% | 101% | 98% |
| Class A2 | -6.09% | -4.92% | -4.64% | 145% | 116% | -0.24% | -0.20% | -0.20% | 101% | 99% |
| Class B | -3.39% | -2.33% | -2.43% | 146% | 106% | -0.13% | -0.18% | -0.09% | 101% | 98% |
| Class C | -1.24% | -0.66% | -0.83% | 142% | 101% | 0.02% | 0.10% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.09% | -2.28% | -2.30% | 143% | 106% | -0.10% | -0.10% | -0.09% | 101% | 99% |
| Class D | -1.82% | -1.09% | -1.16% | 142% | 99% | -0.03% | -0.07% | 0.05% | 101% | 100% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.50% | -1.76% | -1.62% | 163% | 109% | -0.01% | -0.29% | 0.15% | 103% | 101% |
| Class C | -1.41% | -0.49% | -0.32% | 147% | 106% | 0.03% | 0.27% | 0.28% | 102% | 100% |
| Class E | -2.15% | -1.87% | -1.89% | 207% | 111% | 0.03% | -0.03% | 0.45% | 103% | 100% |
| Overall | -2.05% | -1.36% | -1.26% | 167% | 109% | 0.01% | -0.04% | 0.27% | 102% | 100% |
| Class D | -2.89% | -1.60% | -1.72% | 140% | 110% | 0.05% | 0.43% | 0.60% | 101% | 101% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

Table 3. Test3 compared to VTM-1.0 and VTM-1.0+affine

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.09% | -1.76% | -1.79% | 140% | 103% | -0.11% | -0.18% | -0.14% | 101% | 99% |
| Class A2 | -6.26% | -5.13% | -4.82% | 145% | 117% | -0.42% | -0.42% | -0.39% | 102% | 100% |
| Class B | -3.53% | -2.41% | -2.56% | 147% | 105% | -0.29% | -0.26% | -0.24% | 102% | 98% |
| Class C | -1.39% | -0.84% | -0.87% | 143% | 99% | -0.13% | -0.08% | -0.06% | 101% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.22% | -2.41% | -2.41% | 144% | 105% | -0.24% | -0.23% | -0.20% | 102% | 99% |
| Class D | -2.04% | -1.26% | -1.40% | 143% | 99% | -0.26% | -0.25% | -0.20% | 102% | 100% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over VTM-1.0+Affine |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.67% | -1.75% | -1.73% | 164% | 109% | -0.19% | -0.27% | 0.05% | 103% | 101% |
| Class C | -1.55% | -0.80% | -0.48% | 148% | 105% | -0.11% | -0.03% | 0.12% | 102% | 98% |
| Class E | -2.54% | -1.99% | -2.49% | 208% | 109% | -0.36% | -0.16% | -0.17% | 104% | 99% |
| Overall | -2.26% | -1.49% | -1.50% | 168% | 108% | -0.20% | -0.17% | 0.02% | 103% | 100% |
| Class D | -3.13% | -2.30% | -2.20% | 141% | 109% | -0.21% | -0.31% | 0.09% | 102% | 100% |
| Class F (optional) | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! | #REF! |

# Conclusion

The shape dependent control point selection is proposed to improve the MV interpolation accuracy for those sub-blocks inside the coding block when it is coded in affine coding mode. It can improve coding efficiency for those sequences on which the affine coding mode is effective. Therefore, it is suggested to be included in the CE related to affine coding.

# References

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018, San Diego, USA.

- BMS software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-J1024, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).