| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0336 |
| --- | --- |

| Title: | CE3-related: Improved multi-directional LM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma, Haitao Yang, Jianle Chen | Email: | maxiang6@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the test results of improved multi-directional LM (MDLM) related, including encoder side complexity reduction(denoted as MDLM1), and encoder side complexity reduction with syntax signalling modification(denoted as MDLM2 ). The tests are also related with PureLM (LM only or LM without Cb-to-Cr prediction), multiple model LM (MMLM) in BMS1.0, including:

- PureLM + MDLM

- PureLM + MDLM1(encoder only modification)

- PureLM + MDLM2 (syntax signalling modification)

- PureLM + MMLM

Simulation results reportedly show BD rate saving on Y, Cb and Cr components for AI configuration over VTM1.0:

- PureLM + MDLM: 	1.30%, 10.35% and 9.73%, with 131% EncTime and 103% DecTime.

- PureLM + MDLM1: 	1.27%, 9.92%, and 9.19%, with 125% EncTime and 103% DecTime.

- PureLM + MDLM2: 	1.29%, 10.16%, and 9.49%, with 126% EncTime and 103% DecTime.

- PureLM + MMLM: 	1.34%, 9.59%, and 8.90%, with 120% EncTime and 102% DecTime.

Tests description

MDLM description

Besides the above template and left template can be used to calculate the linear model coefficients together, they also can be used alternatively in the other 2 LM modes, called MDLM_A, and MDLM_L modes.

In MDLM_A mode, only the above template are used to calculate the linear model coefficients. To get more samples, the above template are extended to (W+H).

Figure 1: MDLM_A mode

In MDLM_L mode, only left template are used to calculate the linear model coefficients. To get more samples, the left template are extended to (H+W).

Figure 2: MDLM_L mode

For a non-square coding block, another (N-W-H) samples will be sampled from (W+H) samples with equal steps to meet the condition of power of 2. Here N is the number of reference samples (to calculate the linear model coefficient). Here note that, N is the smallest integer value that is not smaller than (W+H), and N is a value of power of 2.

Description of improved MDLM

MDLM1 (encoder only modification)

To reduce the encoding complexity of MDLM in [1]. Some encoder only modification are introduced: If the above/left template is not available, then the MDLM_A/MDLM_L mode will not be checked in RDO process; and, for non-square coding block, only the template at the short side will be checked. E.g., for a block with size WxH, if W>H, then only MDLM_L mode will be checked. That is because for a block with W>H, the texture will have a high probability with horizontal direction, so left template is enough to derive an accurate linear model; besides, the samples in long side have a larger overlap with samples used in LM mode, the linear model derived by the long side will have a higher correlation with LM, so there is no need to check that anymore.

MDLM2 (syntax signaling modification)

The methods mentioned in section 1.2.1 also can be used to design a conditional signaling method, like, for current block, if the above/left template is not available, then the MDLM_A/MDLM_L mode will not be signaled. And for a block with size WxH, if W>H, then MDLM_T mode will not be signaled.

All tests

For cross component prediction, there PureLM and MMLM in BMS1.0. For the detail description of those modes, please refer to [2] and [3].To get a clear comparison, the tests results of below tests are presented:

There are 4 MDLM related tests totally:

Test 1: PureLM + MDLM (MDLM in [1])

Test 2: PureLM + MDLM1 (encoder only modification)

Test 3: PureLM + MDLM2 (syntax signalling modification)

Test 4: PureLM + MMLM

Experimental results

The proposed tests are tested under VVC CTC [4], and the results are summarized in Table 1, and Table2. More detailed results are provided in Tabel3, Table4, Tabel5, Table6, Table7, Table8, Table9, and Table10.

Summary of all the tests

Table 1 summary of all the tests results of all tests (AI configuration)

Table 2 summary of all the tests results of all tests (RA configuration)

Test 1: PureLM+MDLM

Table 3 test results of PureLM+MDLM (AI configuration)

Table 4 test results of PureLM+MDLM (RA configuration)

Test 2: PureLM+MDLM1

Table 5 test results of PureLM+MDLM1 (AI configuration)

Table 6 test results of PureLM+MDLM1 (RA configuration)

Test 3: PureLM+MDLM2

Table 7 test results of PureLM+ MDLM2 (AI configuration)

Table 8 test results of PureLM+ MDLM2 (RA configuration)

Test 4: PureLM +MMLM

Table 9 test results of PureLM +MMLM (AI configuration)

Table 10 test results of PureLM +MMLM (RA configuration)

Conclusions

This proposal presents the results of improved MDLM related tests. It can be seen that, at AI configuration over VTM1.0, PureLM+ MDLM1 provide 1.27%, 9.92%, and 9.19% coding gain on Y, Cb and Cr components. And PureLM+ MDLM2 can provide 1.29%, 10.16%, and 9.49% coding gain on Y, Cb and Cr components.

References

- Xiang Ma, Haitao Yang, Jianle Chen, “CE3: Multi-directional LM (Test 4.2.1, 4.2.2, 4.2.3, 4.2.4)”, JVET-K0191, July 2018

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, and Jill Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001-v1, July 2017

- Geert Van der Auwera, Jin Heo, and Alexey Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023_r1, Apr. 2018

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).