| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0337 |
| --- | --- |

| Title: | CE4.1.3: Affine motion compensation prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Han Huang, Yan, Zhang, Chao-Hsiung Hung, Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes affine motion compensation prediction technologies which was firstly proposed in the contribution JVET-J0021 in the 10th JVET meeting. This contribution presents the test results of CE4 test 4.1.3 including 4 sub tests. For the “Tool on case 1 (VTM as reference)” tests in RA case, the sub test a) “Affine MVP construction” achieved objective gain of 3.22% in terms of average luma BD-rate improvement; the sub test b) “a + MVD prediction” achieved 3.42% coding gain; the sub test c) “b + CU level 4-para/6-para switching” achieved 3.90% coding gain; the sub test d) “c + slice level 4-para/6-para switching” achieved 3.88% coding gain.

# Introduction

In HEVC, only translation motion model is applied for motion compensation prediction. While in the real world, there are many kinds of motion, e.g. zoom in/out, rotation, perspective motions and he other irregular motions. In the JEM, a simplified 4-parameter affine transform motion compensation prediction is applied, and this technique is integrated into BMS1.0. This contribution introduces 6-parameter affine model in addition to the 4-parameter model. Affine motion vector prediction is simplified.

# Technical description

## Adaptive affine model selection

4-parameter affine model with two control points Figure 1 (a), and an additional 6-parameter affine model with three control point MVs Figure 1 (b), are used.

(a) 				         (b)

Figure 1: 4-paramenter affine model (a) and 6-parameter affine model (b).

When the 6-parameter affine model is applied, the current block is divided into sub-blocks in the same way as for the 4-parameter affine model in JEM. The motion vector mv=[mvx,mvy] at a location (x,y) in W×H block can be derived from the MVs of the three control points (mv0, mv1, and mv2) as follows

mv x=mv1x-mv0xwx+mv2x-mv0xhy+mv0x

mv y=mv1y-mv0ywx+mv2y-mv0yhy+mv0x.

The affine models are adaptively selected at both slice level and block level. Slice header flags affine4_flag and affine6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. To determine the affine4_flag and affine6_flag for the current slice the statistics from the previous slice are used.

## Affine motion vector prediction

Two candidate sets with two (three) candidates mv0, mv1  mv0, mv1, mv2 are used to predict two (three) control points of the affine motion model. Given motion vector difference vectors, mvd0, mvd1, mvd2, the control points are calculated:

mv0=mv0+mvd0

mv1=mv1+mvd1+mvd0

mv2=mv2+mvd2+mvd0

The affine candidate list is appended sequentially by extending affine motion from spatial neighboring blocks (extrapolated affine candidates), the combination of motion vectors from spatial neighboring blocks (virtual affine candidates) and HEVC motion vector prediction (MVP) candidates until there are two affine MVPs in the candidate list. The candidate sets are constructed as follows:

- Up to two different affine MV predictor sets are derived from affine motion of the neighboring blocks.  Neighboring blocks A0, A1, B0, B1, and B2 as shown in Figure 2 are checked. If the neighboring block is coded using affine motion model and its reference frame is same as the reference frame of the current block, MVs at two (for 4-parameter affine model) or three (for 6-parameter affine model) control points of the current block are derived from the affine model of this neighbor.

Figure 2: Affine motion predictors derivation from affine motion of neighboring blocks.

Figure 3: Affine motion predictors derivation from motion vectors of neighboring blocks.

- Virtual affine candidates. Figure 3 shows the neighboring blocks used to generate the virtual affine candidate set. The neighboring MVs are divided into three groups: S0=mvA,mvB,mvC , S1=mvD,mvE and S2=mvF,mvG. mv0  is the first MV in S0 that refers to the same reference picture as the current block; mv1 is the first MV in S1 that refers to the same reference picture of the current block; and mv2 is the first in S2 that refers to the same reference picture of the current block.

- If only mv0 and mv1 can be found, mv2 is derived as:

mv2x=mv0x-hmv1y-mv0yw, mv2y=mv0y+hmv1x-mv0xw,

where the current block size is W×H.

- If only mv0 and mv2 can be found, mv1 is derived as:

mv1x=mv0x+hmv2y-mv0yw, mv1y=mv0y-hmv2x-mv0xw.

# Simulation results

As descripted in CE4 [2], the test specification is as below, and the “Tool on case 1 (VTM as reference)” and “Tool off case 2 (BMS-Affine as reference)” are tested.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.1.3 | J0021 a) Affine MVP construction b) a + MVD prediction c) b + CU level 4-para/6-para switching d) c + slice level 4-para/6-para switching | Yu Han (Qualcomm) | Huanbang Chen (Huawei) |

The simulation results are as below:

Because of the limited resource, the anchor of “Tool off case 2” was preformed on different clusters from the tests method, and some of the Class B tests in LB case were preformed in different clusters which will affect the encoding/decoding time. We agree with the crosschecker’s number.

## Test4.1.3.a Affine MVP construction

## Test4.1.3.b a + MVD prediction

## Test4.1.3.c b + CU level 4-para/6-para switching

## Test4.1.3.d c + slice level 4-para/6-para switching

# Conclusion

This contribution presents affine motion compensation prediction technologies which are studied in CE4. This contribution is CE4 test 4.1.3 which includes 4 sub tests. For the “Tool on case 1 (VTM as reference)” tests in RA case, the sub test a) “Affine MVP construction” achieved objective gain of 3.22% in terms of average luma BD-rate improvement; the sub test b) “a + MVD prediction” achieved 3.42% coding gain; the sub test c) “b + CU level 4-para/6-para switching” achieved 3.90% coding gain; the sub test d) “c + slice level 4-para/6-para switching” achieved 3.88% coding gain.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Macau, Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

- J. Chen, E. Alshina, G. Sullivan, J. Ohm, J. Boyce, JVET-G1001, “Algorithm description of Joint Exploration Test Model 7”, July 2017.

- B. Bross, JVET-J1001, “Versatile video coding”, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).