| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0338 |
| --- | --- |

| Title: | CE4.2.16: Sub-block merge candidates in BMS and JEM |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Yan Zhang, Chao-Hsiung Hung, Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes advanced temporal motion vector prediction (ATMVP) technology and spatial-temporal motion vector prediction (STMVP) technology which was used in BMS. This contribution presents the test results of CE test 4.2.16 including 3 subtests. For the “Tool on case 1 (VTM as reference)” tests in RA case, the subtest a) “ATMVP” achieved objective gain of 0.87% in terms of average luma BD-rate improvement; the subtest b) “STMVP” achieved 0.76% coding gain; the subtest c) “Combination of ATMVP and STMVP” achieved 1.30% coding gain.

# Introduction

ATMVP and STMVP modes are identical to BMS and JEM. This contribution provides the test results of these two sub-block merge candidates.

# Technical description

In the BMS and JEM, two sub-block merge candidates of ATMVP and STMVP are used to further improve coding efficiency [3]. For the sub-PU mode, the PUs are split into square N×N sub-PUs which allows each PU to obtain multiple sets of motion information (including motion vectors and reference frames). The motion vectors of the sub-PUs are derived recursively, following raster scan order. The ATMVP and STMVP modes are enabled as two additional merge candidates and there is no additional syntax element required to signal these modes.

## Advanced temporal motion vector prediction

In the ATMVP method, the PUs are split into square N×N sub-PUs. The motion vectors and reference indices of the sub-PUs are derived recursively, following raster scan order. The motion derivation for sub-PU includes two steps. The first step is to identify the corresponding block in a reference picture with a so-called temporal vector. The reference picture is called the motion source picture. The second step is to split the current PU into sub-PUs and obtain the motion vectors as well as the reference indices of each sub-PU from the block corresponding to each sub-PU. An example is shown in Figure 1.

In the first step, a reference picture and the corresponding block is determined by the motion information of the spatial neighbouring blocks of the current PU. To avoid the repetitive scanning process of neighbouring blocks, the first merge candidate in the merge candidate list of the current PU is used. The first available motion vector as well as its associated reference index are set to be the temporal vector and the index to the motion source picture.

In the second step, a corresponding block of the sub-PU is identified by the temporal vector in the motion source picture, by adding to the coordinate of the current PU the temporal vector. For each sub-PU, the motion information of its corresponding block (the smallest motion grid that covers the center sample) is used to derive the motion information for the sub-PU. After the motion information of a corresponding N×N block is identified, it is converted to the motion vectors and reference indices of the current sub-PU, in the same way as TMVP of HEVC, wherein motion scaling and other procedures apply. For example, the decoder checks whether the low-delay condition (i.e. the POCs of all reference pictures of the current picture are smaller than the POC of the current picture) is fulfilled and possibly uses motion vector MVx (the motion vector corresponding to reference picture list X) to predict motion vector MVy (with X being equal to 0 or 1 and Y being equal to 1−X) for each sub-PU.

Figure 1: ATMVP motion prediction for a PU

## Spatial-temporal motion vector prediction

In the STMVP method, the PUs are split into square N×N sub-PUs. The motion vectors of the sub-PUs are derived recursively, following raster scan order. As shown in Figure 2, an 8x8 PU is used as an example. It contains four 4x4 sub-PUs A, B, C, and D. The neighboring N×N blocks in the current frame are labeled as a, b, c, and d.

The motion derivation for sub-PU A starts by identifying its two spatial neighbors. The first neighbor is N×N block above sub-PU A (block c). If this block c is not available or is intra coded the other N×N blocks above sub-PU A are checked (from left to right, starting at block c). The second neighbor is a block to the left of the sub-PU A (block b). If block b is not available or is intra coded other blocks to the left of sub-PU A are checked (from top to bottom, staring at block b). The motion information obtained from the neighboring blocks for each list is scaled to the first reference frame for a given list. Next, temporal motion vector predictor (TMVP) of sub-block A is derived by following the same procedure of TMVP derivation as specified in HEVC. The motion information of the co-located block at location D is fetched and scaled accordingly. At last, after retrieving and scaling the motion information, all available motion vectors (up to 3) are averaged separately for each reference list. The averaged motion vector is assigned as the motion vector of the current sub-PU.

A

B

C

D

c

d

b

a

Figure 2: Example of one PU with four sub-blocks (A-D) and its neighbouring blocks (a–d)

# Simulation results

As descripted in CE4 [2], the test specification is as below, and the “Tool on case 1 (VTM as reference)” and “Tool off case 1 (BMS as reference)” with default encoder configuration are used as anchors.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.2.16 | Sub-block merge candidates in BMS and JEM a) ATMVP b) STMVP c) Combination of a) and b) | Yu Han (Qualcomm) | Hahyun Lee (ETRI) |

## Test4.2.16.a ATMVP

Configuration as below: enable ATMVP in tool on test, disable ATMVP in tool off test.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| a) ATMVP | SubPuMvp : 1 MaxNumMergeCand : 6 | SubPuMvp : 0 MaxNumMergeCand : 5 |

Simulation results as below:

## Test4.2.16.b STMVP

Configuration as below: enable STMVP in tool on test, disable ATMVP and enable STMVP in tool off test.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| b) STMVP | SubPuMvp : 2 MaxNumMergeCand : 6 | SubPuMvp : 2 MaxNumMergeCand : 6 |

Simulation results as below:

## Test4.2.16.c Combination of a) and b)

Configuration as below: enable ATMVP and STMVP in both of tool on test and tool off test.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| c) Combination of a) and b) | SubPuMvp : 3 MaxNumMergeCand : 7 | SubPuMvp : 3 MaxNumMergeCand : 7 |

Simulation results as below:

# Conclusion

This contribution presents ATMVP and STMVP prediction technologies which are studied in CE4. This contribution is CE test 4.2.16 which includes 3 sub tests. For the “Tool on case 1 (VTM as reference)” tests in RA case, the sub test a) “ATMVP” achieved objective gain of 0.87% in terms of average luma BD-rate improvement; the sub test b) “STMVP” achieved 0.76% coding gain; the sub test c) “Combination of ATMVP and STMVP” achieved 1.30% coding gain.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Macau, Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

- J. Chen, E. Alshina, G. Sullivan, J. Ohm, J. Boyce, JVET-G1001, “Algorithm description of Joint Exploration Test Model 7”, July 2017.

- B. Bross, JVET-J1001, “Versatile video coding”, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).