| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0341 |
| --- | --- |

| Title: | CE4.2.5: Simplifications on advanced temporal motion vector prediction (ATMVP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 4.2.5 Test (a) – (c) on simplifications on advanced temporal motion vector prediction (ATMVP). In Test (a), one simplified collocated block derivation method is proposed which uses the same collocated picture as in HEVC. In Test (b), adaptive ATMVP sub-block size is proposed to allow using various sub-block sizes for the ATMVP motion derivation. Test (c) is the combination of Test (a) and Test (b). Compared to VTM-1.0, with the combination of all the proposed simplifications, the average {Y, U, V} BD-rate savings are {1.04%, 0.81%, 0.80%} for RA and {1.30%, 1.26%, 1.28%} for LD, respectively. The average encoding and decoding time are 102% and 105% for RA, and 103% and 108% for LD.

# Introduction

At the 10th JVET meeting, advanced temporal motion vector prediction (ATMVP) was included in the benchmark set (BMS)-1.0 reference software [1], which derives multiple motion for sub-blocks of one coding unit (CU) based on the motion information of the collocated blocks from temporal neighboring pictures. Although it improves the efficiency of temporal motion vector prediction, the following complexity issues are identified for the existing ATMVP design:

- The collocated pictures of different ATMVP CUs may not be the same if multiple reference pictures are used. This means the motion fields of multiple reference pictures need to be fetched.

- The motion information of each ATMVP CU is always derived based on 4x4 units, resulting in multiple invocations of motion derivation and motion compensation for each 4x4 sub-block inside one ATMVP CU.

To address the above problems, simplification methods were proposed in [2] to reduce both the average and worst-case complexity of the ATMVP and included in the Core Experiment (CE) 4.2.5.

# Technical description

## Simplified collocated block derivation with one fixed collocated picture

In this method, one simplified design is proposed to use the same collocated picture as in HEVC, which is signaled at the slice header, as the collocated picture for ATMVP derivation. At the block level, if the reference picture of a neighboring block is different from this collocated picture, the MV of the block is scaled using the HEVC temporal MV scaling method, and the scaled MV is used in ATMVP.

Denote the motion vector used to fetch the motion field in the collocated picture Rcol as MVcol. To minimize the impact due to MV scaling, the MV in the spatial candidate list used to derive MVcol is selected in the following way: if the reference picture of a candidate MV is the collocated picture, this MV is selected and used as MVcol without any scaling. Otherwise, the MV having a reference picture closest to the collocated picture is selected to derive MVcol with scaling.

## Adaptive ATMVP sub-block size

In this method, it is proposed to support the slice-level adaptation of the sub-block size for the ATMVP motion derivation. Specifically, one default sub-block size that is used for the ATMVP motion derivation is signaled at sequence level. Additionally, one flag is signaled at slice-level to indicate if the default sub-block size is used for the current slice. If the flag is false, the corresponding ATMVP sub-block size is further signaled in the slice header for the slice. At encoder side, a simple method is used to decide the sub-block size for the ATMVP of the slices in the current picture, based on the average size of the ATMVP CUs from the last coded picture in the same temporal layer. Assume the picture just coded is the i-th picture in the k-th temporal layer and it contains N CUs that are coded by the ATMVP mode. Moreover, assume the sizes of those CUs are S0, S1, …, SN-1 The average size of the ATMVP CUs is calculated as . Then, when coding the (i+1)-th picture in the same k-th temporal layer, the corresponding ATMVP sub-block size  is determined based on the following:

where thres is the threshold which is set to 75 for low-delay (LD) pictures and 27 for non-LD pictures. Additionally, for the first picture at each temporal layer, the ATMVP sub-block size is always set to 4.

# Simulation results

To evaluate the performance/complexity impact, the proposed ATMVP simplification methods were implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. Three sets of simulation results are reported:

- Test (a): the simplified collocated block derivation with one fixed collocated picture

- Test (b): the adaptive ATMVP sub-block size

- Test (c): test (a) + test (b)

Additionally, the test condition and evaluation criteria used for ATMVP-related tests in CE4 [3] are used for the evaluation. Specifically, the results of each test are compared with the anchors as specified in the table below, where Tool on case 1 and Tool off case 2 are mandatory, and Tool on case 2 and Tool off case 1 are optional.

| Test# | Tool on case 1 (VTM as reference) | Tool off case 1 (BMS as reference) | Tool on case 2 (VTM+ATMVP as reference) | Tool off case 2 (BMS-ATMVP as reference) |
| --- | --- | --- | --- | --- |

## Tool on case 1 (VTM-1.0 as anchor, mandatory)

Table 1 The performance of Test (a), compared to VTM-1.0 anchor

|  | RA, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.75% | -0.68% | 105% | 117% |
| Class A2 | -1.05% | -0.80% | -0.86% | 106% | 127% |
| Class B | -1.01% | -0.75% | -0.68% | 106% | 120% |
| Class C | -1.44% | -1.17% | -1.25% | 105% | 126% |
| Class E |  |  |  |  |  |
| Overall | -1.09% | -0.87% | -0.87% | 105% | 122% |
| Class D | -1.59% | -0.81% | -1.02% | 106% | 129% |

|  | LD, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.63% | -0.77% | -0.42% | 108% | 113% |
| Class C | -1.35% | -1.14% | -0.96% | 105% | 116% |
| Class E | -2.40% | -2.33% | -2.57% | 105% | 119% |
| Overall | -1.31% | -1.28% | -1.14% | 106% | 115% |
| Class D | -1.82% | -1.28% | -1.29% | 105% | 114% |

Table 2 The performance of Test (b), compared to VTM-1.0 anchor

|  | RA, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.84% | -0.83% | -0.71% | 103% | 101% |
| Class A2 | -1.04% | -0.81% | -0.82% | 103% | 105% |
| Class B | -0.99% | -0.72% | -0.65% | 103% | 107% |
| Class C | -1.32% | -0.94% | -1.03% | 103% | 109% |
| Class E |  |  |  |  |  |
| Overall | -1.06% | -0.82% | -0.80% | 103% | 106% |
| Class D | -1.49% | -0.78% | -1.06% | 104% | 110% |

|  | LD, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.65% | -0.83% | -0.57% | 104% | 108% |
| Class C | -1.30% | -1.06% | -0.83% | 103% | 110% |
| Class E | -2.34% | -2.37% | -2.65% | 101% | 109% |
| Overall | -1.29% | -1.29% | -1.17% | 103% | 109% |
| Class D | -1.85% | -1.31% | -1.14% | 103% | 109% |

Table 3 The performance of Test (c), compared to VTM-1.0 anchor

|  | RA, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.80% | -0.74% | -0.65% | 102% | 102% |
| Class A2 | -1.01% | -0.78% | -0.79% | 102% | 103% |
| Class B | -0.96% | -0.69% | -0.65% | 102% | 105% |
| Class C | -1.33% | -1.05% | -1.11% | 102% | 108% |
| Class E |  |  |  |  |  |
| Overall | -1.04% | -0.81% | -0.80% | 102% | 105% |
| Class D | -1.48% | -0.68% | -0.92% | 103% | 110% |

|  | LD, Over VTM-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.64% | -0.76% | -0.51% | 104% | 107% |
| Class C | -1.36% | -1.12% | -0.95% | 103% | 110% |
| Class E | -2.34% | -2.27% | -3.01% | 101% | 108% |
| Overall | -1.30% | -1.26% | -1.28% | 103% | 108% |
| Class D | -1.83% | -1.28% | -1.31% | 102% | 108% |

## Tool off case 2 (BMS-1.0 with ATMVP disabled as anchor, mandatory)

Table 4 The performance of Test (a), compared to BMS-1.0 with ATMVP disabled as anchor

|  | RA, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.61% | -0.65% | -0.57% | 100% | 104% |
| Class A2 | -0.60% | -0.44% | -0.40% | 100% | 104% |
| Class B | -0.56% | -0.43% | -0.52% | 100% | 105% |
| Class C | -1.13% | -1.01% | -1.00% | 99% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.73% | -0.63% | -0.63% | 100% | 105% |
| Class D | -1.18% | -1.00% | -1.05% | 100% | 108% |

|  | LD, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.68% | -0.92% | 101% | 106% |
| Class C | -0.97% | -0.92% | -1.04% | 99% | 107% |
| Class E | -2.20% | -1.75% | -2.80% | 98% | 112% |
| Overall | -1.10% | -1.03% | -1.43% | 99% | 108% |
| Class D | -1.20% | -1.25% | -1.81% | 99% | 106% |

Table 5 The performance of Test (b), compared to BMS-1.0 with ATMVP disabled as anchor

|  | RA, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.55% | -0.48% | 100% | 103% |
| Class A2 | -0.58% | -0.44% | -0.43% | 100% | 100% |
| Class B | -0.54% | -0.38% | -0.47% | 100% | 101% |
| Class C | -1.03% | -0.93% | -0.85% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.57% | -0.57% | 100% | 101% |
| Class D | -1.10% | -0.82% | -0.90% | 99% | 100% |

|  | LD, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.49% | -0.31% | -0.83% | 100% | 102% |
| Class C | -0.88% | -1.06% | -0.95% | 99% | 104% |
| Class E | -2.09% | -2.51% | -2.48% | 97% | 105% |
| Overall | -1.02% | -1.11% | -1.28% | 99% | 104% |
| Class D | -1.18% | -1.29% | -0.77% | 98% | 103% |

Table 6 The performance of Test (c), compared to BMS-1.0 with ATMVP disabled as anchor

|  | RA, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.60% | -0.57% | -0.54% | 100% | 102% |
| Class A2 | -0.57% | -0.43% | -0.40% | 100% | 101% |
| Class B | -0.51% | -0.37% | -0.48% | 100% | 101% |
| Class C | -1.03% | -0.91% | -0.90% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.68% | -0.56% | -0.59% | 99% | 101% |
| Class D | -1.07% | -0.88% | -0.96% | 99% | 99% |

|  | LD, Over BMS-1.0 - ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.55% | -0.67% | -0.92% | 100% | 102% |
| Class C | -0.99% | -0.86% | -1.02% | 98% | 104% |
| Class E | -2.03% | -2.24% | -2.61% | 97% | 105% |
| Overall | -1.07% | -1.13% | -1.38% | 99% | 104% |
| Class D | -1.19% | -1.28% | -1.91% | 98% | 102% |

## Tool on case 2 (VTM-1.0 with ATMVP enabled as anchor, optional)

Table 7 The performance of Test (a), compared to VTM-1.0 with ATMVP enabled as anchor

|  | RA, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.11% | 0.03% | 100% | 100% |
| Class A2 | 0.02% | 0.02% | -0.01% | 100% | 98% |
| Class B | 0.03% | 0.04% | 0.03% | 100% | 101% |
| Class C | -0.03% | -0.12% | -0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | -0.02% | 100% | 100% |
| Class D | 0.01% | 0.07% | 0.14% | 100% | 101% |

|  | LD, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.07% | 0.16% | 100% | 98% |
| Class C | -0.06% | -0.12% | -0.11% | 100% | 100% |
| Class E | 0.04% | 0.06% | 0.35% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.11% | 100% | 98% |
| Class D | 0.03% | 0.01% | -0.11% | 100% | 99% |

Table 8 The performance of Test (b), compared to VTM-1.0 with ATMVP enabled as anchor

|  | RA, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.03% | 0.00% | 98% | 86% |
| Class A2 | 0.04% | 0.01% | 0.04% | 97% | 81% |
| Class B | 0.05% | 0.07% | 0.06% | 97% | 90% |
| Class C | 0.10% | 0.11% | 0.11% | 98% | 86% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.06% | 0.06% | 97% | 86% |
| Class D | 0.12% | 0.10% | 0.11% | 98% | 86% |

|  | LD, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.01% | 0.01% | 97% | 94% |
| Class C | -0.01% | -0.04% | 0.02% | 98% | 95% |
| Class E | 0.10% | 0.02% | 0.25% | 96% | 89% |
| Overall | 0.03% | 0.00% | 0.07% | 97% | 93% |
| Class D | 0.01% | -0.03% | 0.04% | 98% | 95% |

Table 9 The performance of Test (c), compared to VTM-1.0 with ATMVP enabled as anchor

|  | RA, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.13% | 0.06% | 97% | 87% |
| Class A2 | 0.06% | 0.05% | 0.07% | 97% | 80% |
| Class B | 0.09% | 0.10% | 0.07% | 97% | 89% |
| Class C | 0.08% | 0.00% | 0.02% | 97% | 85% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.07% | 0.05% | 97% | 86% |
| Class D | 0.12% | 0.19% | 0.25% | 97% | 86% |

|  | LD, Over VTM-1.0+ATMVP |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | 0.08% | 0.05% | 97% | 93% |
| Class C | -0.07% | -0.10% | -0.11% | 98% | 95% |
| Class E | 0.10% | 0.14% | -0.11% | 96% | 88% |
| Overall | 0.01% | 0.04% | -0.04% | 97% | 92% |
| Class D | 0.03% | 0.00% | -0.13% | 97% | 94% |

## Tool off case 1 (BMS-1.0 as anchor, optional)

Table 10 The performance of Test (a), compared to BMS-1.0 anchor

|  | RA, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.10% | -0.03% | 100% | 99% |
| Class A2 | 0.01% | 0.01% | 0.02% | 100% | 100% |
| Class B | 0.02% | 0.00% | 0.00% | 100% | 99% |
| Class C | -0.01% | 0.01% | -0.09% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.02% | 100% | 99% |
| Class D | 0.03% | -0.11% | -0.07% | 100% | 100% |

|  | LD, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.39% | -0.10% | 101% | 102% |
| Class C | -0.09% | 0.14% | -0.15% | 100% | 100% |
| Class E | -0.13% | 0.70% | -0.34% | 100% | 99% |
| Overall | -0.09% | 0.06% | -0.18% | 100% | 100% |
| Class D | -0.02% | 0.14% | -0.99% | 100% | 100% |

Table 11 The performance of Test (b), compared to BMS-1.0 anchor

|  | RA, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.01% | 0.07% | 100% | 97% |
| Class A2 | 0.03% | 0.01% | 0.00% | 99% | 95% |
| Class B | 0.04% | 0.06% | 0.05% | 99% | 95% |
| Class C | 0.09% | 0.09% | 0.06% | 100% | 94% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.05% | 0.05% | 99% | 95% |
| Class D | 0.11% | 0.07% | 0.09% | 100% | 93% |

|  | LD, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.03% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | -0.07% | 100% | 97% |
| Class E | -0.02% | -0.07% | -0.02% | 99% | 93% |
| Overall | -0.01% | -0.03% | -0.03% | 100% | 97% |
| Class D | 0.00% | 0.10% | 0.06% | 99% | 97% |

Table 12 The performance of Test (c), compared to BMS-1.0 anchor

|  | RA, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.01% | 0.00% | 100% | 96% |
| Class A2 | 0.04% | 0.02% | 0.03% | 99% | 96% |
| Class B | 0.07% | 0.07% | 0.04% | 99% | 95% |
| Class C | 0.09% | 0.12% | 0.01% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.06% | 0.02% | 99% | 95% |
| Class D | 0.14% | 0.01% | 0.02% | 99% | 93% |

|  | LD, Over BMS-1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | -0.38% | -0.09% | 100% | 98% |
| Class C | -0.12% | 0.20% | -0.14% | 99% | 97% |
| Class E | 0.04% | 0.21% | -0.15% | 99% | 93% |
| Overall | -0.05% | -0.04% | -0.12% | 99% | 97% |
| Class D | -0.01% | 0.11% | -1.10% | 99% | 97% |

# References

BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. He, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-J1024, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).