| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0342 |
| --- | --- |

| Title: | CE9.1.3: Complexity reduction on decoder-side motion vector refinement (DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 9.1.3 on complexity reduction of decoder-side motion vector refinement (DMVR). In this CE test, the existing DMVR design in the benchmark set (BMS)-1.0 is used as the base. Additionally, one early termination method is applied to terminate the DMVR process of one bi-prediction merge candidate if its L0 and L1 prediction signals are sufficiently similar. Simulation results show that compared to VTM-1.0 anchors, the proposed DMVR method provides on average {Y, U, V} BD-rate savings of {-2.60%, -2.52%, -2.64%} for RA with average encoding and decoding time of 109% and 119%.

# Introduction

At the 10th JVET meeting, decoder-side motion vector refinement (DMVR) was included in the benchmark set (BMS)-1.0 reference software [1]. The DMVR tool is designed to improve the efficiency of motion compensation by refining the motion vectors (MVs) based on the prediction signals in list L0 and L1. In the BMS-1.0 [1], the DMVR is always enabled for each merge candidate that is bi-predicted, which introduces significant complexity increase to both encoder and decoder. In [2], an early termination method is applied to the DMVR. Specifically, the sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the initial MVs of the current CU is calculated. If the SAD is no larger than a predefined threshold, i.e.,  per sample, the DMVR is skipped; otherwise, the DMVR is still applied to refine the two motion vectors of the current block. The method was included in Core Experiment (9) as test 1.3 for further evaluation [3]. This document reports the simulation results of the CE test.

# Simulation results

The proposed method was implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE9 [3] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

Table 1 The performance of the proposed DMVR method, compared to VTM-1.0 anchor and BMS-1.0 anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.00% | -2.37% | -2.64% | 108% | 113% | 0.09% | 0.02% | 0.07% | 100% | 96% |
| Class A2 | -4.68% | -4.63% | -4.64% | 109% | 123% | 0.06% | 0.07% | 0.03% | 100% | 98% |
| Class B | -2.07% | -2.00% | -2.08% | 109% | 119% | 0.00% | -0.06% | -0.03% | 99% | 96% |
| Class C | -1.38% | -1.68% | -1.82% | 110% | 120% | 0.01% | 0.01% | 0.02% | 99% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.60% | -2.52% | -2.64% | 109% | 119% | 0.03% | 0.00% | 0.02% | 100% | 96% |
| Class D | -1.26% | -1.55% | -1.68% | 113% | 124% | 0.04% | -0.05% | -0.04% | 99% | 97% |

Additionally, one complementary comparison is provided in Table 2 where the results of the proposed method is compared against the results of VTM-1.0 with the DMVR being enabled.

Table 2 The performance of the proposed DMVR method, compared to the results of VTM-1.0 with DMVR being enabled

|  | Over VTM-1.0+DMVR |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.00% | 0.06% | 99% | 91% |
| Class A2 | 0.15% | 0.09% | 0.12% | 98% | 92% |
| Class B | 0.01% | 0.00% | 0.02% | 98% | 91% |
| Class C | 0.00% | 0.01% | -0.02% | 99% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.02% | 0.04% | 98% | 92% |
| Class D | 0.00% | 0.02% | 0.09% | 98% | 92% |

# References

BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. He, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

S. Esenlik, Y.-W. Chen, X. Xiu, A. Robert, X. Chen, T.-D. Chuang, B. Choi, J. Kang, N. Park,, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-J1029, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).