| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0343-v1 |
| --- | --- |

| Title: | CE9.2.2: Simplifications on bilateral matching mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 9.2.2 on simplifications on bilateral matching mode. In this CE test, the bilateral matching mode of the existing frame-rate up conversion (FRUC) in the joint exploration model (JEM)-7.0 is used as the base. Additionally, several simplifications are applied to reduce both the average and worst-case complexity of the bilateral matching mode: 1) one clustering-based motion search method to reduce the number of initial motion vector (MV) candidates; 2) one multi-stage motion refinement to find the optimal MV within a limited number of iterations; 3) one early termination method to conditionally skip the sub-CU-level motion derivation. Simulation results show that compared to VTM-1.0 anchors, the proposed BIO method provides on average {Y, U, V} BD-rate savings of {2.45%, 2.25%, 2.22%} for RA and {1.26%, 1.36%, 1.48%} for LD. The average encoding and decoding time are 125% and 162% for RA, and 142% and 179% for LD.

# Introduction

Bilateral matching is one coding mode of the frame-rate up conversion (FRUC) tool in joint exploration model (JEM)-7.0 [1], which derive the motion vectors (MVs) of one coding unit (CU) by finding the best match between two reference blocks along the motion trajectory of the current CU in two different reference pictures. The bilateral matching based motion derivation is composed of four steps:  CU-level motion search, CU-level motion refinement, sub-CU-level motion search and sub-CU-level motion refinement Although it can improve the coding efficiency, the bilateral matching also introduces significant complexity increase to both encoder and decoder [1].

To reduce the complexity of bilateral matching mode, several simplification methods are proposed in [2], which includes:

- For the CU-level motion search and sub-CU-level motion search, an MV clustering method is proposed to reduce the number of tested MVs searched at these two stages.

- For the CU-level motion refinement and the sub-CU-level motion refinement, a multi-stage motion refinement method is proposed to find the optimal MV within a limited number of search iterations.

- Early termination methods are proposed to conditionally skip the entire sub-CU-level motion derivation process, that is, to skip both the sub-CU-level motion search and the sub-CU-level motion refinement.

The methods were included in Core Experiment (CE) 9 as test 2.2 for further evaluation [3]. This document reports the simulation results of the CE test.

# Technical descriptions

## Clustering-based initial motion search

To reduce the number of initial MV candidates, one clustering-based motion search is proposed, which consists of the following steps:

- All the MV candidates generated from picture-level motion field interpolation are removed from the initial motion candidate list.

- One reference picture selection method is used to pick one L0 refence picture (along with its paired L1 reference picture) which have the highest usage among the initial MV candidates.

- After the initial L0/L1 reference picture is selected, all the initial MV candidates are scaled to the selected reference pictures; additionally, pruning is also performed to remove any redundant MV candidates.

- MV clustering is applied to group similar MV candidates into multiple sets; only the centroids of the resulting MV sets are used as the final MV candidates for the bilateral-matching based motion search at both CU-level and sub-CU-level.

## Multi-stage motion refinement

One multi-stage motion refinement method is applied at both CU-level and sub-CU-level to improve the accuracy of searched MVs, which consists of two steps:

- Integer motion refinement: the initial MV is rounded to the integer precision; the matching costs of a set of integer MVs based on a diamond pattern are calculated and the best integer MV that minimize the cost is maintained as new search center. The above search process continues until either the search center is not updated or the number of the search iterations reaches 12.

- Fractional motion refinement: after the best integer MV is identified, the MV precision is further improved progressively at fractional sample positions (including 1/2-, 1/4- and 1/8-pel precisions) based on the cross search; additionally, the fractional motion refinement is conditionally terminated when the SAD between the two L0/L1 prediction signals are smaller than one pre-defined threshold, i.e.,.

## Early termination of sub-CU-level motion derivation

One early termination method is applied to conditionally skip the sub-CU-level motion derivation process, i.e., sub-CU-level motion search and sub-CU-level motion refinement, when the SAD between two prediction signals at the CU-level is no larger than one predefined threshold, i.e., .

# Simulation results

The proposed method was implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE9 [3] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

Table 1 The performance of the proposed bilateral matching method, compared to VTM-1.0 anchor and BMS-1.0 anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.05% | -1.88% | -2.04% | 118% | 138% | -0.82% | -0.98% | -0.94% | 106% | 105% |
| Class A2 | -2.72% | -2.61% | -2.48% | 123% | 154% | -0.88% | -0.98% | -0.92% | 108% | 107% |
| Class B | -2.27% | -1.94% | -1.89% | 126% | 159% | -0.91% | -0.96% | -0.92% | 107% | 111% |
| Class C | -2.79% | -2.62% | -2.59% | 130% | 196% | -1.40% | -1.46% | -1.50% | 108% | 124% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.45% | -2.25% | -2.22% | 125% | 162% | -1.02% | -1.10% | -1.08% | 107% | 112% |
| Class D | -3.49% | -2.81% | -2.98% | 136% | 227% | -1.95% | -1.87% | -1.84% | 109% | 136% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.96% | -1.23% | -1.28% | 137% | 132% | -0.37% | -1.18% | -0.79% | 112% | 109% |
| Class C | -1.21% | -1.37% | -1.53% | 139% | 159% | -0.61% | -0.98% | -1.48% | 114% | 119% |
| Class E | -1.82% | -1.57% | -1.74% | 138% | 162% | -0.87% | 0.37% | 0.41% | 113% | 130% |
| Overall | -1.26% | -1.36% | -1.48% | 138% | 148% | -0.57% | -0.73% | -0.72% | 113% | 117% |
| Class D | -1.80% | -1.89% | -1.88% | 142% | 179% | -0.81% | -1.49% | -1.98% | 115% | 132% |

# References

J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of joint exploration test mode (JEM7)”, JVET-G1001, August. 2017, Torino, Italy.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. He, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

S. Esenlik, Y.-W. Chen, X. Xiu, A. Robert, X. Chen, T.-D. Chuang, B. Choi, J. Kang, N. Park,, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-J1029, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).