| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0344-v1 |
| --- | --- |

| Title: | CE9.5.3: Bi-directional optical flow (BIO) simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 9.5.3 on complexity reduction of bi-directional optical flow (BIO). In this CE test, the existing BIO design in the joint exploration model (JEM)-7.0 is used as the base. Additionally, one two-stage early termination method is used to conditionally disable the BIO based on the similarity between two bi-prediction signals at both CU and sub-CU levels. Simulation results show that compared to VTM-1.0 anchors, the proposed BIO method provides on average {Y, U, V} BD-rate savings of {2.61%, 1.02%, 0.74%} for RA with average encoding and decoding time of 111% and 137%.

# Introduction

Bi-directional optical flow (BIO) is one coding tool that is included in joint exploration model (JEM)-7.0 [1]. The BIO tool was designed to enhance the conventional motion compensation (MC) by improving both the granularity and the accuracy of the motion vectors (MVs) that are used at the MC stage. Although the BIO can improve the coding efficiency, it also introduces significant complexity increase to both encoder and decoder, because it is applied to bi-predicted coding units (CUs) in all inter coding modes when the conditions that enable the BIO are satisfied [1].

In [2], a two-stage early termination method is used to conditionally disable the BIO operations depending on the similarity between the two prediction signals. The early termination is first applied at the CU-level and then at the sub-CU-level. Specifically, the proposed method first calculates the SAD between the L0 and L1 prediction signals at the CU level. Given that the BIO is only applied to luma [1], only the luma samples need to be considered for the SAD calculation. If the CU-level SAD is no larger than a predefined threshold, the BIO process is completely disabled for the whole CU. The CU-level threshold is set to   per sample. If the BIO process is not disabled at the CU level, and if the current CU contains multiple sub-CUs, the SAD of each sub-CU inside the CU will be calculated. Then, the decision on whether to enable or disable the BIO process is made at the sub-CU-level based on a predefined sub-CU-level SAD threshold, which is set to  per sample. Additionally, in [2], the BIO is always disabled when the current picture is one low-delay (LD) picture. The method was included in Core Experiment (CE) 9 as test 5.3 for further evaluation [3]. This document reports the simulation results of the CE test.

# Simulation results

The proposed method was implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE9 [3] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

Table 1 The performance of the proposed DMVR method, compared to VTM-1.0 anchor and BMS-1.0 anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.31% | -0.79% | -0.51% | 110% | 135% | -0.68% | -0.32% | -0.22% | 103% | 108% |
| Class A2 | -3.87% | -0.93% | -0.39% | 110% | 135% | -1.81% | -0.63% | -0.50% | 103% | 109% |
| Class B | -2.52% | -1.00% | -0.72% | 112% | 139% | -1.15% | -0.49% | -0.44% | 103% | 106% |
| Class C | -2.76% | -1.27% | -1.20% | 112% | 138% | -1.43% | -0.73% | -0.74% | 103% | 110% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.61% | -1.02% | -0.74% | 111% | 137% | -1.26% | -0.55% | -0.49% | 103% | 108% |
| Class D | -4.86% | -2.27% | -2.11% | 112% | 139% | -2.18% | -1.31% | -1.01% | 103% | 108% |

# References

J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of joint exploration test mode (JEM7)”, JVET-G1001, August. 2017, Torino, Italy.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. He, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

S. Esenlik, Y.-W. Chen, X. Xiu, A. Robert, X. Chen, T.-D. Chuang, B. Choi, J. Kang, N. Park,, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-J1029, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).