| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K345-v1 |
| --- | --- |

| Title: | CE10.2.2: Complexity reduction for over-lapped block motion compensation (OBMC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This document reports the results of Core Experiment (CE) 10.2.2 on simplifications on overlapped block motion compensation (OBMC). In this CE test, the OBMC design in the joint exploration model (JEM)-7.0 is used as the base. Additionally, two simplifications are proposed to reduce its complexity: 1) block grouping is applied in order toto reduce the number of motion compensations invoked during the OBMC; 2) skip some motion compensation steps in the OBMC based on the difference between the MVs of the current block and those of its spatial neighbor. Compared to VTM-1.0, with the combination of all the proposed simplifications, the average {Y, U, V} BD-rate savings are {0.96%, 1.91%, 1.98%} for RA and {1.37%, 1.80%, 1.90%} for LD, respectively. The average encoding and decoding time are 107% and 111% for RA, and 110% and 112% for LD.

# Introduction

Overlapped block motion compensation (OBMC) is one coding tool in joint exploration model (JEM)-7.0 [1], which reduce the blocking artifacts at the motion compensation stage by combining multiple prediction signals using the motion vectors (MVs) of spatial neighboring blocks of the current coding unit (CU). Although it can improve the coding efficiency, the OBMC also introduces complexity increase to both encoder and decoder [1]. To reduce the OBMC complexity, two simplifications are proposed in [2], which includes:

- Block grouping is applied in order toto reduce the number of motion compensations invoked during the OBMC.

- Skip some motion compensation steps in the OBMC based on the difference between the MVs of the current block and those of its spatial neighbor.

The methods were included in Core Experiment (CE) 10 as test 2.2 for further evaluation [3]. This document reports the simulation results of the CE test

# Technical description

## Block grouping in OBMC

Different from the OBMC in JEM-7.0 where the OBMC is applied to the 4×4 sub-blocks inside a CU in the raster scan order, the OBMC process in this response is performed in two steps. In the first step, referred to as “boundary OBMC,” the samples on the top/left boundaries of the current CU are adjusted using the MVs from its spatial neighboring CUs. If the CU is not coded in sub-block mode, the OBMC process ends after the first step. Otherwise (if the CU is coded in sub-block mode), the second step, referred as “internal OBMC,” is performed. In the second step, the samples on the top, left, bottom and right boundaries of each sub-block, except the samples on the CU boundaries that have already been adjusted in the first step, are adjusted using the MVs of its neighboring sub-blocks in the same CU. Note that due to the modified OBMC process, the order in which the boundary samples of the current CU are adjusted during the OBMC process is different from that in the JEM-7.0. Assuming the current CU is in the size of 16×4, Figure 1 compares the OBMC process in JEM-7.0 and the proposed OBMC process, where the numbers specify the order in which the boundary samples within the CU are processed. As shown in Figure 1, the OBMC using the MVs of the four neighboring sub-blocks on top of the current CU are grouped into the boundary OBMC process, i.e., OBMC #0, #3, #6 and #9 in JEM-7.0 now become OBMC #0, #1, #2 and #3 in the proposed process. Given that some of those sub-blocks may belong to the same neighboring CU, their MVs are more likely to be identical, in which case the OBMC using those MVs can be combined to one single MC with a larger block size for reducing the complexity.  Based on such consideration, block grouping is applied to reduce the number of motion compensations invoked during the proposed OBMC process. Unlike the JEM-7.0 where the OBMC is always performed on 4×4 units, variable sub-block sizes are used in the proposed method. Specifically, for the boundary OBMC process, the adjacent 4×4 neighboring blocks that are located along the same CU boundary (i.e., either top or left) and have the same motion are grouped into one block group; for each block group, only one single MC is performed. For the internal OBMC process, it is proposed to align the sub-block size with that used for the sub-block-level motion derivation. Figure 2 illustrates the proposed OBMC process where the bold arrows and the patterned areas indicate the used MVs and the sample areas that are adjusted at each step.

Figure 1. The order of adjusting boundary samples during the OBMC process in JEM-7.0 (left) and the proposed OBMC process (right).

Figure 2. Illustration of the proposed OBMC process

## Conditional skipping of motion compensation during the OBMC

Some motion compensations are skipped during the OBMC process based on the motion similarity between one current block group and its neighboring block groups. Specifically, each time before motion compensation is invoked using the motion information from a given neighboring block group, the neighboring motion is compared to the current block’s motion based on the following three conditions:

- If the neighboring motion and the current motion are both predicted using uni-pred or both predicted using bi-pred;

- If the same reference picture(s) are used by the neighboring motion and the current motion;

- If MV difference between the neighboring motion and the current motion is no larger than 1/2-pel (both L0 and L1 MVs are checked if bi-pred is used).

If all the 3 conditions are met, then motion compensation is not performed (this is applicable to luma and chroma) using that given neighboring motion.

# Simulation results

The proposed method was implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE10 [3] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.41% | -1.62% | -1.84% | 104% | 108% | -0.43% | -1.03% | -1.19% | 101% | 104% |
| Class A2 | -1.00% | -1.86% | -1.72% | 107% | 111% | -1.17% | -2.11% | -2.71% | 103% | 110% |
| Class B | -0.91% | -1.83% | -1.84% | 108% | 110% | -1.11% | -2.30% | -2.15% | 102% | 107% |
| Class C | -1.40% | -2.26% | -2.47% | 109% | 114% | -1.67% | -2.81% | -2.80% | 102% | 111% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.96% | -1.91% | -1.98% | 107% | 111% | -1.14% | -2.14% | -2.24% | 102% | 108% |
| Class D | -1.32% | -2.05% | -1.94% | 112% | 114% | -1.74% | -2.62% | -2.74% | 104% | 113% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.21% | -1.66% | -1.86% | 110% | 110% | -1.40% | -2.62% | -2.33% | 105% | 115% |
| Class C | -1.64% | -2.11% | -2.07% | 112% | 116% | -2.07% | -3.02% | -3.27% | 104% | 116% |
| Class E | -1.30% | -1.62% | -1.74% | 109% | 108% | -2.10% | -3.27% | -2.16% | 105% | 114% |
| Overall | -1.37% | -1.80% | -1.90% | 110% | 112% | -1.80% | -2.92% | -2.60% | 105% | 115% |
| Class D | -1.89% | -2.20% | -2.09% | 113% | 119% | -2.32% | -2.88% | -3.46% | 106% | 121% |

# References

J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of joint exploration test mode (JEM7)”, JVET-G1001, August. 2017, Torino, Italy.

X. Xiu, P. Hanhart, R. Vanam, Y. He, Y. He, “Description of SDR, HDR and 360° video coding technology proposal by InterDigital Communications and Dolby Laboratories”, JVET-J0015, April 2018, San Diego, USA.

C.-W. Hsu, M. Winken, X. Xiu, “Description of Core Experiment 10 (CE9): Combined and multi-hypothesis prediction”, JVET-J1030, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).