| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0347-v1 |
| --- | --- |

| Title: | CE9-related: Decoding latency improvement for decoder-side motion vector refinement (DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Decoder-side motion vector refinement (DMVR) is included in the benchmark set (BMS)-1.0. The DMVR needs to generate L0 and L1 prediction signals to derive the refined motion vectors (MVs) of one bi-predicted merge candidate. In the current DMVR design, the refined MVs are used to predict the neighboring MVs of the current coding unit (CU). Such design is reported to have decoding latency issue due to the interdependency among the decoding of spatial neighboring CUs.

In this contribution, two solutions are provided to address DMVR’s decoding latency issue. In solution one, instead of using the refined MVs, it is proposed to use the original MVs (i.e., unrefined MVs) to predict the MVs of a DMVR CU’s spatial neighbors. Solution one removes the decoding latency of the DMVR. In solution two, it is proposed to use the unrefined MVs to predict the MVs of a DMVR CU’s neighboring CUs that are in the same coding tree unit (CTU). But, for the neighboring CUs that are not in the same CTU, the refined MVs are used as the predictor. This method allows independent decoding of multiple inter CUs inside one CTU, but not across different CTUs. Additionally, for both solutions, the unrefined MVs are used for deblocking, and the refined MVs are temporal motion vector prediction.

Experimental results show that compared to VTM-1.0 anchor, solution one provides average {Y, U, V} BD-rate savings of {1.41%, 1.45%, 1.47%} for the RA configuration with the encoding and decoding time of 112% and 130%. For solution two, the corresponding {Y, U, V} BD-rate savings are {1.71%, 1.67%, 1.73%} with the encoding and decoding time of 111% and 129%.

# Problem statement

At the 10th JVET meeting, decoder-side motion vector refinement (DMVR) was included in the benchmark set (BMS)-1.0 reference software [1]. To reduce the signaling overhead, in the current DMVR design, the motion refinements are derived at both encoder and decoder using the L0 and L1 prediction signals of each merge candidate that is bi-predicted. Moreover, the refined MVs of each DMVR coding unit (CU) are used to predict the MVs of its spatial and temporal neighboring CUs and calculate the boundary strength of deblocking filter.

Although it can provide good coding gains, the existing DMVR design is reported to have decoding latency issue, because of the interdependency that is introduced between the coding of spatial neighboring CUs. If the MV of the current CU is predicted from one of its spatial neighbors which is coded by the DMVR mode, its decoding cannot start until the refined MVs of the neighboring CU are fully reconstructed through the DMVR.

# Proposal

Given the problem mentioned above, two solutions are proposed in this contribution to address the decoding latency issue of the current DMVR design. Specifically, the main modules of the DMVR method, e.g., the generation of the bi-prediction template and the block-matching based motion refinement, are kept unchanged as the existing design. But, the MVs of one DMVR CU that are used for spatial/temporal motion vector prediction and deblocking are modified in the proposed solutions.

## Solution one

In the first solution, instead of using the refined MVs, it is proposed to use the unrefined MVs of each DMVR CU to derive the spatial MV predictors (e.g., spatial advance motion vector prediction (AMVP) predictor and spatial merge candidate) of its neighboring CUs. For the temporal motion prediction methods, i.e., temporal motion vector prediction (TMVP) and advanced temporal motion vector prediction (ATMVP), the refined MVs are used; this does not have latency issue, because the collocated picture has been already fully reconstructed. Finally, to avoid increasing the on-chip memory by caching both the unrefined and refined MVs, it is proposed to use the same DMVR MVs that are used for the spatial motion prediction (i.e., the unrefined MVs) for the deblocking process. Figure 1 illustrates idea of the first solution, where unrefined MVs are used for “spatial AMVP”, “spatial merge candidates” and “deblocking”, and refined MVs are used for “TMVP” and “ATMVP”. As illustrated in Figure 1, decoding latency of the DMVR can be fully removed as the coding dependency between spatial neighboring CUs no longer exists. In this solution, only unrefined MVs need to be stored for spatial MV prediction.

`

Figure 1 Illustration of solution one

## Solution two

Although solution one can completely remove the decoding latency of the DMVR, it comes with the degraded coding efficiency because less accurate unrefined MVs are used for the spatial motion prediction. On the other hand, the worst-case decoding latency of the DMVR depends the maximum number of consecutive CUs that are coded by the DMVR mode. Therefore, to reduce the DMVR decoding latency while lowering the coding loss of solution one, the second solution uses the unrefined MVs of one DMVR CU to predict the MVs of it neighboring CUs that are in the same CTU. But, for the neighboring CUs that are not in the same CTU, the refined MVs of the current CU are used as the predictor to achieve a better MV prediction accuracy. Additionally, like solution one, the refined MVs and the unrefined MVs are used for the TMVP/ATMVP and deblocking filter, respectively, in the second solution. Figure 2 illustrates idea of the second solution. As shown in Figure 2, with solution two, the decoding of multiple inter CUs inside one CTU can be performed in parallel while the decoding of the CUs of the current CTU still depends on that of neighboring CTUs. In this solution, if a CU is located on the bottom or right boundary of the CTU, then both refined and unrefined MVs need to be stored for spatial MV prediction. For all other CUs, only the unrefined MVs need to be stored for spatial MV prediction.

Figure 2 Illustration of solution two

# Simulation results

To evaluate the performance/complexity impact, the proposed two solutions are implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE9 [2] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

## Solution one

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.48% | -1.27% | -1.41% | 110% | 126% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | -2.51% | -2.49% | -2.50% | 112% | 132% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -1.12% | -1.21% | -1.18% | 112% | 130% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.90% | -1.09% | -1.12% | 114% | 131% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.41% | -1.45% | -1.47% | 112% | 130% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.86% | -0.99% | -1.21% | 116% | 133% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

## Solution two

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | -1.66% | -1.82% | 167% | 124% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | -3.03% | -2.91% | -2.96% | 166% | 132% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -1.34% | -1.39% | -1.35% | 162% | 130% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.92% | -1.10% | -1.22% | 158% | 129% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.71% | -1.67% | -1.73% | 163% | 129% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.89% | -1.07% | -1.11% | 161% | 132% | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |

# References

BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

S. Esenlik, Y.-W. Chen, X. Xiu, A. Robert, X. Chen, T.-D. Chuang, B. Choi, J. Kang, N. Park,, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-J1029, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).