| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0347-v2 |
| --- | --- |

| Title: | CE9-related: Addressing the decoding latency issue for decoder-side motion vector refinement (DMVR) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaoyu Xiu, Yuwen He, Yan Ye 9276 Scranton Road, Suite 300 San Diego, CA 92121, USA | Tel: Email: | +1-858-210-4830 Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Yan.Ye@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

Decoder-side motion vector refinement (DMVR) is included in the benchmark set (BMS)-1.0. The DMVR needs to generate L0 and L1 prediction signals to derive the refined motion vectors (MVs) of one bi-predicted merge candidate. In the current DMVR design, the refined MVs are used to predict the neighboring MVs of the current coding unit (CU). Such design is reported to have decoding latency issue due to the interdependency among the decoding of spatial neighboring CUs.

In this contribution, three solutions are provided to address DMVR’s decoding latency issue. In solution one, instead of using the refined MVs, it is proposed to use the original MVs (i.e., unrefined MVs) to predict the MVs of a DMVR CU’s spatial neighbors. Solution one removes the decoding latency of the DMVR. In solution two, it is proposed to use the unrefined MVs to predict the MVs of a DMVR CU’s neighboring CUs that are in the same coding tree unit (CTU). But, for the neighboring CUs that are not in the same CTU, the refined MVs are used as the predictor. Solution two allows independent decoding of multiple inter CUs inside one CTU, but not across different CTUs. In solution three, it is proposed to use the unrefined MVs of one DMVR CU to predict the MVs of its neighboring CU that is in the same CTU row. But, for the neighboring CUs that is from one different CTU row, the refined MVs are used for spatial MV prediction. The third solution allows parallel decoding of multiple inter CUs within the same CTU row while the decoding of the CUs on the top boundary of one CTU is still dependent on that of the top neighboring CTU. Additionally, for both solutions, the unrefined MVs are used for deblocking, and the refined MVs are temporal motion vector prediction.

Experimental results show that compared to VTM-1.0 anchor, solution one provides average {Y, U, V} BD-rate savings of {1.41%, 1.45%, 1.47%} for the RA configuration with the encoding and decoding time of 112% and 130%. For solution two, the corresponding {Y, U, V} BD-rate savings are {1.71%, 1.67%, 1.73%} with the encoding and decoding time of 111% and 129%. For solution three, the corresponding {Y, U, V} BD-rate savings are {1.58%, 1.56%, 1.60%} with the encoding and decoding of 111% and 132%.

# Problem statement

At the 10th JVET meeting, decoder-side motion vector refinement (DMVR) was included in the benchmark set (BMS)-1.0 reference software [1]. To reduce the signaling overhead, in the current DMVR design, the motion refinements are derived at both encoder and decoder using the L0 and L1 prediction signals of each merge candidate that is bi-predicted. Moreover, the refined MVs of each DMVR coding unit (CU) are used to predict the MVs of its spatial and temporal neighboring CUs and calculate the boundary strength of deblocking filter.

Although it can provide good coding gains, the existing DMVR design is reported to have decoding latency issue, because of the interdependency that is introduced between the coding of spatial neighboring CUs. If the MV of the current CU is predicted from one of its spatial neighbors which is coded by the DMVR mode, its decoding cannot start until the refined MVs of the neighboring CU are fully reconstructed through the DMVR.

# Proposal

Given the problem mentioned above, two solutions are proposed in this contribution to address the decoding latency issue of the current DMVR design. Specifically, the main modules of the DMVR method, e.g., the generation of the bi-prediction template and the block-matching based motion refinement, are kept unchanged as the existing design. But, the MVs of one DMVR CU that are used for spatial/temporal motion vector prediction and deblocking are modified in the proposed solutions.

## Solution one

In the first solution, instead of using the refined MVs, it is proposed to use the unrefined MVs of each DMVR CU to derive the spatial MV predictors (e.g., spatial advance motion vector prediction (AMVP) predictor and spatial merge candidate) of its neighboring CUs. For the temporal motion prediction methods, i.e., temporal motion vector prediction (TMVP) and advanced temporal motion vector prediction (ATMVP), the refined MVs are used; this does not have latency issue, because the collocated picture has been already fully reconstructed. Finally, to avoid increasing the on-chip memory by caching both the unrefined and refined MVs, it is proposed to use the same DMVR MVs that are used for the spatial motion prediction (i.e., the unrefined MVs) for the deblocking process. Figure 1 illustrates idea of the first solution, where unrefined MVs are used for “spatial AMVP”, “spatial merge candidates” and “deblocking”, and refined MVs are used for “TMVP” and “ATMVP”. As illustrated in Figure 1, decoding latency of the DMVR can be fully removed as the coding dependency between spatial neighboring CUs no longer exists. In this solution, only unrefined MVs need to be stored for spatial MV prediction.

`

Figure 1 Illustration of solution one

## Solution two

Although solution one can completely remove the decoding latency of the DMVR, it comes with the degraded coding efficiency because less accurate unrefined MVs are used for the spatial motion prediction. On the other hand, the worst-case decoding latency of the DMVR depends the maximum number of consecutive CUs that are coded by the DMVR mode. Therefore, to reduce the DMVR decoding latency while lowering the coding loss of solution one, the second solution uses the unrefined MVs of one DMVR CU to predict the MVs of it neighboring CUs that are in the same CTU. But, for the neighboring CUs that are not in the same CTU, the refined MVs of the current CU are used as the predictor to achieve a better MV prediction accuracy. Additionally, like solution one, the refined MVs and the unrefined MVs are used for the TMVP/ATMVP and deblocking filter, respectively, in the second solution. Figure 2 illustrates idea of the second solution. As shown in Figure 2, with solution two, the decoding of multiple inter CUs inside one CTU can be performed in parallel while the decoding of the CUs of the current CTU still depends on that of neighboring CTUs. In this solution, if a CU is located on the bottom or right boundary of the CTU, then both refined and unrefined MVs need to be stored for spatial MV prediction. For all other CUs, only the unrefined MVs need to be stored for spatial MV prediction.

Figure 2 Illustration of solution two

## Solution three

In the second solution, the refined MVs are used for the spatial MV prediction across the left and right boundaries of one CTU. Therefore, the decoding of multiple CTUs in one same CTU row are still dependent. To further increase the decoding parallelism, in the third solution, it is proposed to use the unrefined MVs of one DMVR CU to predict the MVs of its neighboring CU that is in the same CTU row. For the neighboring CUs located in different CTU rows, the refined MVs are used for spatial MV prediction. By this way, multiple CUs within the same CTU row can be decoded in parallel while the decoding of the CUs on the top boundary of one CTU is still dependent on that of the top neighboring CTUs. Figure 3 illustrates idea of the third solution. As illustrated in Figure 3, solution three only needs to store both refined and unrefined MVs for the CUs located on the bottom boundary of one CTU. For all the other CUs, only the unrefined MVs need to be stored.

Figure 3 Illustration of solution three

# Simulation results

To evaluate the performance/complexity impact, the proposed three solutions are implemented based on the BMS-1.0 reference software [1] and tested using both VTM and BMS configurations. The test condition and evaluation criteria that are used in CE9 [2] are applied. Specifically, the proposed method is compared with two anchors: for the VTM test, the anchor is the BD-rate performance of VTM-1.0; for the BMS test, the anchor is the BD-rate performance of BMS-1.0.

## Solution one

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.48% | -1.27% | -1.41% | 110% | 126% | 0.91% | 0.63% | 0.76% | 100% | 100% |
| Class A2 | -2.51% | -2.49% | -2.50% | 112% | 132% | 1.04% | 1.06% | 1.08% | 101% | 101% |
| Class B | -1.12% | -1.21% | -1.18% | 112% | 130% | 0.39% | 0.34% | 0.40% | 100% | 96% |
| Class C | -0.90% | -1.09% | -1.12% | 114% | 131% | 0.32% | 0.45% | 0.41% | 100% | 87% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.41% | -1.45% | -1.47% | 112% | 130% | 0.60% | 0.57% | 0.61% | 100% | 95% |
| Class D | -0.86% | -0.99% | -1.21% | 116% | 133% | 0.31% | 0.32% | 0.37% | 100% | 92% |

## Solution two

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.06% | -1.66% | -1.82% | 167% | 124% | 0.53% | 0.46% |  | 0.53% |  | 100% | 100% |  |
| Class A2 | -3.03% | -2.91% | -2.96% | 166% | 132% | 0.74% | 0.73% | 0.82% | 100% | 102% |
| Class B | -1.34% | -1.39% | -1.35% | 162% | 130% | 0.28% | 0.29% | 0.32% | 100% | 100% |
| Class C | -0.92% | -1.10% | -1.22% | 158% | 129% | 0.31% | 0.49% | 0.44% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.71% | -1.67% | -1.73% | 163% | 129% | 0.43% | 0.47% | 0.49% | 100% | 100% |
| Class D | -0.89% | -1.07% | -1.11% | 161% | 132% | 0.26% | 0.32% | 0.57% | 100% | 101% |

## Solution three

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | -1.45% | -1.64% | 109% | 128% | 0.68% | 0.50% | 0.59% | 98% | 99% |
| Class A2 | -2.80% | -2.78% | -2.75% | 111% | 133% | 0.85% | 0.80% | 0.91% | 99% | 100% |
| Class B | -1.24% | -1.27% | -1.26% | 112% | 133% | 0.33% | 0.33% | 0.35% | 100% | 100% |
| Class C | -0.90% | -1.11% | -1.14% | 113% | 133% | 0.30% | 0.48% | 0.43% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.58% | -1.56% | -1.60% | 111% | 132% | 0.49% | 0.50% | 0.53% | 100% | 100% |
| Class D | -0.87% | -0.98% | -1.22% | 116% | 136% | 0.29% | 0.31% | 0.33% | 101% | 100% |

## Complementary results on disabling 4x4/8x4/4x8 CUs for the DMVR

Based on the current DMVR design [1], additional samples (2 additional rows and columns of the CU) need to be accessed from the reference pictures. This results in memory bandwidth increase of the DMVR CUs, especially in small CU sizes. In the table below, additional testing results are provided where the anchor is performance of the VTM-1.0 plus the DMVR in BMS-1.0 being enabled and the test is performance of the anchor with disabling the DMVR for the CUs in the sizes of 4x4, 8x4 and 4x8. The encoding and decoding are not reliable due to the heterogeneous nature of the cluster system used for the simulation.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 + DMVR |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | 0.01% | -0.02% | 142% | 121% |
| Class A2 | -0.04% | 0.07% | 0.07% | 143% | 122% |
| Class B | 0.06% | 0.11% | 0.13% | 141% | 122% |
| Class C | 0.10% | 0.12% | 0.13% | 139% | 129% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.09% | 141% | 124% |
| Class D | 0.18% | 0.21% | 0.13% | 138% | 133% |

# References

BMS-1.0 reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/.

S. Esenlik, Y.-W. Chen, X. Xiu, A. Robert, X. Chen, T.-D. Chuang, B. Choi, J. Kang, N. Park,, “Description of Core Experiment 9 (CE9): Decoder-side motion vector derivation”, JVET-J1029, April 2018, San Diego, USA.

# Patent rights declaration(s)

InterDigital Communications Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).