| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0348-v1 |
| --- | --- |

| Title: | CE3-related: On the combination of multiple reference lines, bilateral reference line filtering, PDPC and 65 directional modes for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Adarsh K. Ramasubramonian Vadim Seregin Marta Karczewicz Benjamin Bross Philipp Merkle Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com firstname.lastname@hhi.fraunhofer.de |
| Source: | Qualcomm Inc., Fraunhofer HHI |  |  |

_____________________________

# Abstract

It is proposed to combine tools CE3-5.4.4 (multiple reference sample rows for intra prediction), CE3-2.7.1 (bilateral reference sample filter), CE3-2.4.1 (simplified PDPC), and 65 intra directional modes.  It is demonstrated that the gains of the tools are additive with a BD-rate improvement of -3.16% (Y), -1.77% (U), -1.73% (V) with runtimes of 188% (Enc) and 107% (Dec) for AI configuration over BMS-VTM-1.0, and -2.03% (Y), -1.31% (U), -1.17% (V) with runtimes of 120% (Enc) and 100% (Dec) for AI configuration over BMS-1.0. For the random-access configuration, the BD-rate improvement is -1.57% (Y), -0.27% (U), -0.35% (V) over BMS-VTM-1.0. The source code of this tool combination is provided together with this proposal.

# Results

All intra and random-access results for the combination of tools CE3-2.4.1 (simplified PDPC), CE3-5.4.4 (multiple reference sample rows for intra prediction), CE3-2.7.1 (bilateral reference sample filter) and 65 intra directional modes are provided.

## Over BMS-VTM-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.96% | -1.87% | -1.43% | 194% | 107% |
| Class A2 | -2.72% | -2.15% | -1.63% | 188% | 104% |
| Class B | -3.21% | -1.58% | -1.84% | 190% | 111% |
| Class C | -2.89% | -0.60% | -0.82% | 183% | 102% |
| Class E | -4.06% | -3.13% | -3.18% | 187% | 108% |
| Overall | -3.16% | -1.77% | -1.73% | 188% | 107% |
| Class D | -1.58% | 0.15% | 0.34% | 183% | 109% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -1.34% | -1.12% | 120% | 97% |
| Class A2 | -1.59% | -0.66% | -0.13% | 113% | 96% |
| Class B | -1.65% | 0.00% | -0.57% | 115% | 101% |
| Class C | -1.38% | 0.49% | 0.35% | 115% | 103% |
| Overall | -1.57% | -0.27% | -0.35% | 116% | 100% |
| Class D | -0.76% | 0.66% | 0.97% | 114% | 100% |

## Over BMS-1.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.12% | -2.52% | -2.04% | 120% | 100% |
| Class A2 | -1.97% | -1.45% | -1.18% | 121% | 100% |
| Class B | -2.02% | -1.01% | -0.93% | 121% | 98% |
| Class C | -1.78% | -0.80% | -0.73% | 117% | 100% |
| Class E | -2.36% | -1.11% | -1.27% | 120% | 103% |
| Overall | -2.03% | -1.31% | -1.17% | 120% | 100% |
| Class D | -1.07% | 0.04% | 0.24% | 117% | 100% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.15% | -0.14% | -0.41% | 113% | 93% |
| Class C | -0.87% | -0.33% | -0.45% | 111% | 99% |
| Overall |  |  |  |  |  |
| Class D | -0.57% | 0.06% | 0.38% | 112% | 98% |

# Patent rights declarations

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).