| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0348-v3 |
| --- | --- |

| Title: | CE3-related: On the combination of multiple reference lines, bilateral reference line filtering, PDPC and 65 directional modes for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross Philipp Merkle Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes combining tools described in CE3-5.4.4 (multiple reference lines for intra prediction), CE3-2.7.1 (bilateral reference sample filter), CE3-2.4.1 (simplified PDPC), and 65 intra directional modes. It is asserted that the gains of the tools are additive with a BD-rate improvement of −3.16% (Y), −1.77% (U), −1.73% (V) with runtimes of 188% (Enc) and 107% (Dec) for AI configuration over BMS-VTM-1.0, and −2.03% (Y), −1.31% (U), −1.17% (V) with runtimes of 120% (Enc) and 100% (Dec) for AI configuration over BMS-1.0. For the random-access configuration, the BD-rate improvement is −1.57% (Y), −0.27% (U), −0.35% (V) with runtimes of 116% (Enc) and 100% (Dec) over BMS-VTM-1.0 and −1.1% (Y), −0.6% (U), − 0.6 % (V) with runtimes of 112% (Enc) and 101 % (Dec) over BMS-1.0. The source code of this tool combination is provided together with this proposal. In addition, informative results for combining the intra tools with more intra tools and proposed tools from transform and quantization showing that BD-rate savings resulting from these combinations are additive as well.

# Introduction

The core experiment CE3 Intra Prediction and Mode Coding (JVET-J1023 [1]) investigates several tools that improve the coding efficiency of intra coding. Although most of these tools are tested alone, or in combination with one or two minor tools, it is important to look at the performance of these tools in combination. This proposal attempts to combine a subset of major tools that addresses several categories of intra mode coding, and it is demonstrated that each of these tools brings additive gains; it is therefore presented as a good combination to be included in the VVC draft and software.

The four tools combined here are:

- MRL: multiple reference line intra prediction as tested in CE3 5.4.4 (JVET-K0051 [2]) – the reference samples used for intra prediction in HEVC are obtained from the nearest lines of neighbouring blocks (above and left) of the current CU. In the proposed tool, the reference samples for angular luma prediction modes may be derived from one of any three reference lines that are in the neighbourhood of the current CU. For example, the nearest line (pixel distance 0 from the corresponding edge of the current CU), and lines at pixel distance of 1 and 3 are candidates for obtaining the reference samples. The reference line that is chosen is signalled in the bitstream.

- BiFi: bilateral reference sample filter as tested in CE3 2.7.1 (JVET-K0043 [3])  – this tool proposes a bilateral filter to smooth the reference samples used for intra prediction. The use of the bilateral filter reduces noise while preserving edges and is proposed as a replacement to the 3-tap [1 2 1]/4 filter currently used. The bilateral filter weights are stored as pre-computed 8-bit integers. The bilateral filter is applied to luma blocks with size  and all prediction modes  with .

- Simp. PDPC: simplified position dependent intra prediction combination (PDPC) as tested in CE3 2.4.1 (JVET-K0063 [4]) is applied to the following intra modes without signalling: planar, DC, horizontal, vertical, bottom-left angular mode and its x adjacent angular modes, and top-right angular mode and its x adjacent angular modes where x is equal to 4 or 8 for respectively 33 (VTM1) or 65 (BMS1) angular modes. PDPC computes a linear combination of neighboring reference samples with weights dependent on the position within the block. The weights are computed by using shift arithmetic only and no divisions nor special look-up tables are used. With PDPC prediction, additional across the boundary filtering (secondary boundary) is not needed.

- Intra65: 65 intra directional modes - The JEM7 [5] model describes intra mode coding with 65 directional modes (totalling 67 modes including DC and PL) and six entries in the MPM list. The 65 directional modes are depicted in Figure 1 (black and red dotted arrows). When the mode used for coding is not one of the selected modes, the value of the remaining mode is signalled.

Figure 1: The 65 directional modes used in JEM7 are depicted using black and red dotted arrows. Along with DC and Planar, a total of 67 modes are used.

The results are summarized in Table 1 and Table 2 wherein the individual results for each tool are taken from the respective CE3 documents JVET-K0051, JVET-K0043 and JVET-K0063, except for Intra65 where the results are taken from the AHG13 activity with results available at:

https://hevc.hhi.fraunhofer.de/svn/svn_VVCTestConfig/branches/VTM-1.0/Results/JVET-BMS1_65IPM_tester.XLSM

Detailed results for the combination of all four tools (All) are presented in section 4 with software settings to generate the results listed in section 2.

Table 1 Average BD-rates and relative runtimes for VTM configuration

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| MRL | -0.7% | -0.3% | -0.3% | 147% | 101% | -0.4% | -0.2% | -0.3% | 108% | 101% |
| BiFi | -0.2% | -0.1% | -0.1% | 101% | 101% | -0.1% | -0.0% | -0.0% | 100% | 102% |
| Simp. PDPC | -1.0% | -0.1% | -0.1% | 109% | 107% | -0.5% | 0.4% | 0.5% | 101% | 101% |
| Intra65 | -1.2% | -1.3% | -1.3% | 115% | 102% | -0.6% | -0.4% | -0.4% | 103% | 101% |
| All | -3.2% | -1.8% | -1.7% | 188% | 107% | -1.6% | -0.3% | -0.4% | 116% | 100% |

Table 2 Average BD-rates and relative runtimes for BMS configuration

| BMS | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| MRL | -0.5% | -0.1% | -0.1% | 117% | 100% | -0.3% | -0.0% | -0.1% | 110% | 100% |
| BiFi | -0.2% | -0.2% | -0.2% | 101% | 102% | -0.1% | -0.1% | -0.2% | 101% | 100% |
| Simp. PDPC | -1.4% | -0.9% | -0.8% | 103% | 105% | -0.7% | -0.4% | -0.6% | 102% | 101% |
| All | -2.0% | -1.3% | -1.2% | 120% | 100% | -1.1% | -0.6% | -0.6% | 112% | 101% |

# Additional Informative Results

In this section additional results for combining the intra tools with more intra tools and proposed tools from transform and quantization are shown to illustrate the effect on the coding efficiency and runtimes.

## Combination with JEM intra tools

In addition to the combination described above, two JEM intra tools have been tested on top of that, namely:

- CCLM: Cross-component (CC) linear model (LM) chroma which has been studied in CE3 test 4.1.9 LM (PureLM+ Cb-to-Cr prediction) to be enabled in provided SW package as well as in BS with parameter --LMChroma=1

- 4TAP: 4-tap intra interpolation filters which has been studied in CE3 test 2.3.3 to be enabled in provided SW package as well as in BS with parameter --Intra4Tap=1

The individual results are taken from CE3 documents JVET-K0190 for CCLM and  JVET-K0097 for 4TAP while the combined results (All) are generated with the SW attached with this proposal and the two above-mentioned config parameters. Detailed results can be found in JVET-K0348+CCLM+4TAP-VTM.xlsm.

Table 3 Average BD-rates and relative runtimes for VTM configuration relative to VTM anchor

| VTM | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y |  | U | V | enc time | dec time |
| K0348 | -3.2% | -1.8% | -1.7% | 188% | 107% | -1.6% |  | -0.3% | -0.4% | 116% | 100% |
| CCLM | -1.5% | -9.8% | -10.5% | 113% | 104% | -0.9% |  | -11.0% | -11.7% | 102% | 101% |
| 4TAP | -0.5% | -0.4% | -0.5% | 106% | 103% | -0.3% |  | -0.5% | -0.6% | 102% | 102% |
| All | -5.1% | -12.2% | -12.7% | 212% | 113% | -2.7% |  | -12.3% | -12.8% | 120% | 100% |

It can be seen that these gains are also additive.

## Combination with transforms

Having seen that the individual gains of the 5 intra tools add up, it is interesting to the know what the impact on combining the intra tools from 2.1 with the following transform techniques would be (one from BMS and one studied in CE6):

- AMT: Adaptive multiple transform from JEM to be enabled in provided SW package as well as in BS with parameters --EMT=3 --EMTFast=3

- SoT: Set of transforms which combines the transforms from AMT with secondary transforms for 4x4 blocks similar to the NSST in BMS and which has been studied in CE6 test 3.2.1. Software was provided for CE6 in and SoT can be enabled using the parameter --SOT=1:

https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/branches/CE/J_SanDiego/CE6/CE6-3.2

Average VTM results for AI and RA are shown in Table 4. The results for AMT are generated with the BMS software in VTM configuration and parameters --EMT=3 --EMTFast=3 while the combined results (All+AMT) are generated with the SW attached with this proposal and the two config parameters. Detailed results can be found in JVET-K0348+CCLM+4TAP+AMT-VTM.xlsm. SoT results are taken from the CE6 document JVET-K0305 while the combined results (All+SoT) are preliminary and generated with a combination of the SW attached with this proposal and the CE6 SW.

It can be seen that the gains of the combined intra tools and the AMT are also additive. The preliminary results for the combined intra tools and the SoT also indicate additive gain.

Table 4 Average BD-rates and relative runtimes for VTM configuration relative to VTM anchor

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| All K0348+CCLM+4TAP | -5.1% | -12.2% | -12.7% | 212% | 113% | -2.7% | -12.3% | -12.8% | 120% | 100% |
| AMT | -3.5% | -1.6% | -1.5% | 211% | 119% | -2.2% | 0.0% | -0.1% | 167% | 108% |
| All + AMT | -8.9% | -14.4% | -15.1% | 441% | 135% | -5.1% | -13.0% | -13.6% | 223% | 110% |
| SoT | -4.6% | -3.4% | -3.4% | 239% | 121% | -2.1% | -1.9% | -2.0% | 125% | 105% |
| All + SoT* | -11.4% | -14.6% | -14.9% | 389% | 151% |  |  |  |  |  |

*Preliminary 49fr results for All + SoT test

The results of the combinations with AMT and SoT in VTM configuration compared to the BMS anchor are shown in Table 5 with detailed results provided in JVET-K0348+CCLM+4TAP+AMT-BMS.xlsm. For the combination of the intra tools with AMT, it can be seen that the BR-rate loss of the tested VTM combination compared to the BMS are reduced to 2.8% with only 46% of the encoding runtime and 69% of the decoding runtime. Preliminary results for SoT indicate less loss with more reduced encoder and decoder runtimes.

Table 5 Average BD-rates and relative runtimes for VTM configuration relative to BMS anchor

| BMS | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time |
| All + AMT | 2.8% | 6.1% | 7.0% | 46% | 69% |
| All + SoT* | 1.7% | 4.6% | 5.5% | 41% | 68% |

*Preliminary 49fr results for All + SoT test

## Combination with quantization and transform coefficient coding

Another aspect to study is the combination of the intra tools with quantization and transform coding, e.g. as studied in CE7. In the following exemplary combination, the CE7-related implementation of dependent quantization and modified transform coefficient coding:

- DepQ: Dependent quantization and modified transform coefficient coding as proposed in CE7-related document JVET-K0072.

Average VTM results for AI and RA are shown in Table 6. The results for DepQ are taken from the CE7-related document JVET-K0072 while the combined results (All+DepQ) are generated with a combination of the SW attached to this proposal and the CE7-related JVET-K0072 SW. Detailed results are provided in JVET-K0348+CCLM+4TAP+DEPQ-VTM.xlsm. It can be seen that the gains of the combined intra tools and DepQ are not 100% additive but 90% of the DepQ gain is still preserved.

Table 6 Average BD-rates and relative runtimes for VTM configuration relative to VTM anchor

| VTM | All Intra |  |  |  |  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time | Y | U | V | enc time | dec time |
| All K0348 + CCLM + 4TAP | -5.1% | -12.2% | -12.7% | 212% | 113% | -2.7% | -12.3% | -12.8% | 120% | 100% |
| DepQ | -4.8% | -1.6% | -1.6% | 119% | 100% | -3.2% | -1.5% | -1.5% | 113% | 99% |
| All + DepQ | -9.4% | -14.4% | -13.6% | 257% | 112% | -5.8% | -14.4% | -13.3% | 136% | 101% |

The results of the combinations with DepQ in VTM configuration compared to the BMS anchor are shown in Table 7. Detailed results are provided in JVET-K0348+CCLM+4TAP+DEPQ-BMS.xlsm. For the combination of the intra tools with DepQ, it can be seen that the BR-rate loss of the tested VTM combination compared to the BMS are reduced to 2.2% with only 27% of the encoding runtime and 57% of the decoding runtime.

Table 7 Average BD-rates and relative runtimes for VTM configuration relative to BMS anchor

| BMS | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test | Y | U | V | enc time | dec time |
| All + DepQ | 2.2% | 6.2% | 9.0% | 27% | 57% |

# Software and Settings

The source code is bundled with this proposal as: JVET-K0348_source_code_BMS1.0.zip

The combined test can be carried out by setting the following macros in TypeDef.h:

simp. PDPC:

#define PDPC_SIMP                      1

BiFi:

#define CE3_TEST271_IRS_BIFI           1 // bilateral reference sample filter

MRL:

#define JVET_J0014_MRL                 1 // multiple reference lines (MRL)

#define MRL_FAST_DISABLE_NARROW_BLOCKS 1 // MRL fast mode: exclude blocks with one side smaller

than half of the other side

#define MRL_FAST_ENCODER_ONLY          0 // MRL fast mode: apply restrictions only to search not

to signaling

#define MRL_NO_DC_PLANAR               1 // MRL do not test DC and PLANAR for mrlIdx > 0

#define MRL_MRLIDX_DEPENDENT_MPM       1 // MRL condition intra luma pred mode derivation on mrl_idx

For Intra65 the corresponding configuration parameter has to be set

in the command line by: --Intra65Ang=1

or in the config file by: Intra65Ang : 1

For PDPC the corresponding configuration parameter has to be set

in the command line by: --IntraPDPC=2

or in the config file by: IntraPDPC : 2

# Results

According to the common test conditions in JVET-J1010 [6], simulations were performed for all intra and random-access configurations for the combination of tools CE3-2.4.1 (simplified PDPC), CE3-5.4.4 (multiple reference lines for intra prediction), CE3-2.7.1 (bilateral reference sample filter) and 65 intra directional modes. The main parameters of the simulation environment are shown in Table 8.

Table 8 Simulation environment

| CPU | Turbo Boost | SIMD settings | Compiler | OS |
| --- | --- | --- | --- | --- |
| E5-2697A v4 | off | AVX2 | GCC 7.2.1 | Linux |

The results reported in the following sections have been cross-checked by Sharp as reported in JVET-K0504 [7].

## Over BMS-VTM-1.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.96% | -1.87% | -1.43% | 194% | 107% |
| Class A2 | -2.72% | -2.15% | -1.63% | 188% | 104% |
| Class B | -3.21% | -1.58% | -1.84% | 190% | 111% |
| Class C | -2.89% | -0.60% | -0.82% | 183% | 102% |
| Class E | -4.06% | -3.13% | -3.18% | 187% | 108% |
| Overall | -3.16% | -1.77% | -1.73% | 188% | 107% |
| Class D | -1.58% | 0.15% | 0.34% | 183% | 109% |
| Class F (optional) | -0.70% | 1.10% | 1.77% | 182% | 108% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -1.34% | -1.12% | 120% | 97% |
| Class A2 | -1.59% | -0.66% | -0.13% | 113% | 96% |
| Class B | -1.65% | 0.00% | -0.57% | 115% | 101% |
| Class C | -1.38% | 0.49% | 0.35% | 115% | 103% |
| Class E |  |  |  |  |  |
| Overall | -1.57% | -0.27% | -0.35% | 116% | 100% |
| Class D | -0.76% | 0.66% | 0.97% | 114% | 100% |
| Class F (optional) | -0.76% | 0.66% | 0.97% | 114% | 100% |

## Over BMS-1.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.12% | -2.52% | -2.04% | 120% | 100% |
| Class A2 | -1.97% | -1.45% | -1.18% | 121% | 100% |
| Class B | -2.02% | -1.01% | -0.93% | 121% | 98% |
| Class C | -1.78% | -0.80% | -0.73% | 117% | 100% |
| Class E | -2.36% | -1.11% | -1.27% | 120% | 103% |
| Overall | -2.03% | -1.31% | -1.17% | 120% | 100% |
| Class D | -1.07% | 0.04% | 0.24% | 117% | 100% |
| Class F (optional) | 0.06% | 0.19% | 0.26% | 119% | 103% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.23% | -1.65% | -1.41% | 114% | 103% |
| Class A2 | -1.03% | -0.64% | -0.45% | 111% | 102% |
| Class B | -1.15% | -0.14% | -0.41% | 113% | 99% |
| Class C | -0.87% | -0.33% | -0.45% | 111% | 101% |
| Class E |  |  |  |  |  |
| Overall | -1.07% | -0.59% | -0.63% | 112% | 101% |
| Class D | -0.57% | 0.06% | 0.38% | 112% | 100% |
| Class F (optional) | 0.03% | 0.27% | 0.44% | 113% | 100% |

# Conclusion

This proposal attempts to combine a subset of major tools that addresses several categories of intra mode coding, and it is demonstrated that each of these tools brings additive gains; it is therefore presented as good combination to be included in the VVC codec. Additional informative results indicate that the interaction with studied tools from other categories also results in additive gains.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Multiple reference line intra prediction (Test 5.4.1, 5.4.2, 5.4.3 and 5.4.4),” JVET-K0051, Ljubljana, SI, July 2018.

- P. Merkle, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Bilateral reference sample filter (Test 2.7.1),” JVET-K0043, Ljubljana, SI, July 2018.

- G. Van der Auwera, V. Seregin, A. Said, A.K. Ramasubramonian, M. Karczewicz, “CE3: Simplified PDPC (Test 2.4.1),” JVET-K0063, Ljubljana, SI, July 2018.

- J. Chen, E. Alshina, G. J. Sullivan, J.-R. Ohm, J. Boyce, “Algorithm description of Joint Exploration Test Model 7,” 7th JVET Meeting, Torino, Italy, July 2017, JVET-G1001

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.

- W. Zhu, A. Segall, “Cross-check of JVET-K0348,” JVET-K0504, Ljubljana, SI, July 2018.

# Patent rights declarations

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).