| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0349-v1 |
| --- | --- |

| Title: | CE4-2.11: MVPlanar prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shunsuke Iwamura Shimpei Nemoto Atsuro Ichigaya | Tel: Email: | +81-3-5494-3353 iwamura.s-gc@nhk.or.jp |
| Source: | NHK |  |  |

_____________________________

- Abstract

This contribution reports CE4-2.11 test results for interpolated motion compensation prediction also noted as MVPlanar prediction. MVPlanar derives sub-CU motion vectors of the current block using motion vectors of surrounding blocks. The proposed algorithm is implemented on top of VTM-1.0/BMS-1.0 and compression performance was evaluated. Compared to VTM-1.0, the proposed algorithm provides 0.32% and 0.09% BD bitrate savings in RA and LD conditions with encoder time increase of 7% and 11%, and decoder time increase of 10% and 8%, respectively. Compared to BMS-1.0, BD bitrate savings in RA and LB conditions with BMS configuration are 0.33% and 0.22% with encoding time increase of 3% and 6%, and decoding time increase of 4% and 3%, respectively.

Proposed method

MVPlanar prediction introduces sub-block motion vector compensation prediction, where the sub-CU MVs are derived by interpolating motion vectors of neighboring blocks by position dependent weighted average. The interpolation is carried out in a similar way as intra planar prediction except that x and y values of each sub-CU MVs are calculated separately. In the case that neighboring CU is intra coded, the closest neighboring MV is substituted in a similar way as intra reference sample substitution.

Figure 1. Derivation of sub-CU MVs of MVPlanar prediction

Let be the reference motion vectors be refMV[ x ][ y ] with x=-1, y=-1..nCbH/nSb and x=0..nCbW/nSb, y=-1, where (nCbW x nCbH) denote the width and the height of the luma coding block and nSb denotes the constant value of the granularity of the sub-CU MV, respectively.

The value of the interpolated motion vectors interpMV[ x ][ y ] are derived as follows:

interpMV[ x ][ y ] = ( ( ( nCbW/nSb -1 – x ) * refMV[ -1 ][ y ]

+ ( x + 1 ) * refMV[ nCbW/nSb ][ -1 ] ) << Log( nCbW/nSb ) )

+ ( ( nCbH/nSb – 1 – y ) * refMV[ x ][ -1 ]

+ ( y + 1 ) * refMV[ -1 ][ nCbH/nSb ] ) << Log( nCbH/nSb ) ) )

>> ( Log( nCbW/nSb ) + Log( nCbH/nSb ) + nCbW * nCbH )

In the coding unit syntax, the proposed methods introduce an additional syntax “mv_planar_flag” which specifies whether the current coding unit is coded using MVPlanar mode or not. If the mv_palanr_flag is equal to 1, inter prediction index “inter_pred_idc” syntax and reference index “ref_idx” syntaxes for both L0 and L1 lists are additionally signalled.

Simulation results

The proposed method has been implemented on top of BMS-1.0 and compression performance under the common test conditions is evaluated. The experimental results of MVPlanar prediction over VTM-1.0/BMS-1.0 in random access and low delay configurations are shown in Table 1 and 2, respectively.

Table 1. BD bitrate results of MVPlanar in RA conditions over VTM-1.0/BMS-1.0

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.40% | -0.31% | -0.36% | 105% | 110% | -0.24% | -0.15% | -0.13% | 102% | 105% |
| Class A2 | -0.58% | -0.36% | -0.44% | 107% | 113% | -0.49% | -0.21% | -0.30% | 103% | 106% |
| Class B | -0.29% | -0.21% | -0.14% | 108% | 110% | -0.38% | -0.25% | -0.22% | 103% | 103% |
| Class C | -0.09% | -0.19% | -0.08% | 107% | 107% | -0.20% | -0.12% | -0.14% | 103% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.32% | -0.26% | -0.23% | 107% | 110% | -0.33% | -0.19% | -0.20% | 103% | 104% |
| Class D | -0.09% | -0.05% | -0.10% | 105% | 105% | -0.10% | 0.13% | -0.14% | 104% | 100% |

Table 2. BD bitrate results of MVPlanar in LB conditions over VTM-1.0/BMS-1.0

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.08% | 0.07% | 0.14% | 110% | 108% | -0.14% | -0.18% | 0.24% | 106% | 108% |
| Class C | 0.04% | 0.00% | -0.06% | 109% | 108% | -0.13% | 0.18% | 0.19% | 105% | 98% |
| Class E | -0.29% | -0.11% | -0.23% | 115% | 108% | -0.45% | -0.34% | -0.46% | 107% | 102% |
| Overall | -0.09% | 0.00% | -0.02% | 111% | 108% | -0.22% | -0.10% | 0.05% | 106% | 103% |
| Class D | 0.01% | 0.11% | 0.00% | 110% | 103% | -0.17% | 0.25% | 0.07% | 106% | 107% |

Conclusion

This contribution reports interpolated motion compensation prediction noted as MVPlanar. Simulation results show 0.32% and 0.09% BD bitrate savings in RA and LD conditions with encoder time increase of 7% and 11%, and decoder time increase of 10% and 8% over VTM-1.0, and 0.33% and 0.22% BD bitrate savings in RA and LD configurations with encoding time increase of 3% and 6%, and decoding time increase of 4% and 3% over BMS-1.0, respectively.

Patent rights declaration(s)

NHK may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).