| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0350 |
| --- | --- |

| Title: | CE4-related: Improvement on Merge/Skip mode with line buffer restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu Han, Yung-Hsuan Chao, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes modification on Merge/Skip mode on top of the CE4 test 4.2.3 which adds additional non-adjacent spatial merge candidates and unified affine/normal merge candidate list. In order to reduce the line buffer size, the non-adjacent spatial merge candidates are restricted to be within the current CTU and its line buffer. For the “Tool on case 1 (VTM as reference)” tests in RA case, the test 1 “Non-adjacent spatial merge candidate candidates” achieved objective gain of 0.xx% in terms of average luma BD-rate improvement. For the “Tool off case 1 (BMS as reference)” test in RA case, the test 2 “Test 1 + Unified merge candidate list” achieved 1.xx% coding gain.

# Introduction

For the skip mode and the merge mode, a motion vector competition scheme is introduced to select a motion candidate from a given candidate set that includes spatial and temporal motion candidates. In the case of a skip mode, the residual signal is omitted. Only merge indices are transmitted for the block coded in merge or skip mode, and the current block inherits the inter prediction indicator, reference indices, and motion vectors from a neighboring block referred by the coded merge index.

# Technical description

## Merge/Skip mode

A merge candidate list is first constructed by inserting the motion vectors and reference indices of the spatial neighboring and temporal neighboring blocks into the candidate list with redundancy checking until the number of the available candidates reaches the maximum candidate signaled in slice header. The merge candidate list for the merge/skip mode is constructed by inserting spatial candidates (Figure 1), temporal candidates, affine candidates, advanced temporal MVP (ATMVP) candidate, spatial temporal MVP (STMVP) candidate (STMVP is disabled in this CE test) and the additional candidates as used in HEVC (Combined candidates and Zero candidates) according to a pre-defined insertion order. In order to reduce the line buffer size, the additional non-adjacent spatial merge candidates are restricted to be within the current CTU and its line buffer.

- Spatial candidates for blocks 1-4.

- Extrapolated affine candidates for blocks 1-4.

- ATMVP.

- STMVP (disabled in this CE test).

- Virtual affine candidate.

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6).

- Extrapolated affine candidate (block 5).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate followed by temporal candidate (blocks 6 to 49) within the current CTU and its line buffer.

offsetYUpperCTURow=((posLT.y≫log2MaxCodingBlockSize)<<                                                    log2MaxCodingBlockSize)-minPUSize-posLT.y

offsetXLeftCTUColumn=((posLT.x>>log2MaxCodingBlockSize)<<                                                       log2MaxCodingBlockSize)-minPUSize-posLT.x

offsetY = offsetY<offsetUpperCTURow?offsetUpperCTURow:offsetY

offsetY = offsetY<offsetUpperCTURow?offsetUpperCTURow:offsetY

- Combined candidates.

- Zero candidates.

Figure 1: Spatial neighboring blocks used to derive the spatial merge candidates.

In this proposal, the sub-block merge candidates ATMVP and STMVP are identical to those used in BMS. The derivation of extrapolated affine candidates and the virtual affine candidate is described in the following sections.

The selected merge candidate is signalled to the decoder.

## Unified affine/normal merge candidate list

Unlike in JEM there is no separate merge mode for affine. A merge candidate in the merge candidate list can be an affine merge candidate or a normal merge candidate. For each adjacent or non-adjacent spatial neighboring block, an affine merge candidate is derived from the affine model as in JEM. The candidate construction process starts with generating four non-affine spatial merge candidates (left, top, top-right, left-bottom), and followed by up to four affine merge candidates if the corresponding spatial neighboring blocks are coded in affine mode. For top-left spatial neighboring block and each non-adjacent spatial neighboring block an affine merge candidate from a spatial neighboring block is inserted into the merge candidate list after the normal merge candidate from that neighboring block has been inserted.

In addition, affine merge candidate constructed as combination of motion vectors from spatial neighboring blocks is inserted after the ATMVP/STMVP candidate. This candidate is constructed in the same way as the candidate for the affine motion vector prediction described in the previous section, except that the target reference picture is not known beforehand. The target reference picture is determined as follows. If there is a reference picture that is simultaneously referred to by motion vectors in S0,  S1 and S2, then such a reference picture with the smallest reference index is selected as the target one;  otherwise, if there is a reference picture that is simultaneously referred to by motion vectors in S0 and in S1, then such a reference picture with the smallest reference index is selected as the target one; otherwise, if there is a reference picture that is simultaneously referred to by a MVX in S0 and a MVZ in S2, then such a reference picture with the smallest reference index is selected as the target one; otherwise, the artificial candidate is unavailable.

# Simulation results

As descripted in CE4 [2], the “Tool on case 1 (VTM as reference)” and “Tool off case 1 (BMS as reference)” with default encoder configuration are used as anchors. The max number of merge candidates is 11 for BMS test and 10 for VTM test.

## Test 1: Non-adjacent spatial merge candidate candidates

Merge candidates list is as below: max number of merge candidates is 11 for BMS test and 10 for VTM test. (new candidates are highlight with underline)

- Spatial candidates for blocks 1-4.

- ATMVP (BMS test).

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6 (or 4 for VTM test)).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate followed by temporal candidate (blocks 6 to 49) within the current CTU and its line buffer.

- Combined candidates.

- Zero candidates.

The simulation results of the proposed technique which restricted the non-abjection spatial merge candidates within the current CTU and its line buffer as below:

The simulation results of CE test4.2.3.b which doesn’t restrict the non-abjection spatial merge candidates within the current CTU and its line buffer as below:

## Test 2: Test 1 + Unified merge candidate list

Merge candidates list is as below: max number of merge candidates is 11 for BMS test. (new candidates are highlight with underline)

- Spatial candidates for blocks 1-4.

- Extrapolated affine candidates for blocks 1-4.

- ATMVP (BMS test).

- Virtual affine candidate.

- Spatial candidate (block 5) (used only when the number of the available candidates is smaller than 6 (or 4 for VTM test)).

- Extrapolated affine candidate (block 5).

- Temporal candidate (derived as in HEVC).

- Non-adjacent spatial candidate followed by temporal candidate (blocks 6 to 49) within the current CTU and its line buffer and extrapolated affine candidate (blocks 6 to 49).

- Combined candidates.

- Zero candidates.

The simulation results of the proposed technique which restricted the non-abjection spatial merge candidates within the current CTU and its line buffer as below:

The simulation results which doesn’t restrict the non-abjection spatial merge candidates within the current CTU and its line buffer as below:

# Conclusion

This contribution presents improvement on Merge/Skip prediction technologies which is on top of CE test4.2.3.b. The proposed technique adds additional non-abjection spatial merge candidates which is within the current CTU and its line buffer and enlarges merge candidate list size to 10. For the “Tool on case 1 (VTM as reference)” tests in RA case, the test 1 “Non-adjacent spatial merge candidate candidates” achieved objective gain of 0.xx% in terms of average luma BD-rate improvement. For the “Tool off case 1 (BMS as reference)” test in RA case, the test 2 “Test 1 + Unified merge candidate list” achieved 1.xx% coding gain.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, Macau, Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

- J. Chen, E. Alshina, G. Sullivan, J. Ohm, J. Boyce, JVET-G1001, “Algorithm description of Joint Exploration Test Model 7”, July 2017.

- B. Bross, JVET-J1001, “Versatile video coding”, Apr. 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, JVET-J1010, “JVET common test conditions and software reference configurations”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).