| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0351-v1 |
| --- | --- |

| Title: | CE1-3.0.7: removing prevention of redundant split partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei-Jung Chien, Vadim Seregin, Marta Karczewicz, Nikolay Shlyakhov 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | +1-858-845-1795 wchien@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This document reports the results of CE1 sub-tests 3.0.7 on removing normative prevention of redundant splits in partition signalling. Four different types of restrictions are enforced in VTM1.0 to avoid signaling of the same block partitioning, which can be obtained by using different tree structures. By removing all the restrictions, simulation results show 0.05% BD-rate loss with average 7% encoder runtime increases in random access configuration, while keeping only one restriction preventing split of center partition of the ternary tree, simulation results show 0.01% BD-rate loss and average 3% encoder runtime increases in random access configuration.

# Introduction

This document reports the results of CE1 sub-tests 3.0.7 on prevention of redundant split partitioning. Four different types of restrictions listed in the next section below are applied in VTM1.0 to avoid creating the same partitioning with two different signalling schemes, and the tests results are summarized in Section 3.

# Description of techniques

There are four types of restrictions on block partitioning normatively enforced in VTM1.0 partition signaling as follows.

- In inter slice, second partition of a binary tree cannot be vertically split to emulate qual-tree split if the parent block can be qual-tree split, as shown in Fig. 1.

Figure 1. restriction on vertical BT partition

- In inter slice, second partition of the binary tree cannot be horizontally split to emulate qual-tree split, as shown in Fig. 2.

Figure 2. restriction on horizontal BT partition

- In inter slice, the second partition of BT split cannot be split into the same ternary tree as first partition of the BT split as shown in Figure 3.

Figure 3. restriction on ternary tree

- For all slice types, the center partition of a ternary tree cannot be split in binary tree to avoid the same block structure, which can be obtained by applying the same type of binary splits in two consecutive depths, as shown in Figure 4.

Figure 4. restriction on center block of a ternary tree

# Simulation results

The software is implemented based on BMS-1.0 and tested following CTC [1]. Two anchors are used: BMS software configured in VTM configuration, and BMS configuration. The results are summarized in following tables, and more detailed results can be found in the accompanied Excel tables.

## All restrictions are removed at encoder and decoder

In this experiment, all restrictions A – D from Section 2 are removed.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.01% | -0.01% | 110% | 99% | 0.00% | 0.03% | 0.00% | 110% | 103% |
| Class A2 | 0.00% | -0.05% | -0.07% | 110% | 102% | 0.00% | -0.06% | -0.05% | 108% | 100% |
| Class B | 0.00% | -0.07% | -0.05% | 110% | 99% | 0.00% | 0.00% | -0.03% | 108% | 98% |
| Class C | -0.01% | -0.15% | -0.25% | 110% | 100% | -0.03% | -0.06% | 0.01% | 109% | 100% |
| Class E | -0.02% | -0.17% | -0.08% | 109% | 99% | -0.02% | 0.01% | -0.21% | 108% | 100% |
| Overall | -0.01% | -0.09% | -0.10% | 110% | 100% | -0.01% | -0.01% | -0.05% | 109% | 100% |
| Class D | -0.02% | -0.03% | -0.13% | 110% | 99% | -0.02% | 0.10% | -0.11% | 109% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.18% | 0.01% | 105% | 100% | 0.05% | 0.00% | 0.01% | 108% | 101% |
| Class A2 | 0.06% | -0.14% | -0.07% | 104% | 99% | 0.04% | -0.10% | -0.03% | 108% | 100% |
| Class B | 0.05% | -0.16% | -0.16% | 105% | 101% | 0.02% | -0.12% | -0.20% | 108% | 100% |
| Class C | 0.06% | -0.28% | -0.20% | 107% | 101% | 0.11% | 0.11% | -0.37% | 109% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.05% | -0.19% | -0.12% | 105% | 100% | 0.05% | -0.03% | -0.17% | 108% | 100% |
| Class D | 0.10% | -0.17% | -0.07% | 108% | 100% | 0.10% | -0.05% | -0.07% | 107% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.08% | 0.26% | 0.55% | 110% | 105% | 0.05% | -0.22% | 0.19% | 108% | 97% |
| Class C | 0.13% | 0.15% | 0.25% | 110% | 101% | 0.08% | 0.07% | -0.30% | 110% | 101% |
| Class E | 0.23% | 0.01% | -0.34% | 108% | 102.7% | 0.10% | -0.02% | 0.20% | 108% | 103% |
| Overall | 0.13% | 0.16% | 0.23% | 109% | 103% | 0.07% | -0.07% | 0.03% | 109% | 100% |
| Class D | 0.12% | 0.46% | 0.41% | 110% | 99% | 0.05% | -0.14% | -0.93% | 110% | 101% |

## All restrictions are always forced by encoder only

In this experiment, all restrictions, A – D, related to redundant partitions listed in Section 2 are removed at the decoder, but are applied at the encoder for fast encoder decisions.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.07% | -0.05% | 100% | 100% | 0.03% | -0.09% | -0.04% | 100% | 102% |
| Class A2 | 0.08% | 0.08% | 0.07% | 100% | 101% | 0.08% | -0.04% | 0.04% | 100% | 99% |
| Class B | 0.07% | 0.07% | 0.07% | 100% | 97% | 0.06% | 0.08% | 0.02% | 100% | 99% |
| Class C | 0.12% | 0.05% | -0.06% | 100% | 99% | 0.10% | 0.05% | 0.10% | 100% | 101% |
| Class E | 0.12% | -0.06% | 0.12% | 100% | 99% | 0.12% | 0.05% | -0.05% | 99% | 100% |
| Overall | 0.09% | 0.05% | 0.03% | 100% | 99% | 0.08% | 0.02% | 0.02% | 100% | 100% |
| Class D | 0.11% | 0.23% | 0.04% | 100% | 98% | 0.10% | 0.23% | 0.05% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.16% | 0.18% | 0.17% | 101% | 101% | 0.20% | 0.02% | 0.16% | 101% | 101% |
| Class A2 | 0.24% | 0.12% | 0.24% | 100% | 100% | 0.25% | 0.22% | 0.13% | 101% | 100% |
| Class B | 0.21% | 0.14% | -0.08% | 101% | 101% | 0.20% | 0.11% | 0.10% | 100% | 99% |
| Class C | 0.24% | 0.16% | -0.05% | 101% | 100% | 0.27% | 0.41% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.21% | 0.15% | 0.04% | 101% | 100% | 0.23% | 0.19% | 0.08% | 100% | 100% |
| Class D | 0.30% | 0.36% | -0.04% | 101% | 100% | 0.27% | 0.36% | 0.05% | 98% | 97% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.23% | 0.15% | 0.63% | 101% | 99% | 0.20% | -0.18% | 0.22% | 100% | 99% |
| Class C | 0.27% | 0.17% | 0.22% | 102% | 98% | 0.23% | 0.27% | 0.11% | 100% | 102% |
| Class E | 0.32% | 0.06% | -0.44% | 102% | 102.8% | 0.02% | 0.08% | -0.10% | 100% | 99% |
| Overall | 0.27% | 0.14% | 0.22% | 102% | 100% | 0.17% | 0.04% | 0.10% | 100% | 100% |
| Class D | 0.31% | 0.18% | -0.14% | 100% | 98% | 0.27% | 0.54% | 0.14% | 101% | 104% |

## Center partition of a ternary tree restriction

In this experiment, only the restriction D are applied in both encoder and decoder while the restrictions A, B and C are removed from both encoder and decoder.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | -0.08% | 103% | 101% | 0.00% | -0.12% | -0.07% | 103% | 101% |
| Class A2 | -0.02% | -0.12% | -0.03% | 102% | 99% | 0.02% | -0.07% | -0.03% | 103% | 101% |
| Class B | 0.02% | -0.04% | -0.04% | 102% | 101% | 0.00% | -0.05% | -0.08% | 103% | 100% |
| Class C | 0.02% | -0.05% | -0.07% | 103% | 101% | 0.03% | -0.08% | -0.11% | 103% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.09% | -0.05% | 103% | 100% | 0.01% | -0.08% | -0.07% | 103% | 100% |
| Class D | 0.02% | 0.17% | -0.08% | 103% | 101% | 0.08% | 0.03% | 0.13% | 101% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.02% | 0.01% | 0.02% | 104% | 103% | -0.01% | -0.10% | -0.07% | 103% | 99% |
| Class C | 0.08% | 0.03% | 0.19% | 104% | 99% | 0.05% | 0.13% | 0.09% | 103% | 98% |
| Class E | 0.13% | 0.01% | 0.21% | 103% | 101.3% | -0.10% | 0.35% | 0.47% | 103% | 104% |
| Overall | 0.07% | 0.02% | 0.13% | 104% | 101% | -0.01% | 0.09% | 0.12% | 103% | 100% |
| Class D | 0.04% | 0.22% | 0.13% | 104% | 99% | 0.08% | 0.27% | -0.81% | 103% | 101% |

## Center partition of a ternary tree restriction with other non-normative restrictions

In this experiment, only the restriction D are enforced in both encoder and decoder. The restrictions A, B and C are removed from the decoder but are applied at the encoder as non-normative encoder decision.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 98% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 99% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% | 0.00% | 0.00% | 0.00% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.05% | 0.07% | 99% | 99% | 0.09% | 0.00% | 0.09% | 99% | 101% |
| Class A2 | 0.05% | 0.02% | 0.06% | 99% | 99% | 0.08% | 0.04% | 0.01% | 100% | 101% |
| Class B | 0.08% | 0.07% | 0.09% | 101% | 102% | 0.06% | -0.06% | 0.02% | 100% | 100% |
| Class C | 0.06% | 0.01% | 0.05% | 101% | 101% | 0.11% | 0.01% | -0.09% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.07% | 0.04% | 0.07% | 100% | 101% | 0.08% | -0.01% | 0.01% | 100% | 100% |
| Class D | 0.09% | 0.19% | 0.06% | 101% | 102% | 0.16% | 0.15% | 0.18% | 99% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.07% | 0.21% | 0.03% | 101% | 103% | 0.06% | -0.22% | 0.35% | 99% | 97% |
| Class C | 0.15% | 0.15% | 0.11% | 102% | 101% | 0.05% | 0.09% | -0.30% | 100% | 99% |
| Class E | 0.08% | 0.11% | -0.12% | 102% | 103.2% | -0.22% | 0.42% | 0.24% | 100% | 101% |
| Overall | 0.10% | 0.17% | 0.02% | 101% | 102% | -0.01% | 0.05% | 0.10% | 100% | 99% |
| Class D | 0.09% | -0.06% | -0.23% | 101% | 99% | 0.10% | 0.33% | 0.02% | 100% | 101% |

# Conclusion

Simulation results show that only ternary tree only restriction leads to similar coding performance with negligible encoder runtime increases. We recommend keeping only ternary tree restriction and remove other restrictions from the VTM.

# Reference

[1] Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).