| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0352 |
| --- | --- |

| Title: | CE4-related: Encoder optimization on top of CE 4.2.13 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jing Ye, Xiang Li, and Shan Liu Tencent America LLC 661 Bryant St, Palo Alto, CA 94301 | Tel: Email: | +1 650-798-3300 jingye@tencent.com xlxiangli@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes an encoder optimization algorithm on top of CE 4.2.13. The proposed method reports 0.30% and 0.29% luma coding gain compare to CE 4.2.13 Test A in RA and LDB configuration on VTM with 1-2% encoding time increase.

# Algorithm description

In VTM-1 and BMS-1, when doing the merge RD checking (including motion compensation and residual coding), if the best cached mode is skip mode, all the merge candidates must be in skip. In this case, it is proposed to do full RD checking for all the candidates. The proposed code change in function xCheckRDCostMerge2Nx2N() is highlighted as follows. The related patch is also attached with the document.

if( !bestIsSkip )

{

…

}

else

{

uiNumMrgSATDCand = mergeCtx.numValidMergeCand;

}

#endif

The one-line change is quite simple. With default fast method in VTM-1 and BMS-1, SATD based merge candidate pruning is enabled so that interpolation for all merge candidates is performed. In this case, further do RD checking for all candidates when all candidates must be in skip would not impact encoding time much. Therefore, the proposed method does not increase encoding time much even more RD checking is used.

# Simulations

The proposed encoder optimization method is implemented on top of CE4.2.13 [1]. The VTM [2] results are generated by running BMS [3] software with VTM configuration. Common test conditions defined in [4] is used in the simulation. The proposed method can be also applied to VTM software. The results of VTM with this encoder only changes is also provided.

Table 1 shows VTM results of proposed method. The first anchor is VTM1.0, and the second anchor is CE4.2.13 test A. The maximum merge candidate number is the same to CE4.2.13 test A in the proposed method. Compared to the second anchor, 0.30% and 0.29% BD-rate saving is achieved with 1-2% encoding time increase.

- Simulation Results of proposed method 1 with VTM configuration

Table 2 show the results of applying this method to VTM.

Table 2. simulation results of VTM with encoder only change

# Conclusions

This proposal describes an encoder optimization method on top of of CE4.2.13. With this method, 0.30% and 0.29% luma coding gain on top of CE 4.2.13 Test A in RA and LDB configuration on VTM is achieved. It is proposed to include this encoder optimization into test model.

# Refences

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-J1024, San Diego, US, April 2018.

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_VTM/tags/VTM-1.0

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, US, April 2018.

# Patent rights declaration(s)

Tencent does not have any current or pending patent rights relating to the technology described in this contribution.