| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0353-v1 |
| --- | --- |

| Title: | CE1: Shared-separate partition tree in QT+BT configuration (Tests 5.3.1 and 5.4.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Weijia Zhu, Andrew Segall 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com zhuw@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This contribution reports the results of tests 5.3.1 and 5.4.1 of CE1. These tests evaluate the performance of enabling shared-separate partitioning trees (SST) for intra-coded regions in intra-slices and inter-slices, respectively, as described in JVET-J0026.  Furthermore, these tests disable ternary tree (TT) partitioning.  From the results of the tests, the proponents assert that for All-Intra configuration, shared-separate partitioning trees (SST) that do not use ternary trees (TT) reduce encoder runtime by 20% while providing coding performance similar to the anchor (which makes use of TT).

# Introduction

In separate shared partitioning (SST), a block of intra samples may be partitioned into separate luma and chroma partitioning trees.  An illustration of the partitioning method appears in Figure 1.1.  As can be seen in the Figure, a CTU partitioning can be selectively changed for different coding units.

Indicating the use of a separate or shared partition is accomplished here with a combination of inference and direct signaling.  Specifically, when a coding unit has a block size greater than or equal to a thresholdA, then no selection flag is received, and a separate luma and chroma partitioning must be used for the coding unit.  Alternatively, when the block size is less than or equal to a thresholdB, then the selection flag is also not received but a shared luma and chroma partitioning is employed. In our experiments thresholdA was set to 1024 and thresholdB was set to 512. These thresholds are signaled in the sequence parameter set.

Please note that when separate trees are signalled, the chroma intra mode prediction makes use of multiple direct modes as implemented in the BMS-1.0 software. Also, when coding separate trees in an intra-slice: (i) chroma may make use of 64x64 TU, and (ii) chroma QT depth is restricted to be smaller than corresponding luma QT depth.

# Experimental Results

In the simulations, ternary tree (TT) partitioning is disabled for both tests. The anchor is BMS-1.0 with VTM and BMS configuration and with TT enabled.  In Test 5.3.1, a coding tree unit (CTU) selects between shared or separate partitioning trees in both intra and inter slices.  Test 5.4.1 has the same configuration with the exception that separate trees are always selected in an inter-slice. Furthermore, in all tests, two additional limitations are enforced: (1) separate trees in an inter-slice can not use a quad tree partition, and (2) shared trees in an intra-CU can not split further.

Figure 1.1: An illustration of combined separate and shared tree partitioning for intra coded regions.

Results for the tests are provided below.  From the results, we observe that, for the all-intra configuration where only intra-slice types are used, SST yields coding efficiency similar to that of TT with a significant reduction in encoder runtime complexity. For other slice-types the number of samples coded with intra is substantially smaller and the disabling of TT leads to a loss.

## Test 5.3.1

| All Intra Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 0.95%; -4.53%; -4.25%; 71%; 104%. Class A2; 1.10%; -4.45%; -2.26%; 79%; 102%. Class B; 0.95%; -4.82%; -7.82%; 81%; 101%. Class C; 1.29%; -2.15%; -4.04%; 82%; 103%. Class E; 1.69%; -8.81%; -11.23%; 88%; 105%. Overall; 1.18%; -4.78%; -6.03%; 80%; 103%. Class D; 0.93%; -1.71%; -2.91%; 80%; 99%. Class F (optional); 0.97%; -1.24%; -0.74%; 86%; 100%. Random Access Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 2.55%; -0.38%; 0.91%; 47%; 112%. Class A2; 3.31%; 0.85%; 2.58%; 48%; 108%. Class B; 2.77%; -1.17%; -4.18%; 47%; 107%. Class C; 2.56%; 1.89%; 0.59%; 46%; 111%. Class E. Overall; 2.78%; 0.21%; -0.54%; 47%; 110%. Class D; 2.11%; 1.61%; 0.17%; 48%; 110%. Class F (optional); 2.20%; 1.65%; 1.58%; 55%; 106%. Low Delay B Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 2.67%; 3.15%; 2.33%; 42%; 94%. Class C; 2.62%; 3.66%; 3.63%; 39%; 95%. Class E; 3.85%; 0.04%; -1.79%; 56%; 90%. Overall; 2.95%; 2.54%; 1.74%; 44%; 93%. Class D; 2.26%; 2.50%; 2.06%; 41%; 95%. Class F (optional); 3.62%; 4.69%; 4.63%; 51%; 96%. Low Delay P Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 2.68%; 3.31%; 1.80%; 42%; 99%. Class C; 2.51%; 3.64%; 3.33%; 39%; 98%. Class E; 3.78%; -0.60%; -2.33%; 51%; 91%. Overall; 2.90%; 2.44%; 1.28%; 43%; 97%. Class D; 2.16%; 2.02%; 2.14%; 42%; 101%. Class F (optional); 3.41%; 4.60%; 4.88%; 49%; 103%. | All Intra Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -0.72%; -9.87%; -5.07%; 67%; 103%. 0.55%; -7.21%; -2.10%; 73%; 105%. 0.21%; -7.24%; -11.71%; 76%; 106%. 0.21%; -5.99%; -6.30%; 72%; 104%. 1.28%; -8.94%; -10.77%; 75%; 101%. 0.29%; -7.68%; -7.64%; 73%; 104%. 0.41%; -5.13%; -4.72%; 74%; 103%. -1.22%; -9.29%; -10.30%; 78%; 105%. Random Access Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.28%; -2.80%; -0.75%; 54%; 110%. 3.18%; -2.87%; 1.25%; 56%; 114%. 2.46%; -4.67%; -7.53%; 54%; 111%. 2.46%; -1.12%; -1.19%; 50%; 109%. 2.57%; -2.99%; -2.73%; 54%; 111%. 2.15%; -1.98%; -2.03%; 52%; 106%. 1.25%; -3.91%; -4.27%; 60%; 106%. Low Delay B Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.46%; 1.23%; 0.50%; 49%; 112%. 2.51%; 2.79%; 2.24%; 44%; 113%. 2.87%; -0.38%; -1.13%; 55%; 103%. 2.58%; 1.35%; 0.67%; 48%; 110%. 2.50%; 1.05%; 1.00%; 48%; 115%. 3.56%; 2.08%; 2.81%; 50%; 108%. Low Delay P Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.48%; 1.51%; 0.04%; 53%; 109%. 2.63%; 2.96%; 2.59%; 45%; 115%. 2.95%; -0.17%; -1.98%; 54%; 106%. 2.65%; 1.57%; 0.38%; 50%; 110%. 2.61%; 0.64%; 1.08%; 47%; 110%. 3.42%; 1.91%; 2.43%; 58%; 120%. |
| --- | --- |

## Test 5.4.1

| All Intra Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 1.70%; -4.57%; -4.14%; 33%; 100%. Class A2; 1.72%; -4.49%; -1.70%; 37%; 102%. Class B; 1.15%; -4.93%; -8.17%; 41%; 107%. Class C; 1.59%; -2.22%; -4.28%; 41%; 106%. Class E; 1.87%; -9.40%; -11.83%; 41%; 105%. Overall; 1.56%; -4.94%; -6.16%; 39%; 105%. Class D; 1.21%; -1.63%; -3.10%; 41%; 102%. Class F (optional); 1.27%; -1.11%; -0.77%; 43%; 102%. Random Access Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 2.65%; -0.49%; 0.97%; 49%; 114%. Class A2; 3.47%; 1.16%; 3.10%; 49%; 110%. Class B; 2.80%; -1.08%; -4.21%; 47%; 110%. Class C; 2.60%; 1.91%; 0.41%; 45%; 114%. Class E. Overall; 2.85%; 0.28%; -0.48%; 47%; 112%. Class D; 2.12%; 1.70%; 0.34%; 46%; 111%. Class F (optional); 2.29%; 1.95%; 1.86%; 51%; 108%. Low Delay B Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 2.66%; 3.38%; 2.60%; 42%; 91%. Class C; 2.66%; 3.58%; 3.44%; 39%; 96%. Class E; 3.81%; -0.60%; -2.17%; 56%; 92%. Overall; 2.95%; 2.45%; 1.69%; 44%; 93%. Class D; 2.26%; 2.89%; 2.17%; 41%; 94%. Class F (optional); 3.63%; 4.67%; 4.97%; 54%; 100%. Low Delay P Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 2.66%; 3.25%; 1.85%; 42%; 94%. Class C; 2.47%; 3.63%; 3.32%; 40%; 98%. Class E; 3.80%; -1.06%; -2.69%; 54%; 96%. Overall; 2.88%; 2.30%; 1.21%; 44%; 96%. Class D; 2.17%; 2.02%; 2.18%; 42%; 100%. Class F (optional); 3.45%; 4.43%; 4.79%; 50%; 104%. | All Intra Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -0.65%; -10.27%; -4.00%; 26%; 103%. 0.76%; -7.36%; -1.71%; 29%; 106%. 0.23%; -7.24%; -11.72%; 29%; 101%. 0.21%; -5.98%; -6.24%; 30%; 102%. 1.35%; -9.33%; -11.35%; 32%; 105%. 0.35%; -7.83%; -7.48%; 29%; 103%. 0.44%; -4.93%; -4.42%; 33%; 110%. -1.21%; -9.37%; -10.51%; 31%; 99%. Random Access Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.28%; -3.02%; -0.63%; 49%; 108%. 3.25%; -2.91%; 1.58%; 47%; 104%. 2.46%; -4.60%; -7.48%; 49%; 108%. 2.45%; -1.12%; -1.05%; 44%; 107%. 2.58%; -3.02%; -2.58%; 47%; 107%. 2.15%; -1.94%; -2.06%; 43%; 104%. 1.22%; -3.87%; -4.46%; 47%; 105%. Low Delay B Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.46%; 1.45%; 0.55%; 44%; 106%. 2.50%; 2.84%; 2.29%; 40%; 106%. 2.77%; -0.59%; -1.56%; 54%; 108%. 2.55%; 1.40%; 0.60%; 45%; 106%. 2.56%; 1.54%; 0.89%; 46%; 114%. 3.39%; 1.82%; 2.25%; 48%; 105%. Low Delay P Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 2.45%; 1.13%; -0.02%; 45%; 105%. 2.62%; 2.81%; 2.29%; 41%; 108%. 2.84%; -0.76%; -1.79%; 50%; 105%. 2.60%; 1.22%; 0.31%; 45%; 106%. 2.63%; 1.26%; 0.80%; 43%; 105%. 3.32%; 1.95%; 1.93%; 51%; 112%. |
| --- | --- |

# Conclusion

It is reported that for the all-intra configuration where only intra-slice types are used; SST yields coding efficiency similar to that of TT with a significant reduction in encoder runtime complexity. It is proposed that SST be adopted into the next version VTM/BMS software and Versatile Video Coding Working Draft.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).