| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0354-v1 |
| --- | --- |

| Title: | CE1: Shared-separate partition tree in QT+BT+TT configuration (Tests 5.3.2 and 5.4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiran Misra, Weijia Zhu, Andrew Segall 5750 NW Pacific Rim Blvd Camas WA 98607 USA | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com zhuw@sharplabs.com asegall@sharplabs.com |
| Source: | Sharp |  |  |

_____________________________

# Abstract

This contribution reports the results of tests 5.3.2 and 5.4.2 of CE1 based on JVET-J0026. These tests evaluate the performance of enabling shared-separate partitioning trees (SST) for intra-coded regions in intra-slices and inter-slices, respectively. For BMS configuration, Test 5.3.2 results show an average BD rate improvement of −0.94%(Y)/−9.75%(U)/−9.93%(V) for all intra and −0.18%(Y)/−7.58%(U)/−7.55%(V) for random access. As the method is only applied to intra-coded regions, it is noted that the random access performance is greater for sequences where inter-prediction fails.  For example, CampFire and Tango provide gains of (−0.44%(Y)/−4.18%(U)/−1.3%(V)) and (0.01%(Y)/−10.31%(U)/−7.61%(V)), respectively.

# Introduction

Figure 1.1: An illustration of combined separate and shared tree partitioning for intra coded regions.

In separate shared partitioning (SST), a block of intra samples may be partitioned into separate luma and chroma partitioning trees.  An illustration of the partitioning method appears in Figure 1.1.  As can be seen in the Figure, a CTU partitioning can be selectively changed for different coding units.

Indicating the use of a separate or shared partition is accomplished here with a combination of inference and direct signaling.  Specifically, when a coding unit has a block size greater than or equal to a thresholdA, then no selection flag is received, and a separate luma and chroma partitioning must be used for the coding unit.  Alternatively, when the block size is less than or equal to a thresholdB, then the selection flag is also not received but a shared luma and chroma partitioning is employed. In our experiments thresholdA was set to 1024 and thresholdB was set to 512. These thresholds are signaled in the sequence parameter set.

Please note that when separate trees are signalled, the chroma intra mode prediction makes use of multiple direct modes as implemented in the BMS-1.0 software. Also, when coding separate trees in an intra-slice: (i) chroma may make use of 64x64 TU, and (ii) chroma QT depth is restricted to be smaller than corresponding luma QT depth.

# Experimental Results

The anchor is BMS-1.0 with VTM and BMS configuration. In Test 5.3.2, a coding tree unit (CTU) selects between shared or separate partitioning trees in both intra and inter slices.  Test 5.4.2 has the same configuration with the exception that separate trees are always selected in an inter-slice. Furthermore, in all tests, two additional limitations are enforced: (1) separate trees in an inter-slice can not use a quad tree partition, and (2) shared trees in an intra-CU can not split further. Results for the tests are provided below.

## Test 5.3.2

| All Intra Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; -0.10%; -5.58%; -5.11%; 168%; 100%. Class A2; -0.36%; -6.52%; -4.35%; 192%; 106%. Class B; -0.33%; -6.97%; -10.12%; 192%; 104%. Class C; -0.38%; -4.98%; -7.21%; 190%; 104%. Class E; -0.32%; -11.45%; -13.59%; 190%; 106%. Overall; -0.31%; -6.97%; -8.26%; 187%; 104%. Class D; -0.30%; -4.13%; -5.53%; 190%; 101%. Class F (optional); -0.82%; -4.03%; -3.70%; 193%; 101%. Random Access Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 0.04%; -3.84%; -2.43%; 127%; 110%. Class A2; 0.02%; -4.39%; -2.47%; 118%; 107%. Class B; -0.07%; -6.05%; -8.98%; 115%; 106%. Class C; -0.18%; -2.96%; -4.58%; 113%; 109%. Class E. Overall; -0.06%; -4.45%; -5.19%; 117%; 108%. Class D; -0.13%; -2.73%; -4.12%; 112%; 110%. Class F (optional); -0.55%; -2.46%; -2.49%; 122%; 105%. Low Delay B Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.03%; -0.72%; -1.91%; 112%; 92%. Class C; 0.01%; -0.45%; -0.64%; 107%; 95%. Class E; -0.28%; -6.27%; -7.26%; 119%; 91%. Overall; -0.05%; -2.02%; -2.83%; 112%; 93%. Class D; -0.06%; -0.85%; -1.84%; 105%; 96%. Class F (optional); -0.26%; -0.29%; -0.39%; 120%; 93%. Low Delay P Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.05%; -0.84%; -2.27%; 111%; 93%. Class C; -0.02%; -0.46%; -0.77%; 104%; 94%. Class E; -0.38%; -7.01%; -7.77%; 110%; 92%. Overall; -0.08%; -2.26%; -3.15%; 108%; 93%. Class D; 0.00%; -1.52%; -1.56%; 103%; 98%. Class F (optional); -0.32%; -0.32%; -0.63%; 121%; 100%. | All Intra Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -1.47%; -10.89%; -6.28%; 162%; 103%. -0.63%; -9.33%; -4.45%; 169%; 105%. -0.88%; -9.41%; -14.13%; 168%; 102%. -1.27%; -8.68%; -9.29%; 171%; 105%. -0.37%; -11.04%; -12.90%; 168%; 102%. -0.94%; -9.75%; -9.93%; 168%; 103%. -0.67%; -7.60%; -7.47%; 185%; 108%. -2.87%; -11.97%; -13.06%; 164%; 99%. Random Access Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -0.14%; -6.28%; -5.03%; 154%; 105%. -0.05%; -7.93%; -3.54%; 141%; 104%. -0.20%; -9.46%; -12.38%; 142%; 104%. -0.29%; -5.94%; -6.40%; 136%; 105%. -0.18%; -7.58%; -7.55%; 142%; 105%. -0.12%; -6.38%; -6.15%; 129%; 103%. -1.38%; -7.96%; -8.18%; 144%; 101%. Low Delay B Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 0.01%; -2.77%; -3.43%; 118%; 100%. -0.03%; -1.38%; -1.77%; 106%; 106%. -0.43%; -4.88%; -5.59%; 100%; 100%. -0.11%; -2.83%; -3.42%; 109%; 102%. 0.07%; -2.50%; -3.23%; 103%; 107%. -0.13%; -2.78%; -2.46%; 115%; 99%. Low Delay P Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 0.05%; -2.39%; -4.22%; 121%; 99%. 0.06%; -1.09%; -1.74%; 117%; 109%. -0.22%; -4.48%; -6.27%; 103%; 104%. -0.02%; -2.48%; -3.91%; 115%; 104%. 0.05%; -2.82%; -2.93%; 107%; 105%. -0.17%; -2.90%; -2.50%; 132%; 106%. |
| --- | --- |

## Test 5.4.2

| All Intra Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 0.65%; -5.95%; -5.08%; 81%; 106%. Class A2; 0.25%; -6.67%; -4.04%; 88%; 109%. Class B; -0.15%; -7.16%; -10.53%; 86%; 102%. Class C; -0.16%; -5.11%; -7.39%; 98%; 111%. Class E; -0.14%; -11.87%; -14.33%; 86%; 105%. Overall; 0.05%; -7.21%; -8.48%; 88%; 106%. Class D; -0.11%; -4.18%; -5.86%; 91%; 100%. Class F (optional); -0.62%; -3.95%; -3.61%; 87%; 99%. Random Access Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1; 0.15%; -3.96%; -2.45%; 129%; 113%. Class A2; 0.15%; -4.22%; -2.15%; 117%; 109%. Class B; -0.06%; -5.93%; -8.96%; 118%; 112%. Class C; -0.12%; -2.99%; -4.56%; 111%; 112%. Class E. Overall; 0.01%; -4.41%; -5.12%; 118%; 112%. Class D; -0.10%; -2.60%; -4.17%; 108%; 113%. Class F (optional); -0.52%; -2.28%; -2.26%; 112%; 109%. Low Delay B Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.00%; -0.95%; -1.61%; 110%; 91%. Class C; -0.02%; -0.38%; -0.70%; 106%; 96%. Class E; -0.18%; -6.47%; -7.86%; 124%; 97%. Overall; -0.05%; -2.14%; -2.87%; 112%; 94%. Class D; -0.01%; -0.96%; -1.38%; 114%; 103%. Class F (optional); -0.31%; -0.22%; -0.50%; 125%; 100%. Low Delay P Main 10. Over BMS-1.0 VTM Cfg. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; 0.03%; -0.99%; -2.35%; 109%; 92%. Class C; -0.06%; -0.35%; -0.95%; 113%; 102%. Class E; -0.25%; -6.77%; -8.14%; 121%; 102%. Overall; -0.07%; -2.22%; -3.33%; 113%; 98%. Class D; -0.02%; -1.26%; -1.88%; 105%; 101%. Class F (optional); -0.27%; -0.55%; -0.06%; 128%; 107%. | All Intra Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -1.41%; -11.44%; -5.16%; 56%; 103%. -0.43%; -9.68%; -4.37%; 63%; 107%. -0.88%; -9.44%; -14.14%; 66%; 106%. -1.25%; -8.67%; -9.27%; 68%; 104%. -0.30%; -11.33%; -13.38%; 67%; 106%. -0.88%; -9.95%; -9.81%; 64%; 105%. -0.66%; -7.56%; -7.27%; 78%; 111%. -2.89%; -12.05%; -13.34%; 71%; 103%. Random Access Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. -0.13%; -6.51%; -4.73%; 147%; 106%. -0.05%; -8.23%; -3.53%; 130%; 106%. -0.21%; -9.46%; -12.51%; 132%; 108%. -0.28%; -5.96%; -6.25%; 125%; 111%. -0.18%; -7.69%; -7.49%; 133%; 108%. -0.10%; -6.44%; -6.07%; 108%; 104%. -1.37%; -7.98%; -8.30%; 115%; 104%. Low Delay B Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 0.03%; -2.81%; -3.61%; 117%; 105%. -0.02%; -1.39%; -1.56%; 109%; 110%. -0.41%; -6.33%; -7.94%; 100%; 104%. -0.10%; -3.22%; -4.01%; 110%; 106%. 0.04%; -2.57%; -2.69%; 107%; 112%. -0.24%; -3.44%; -2.69%; 117%; 104%. Low Delay P Main 10. Over BMS-1.0. Y; U; V; EncT; DecT. 0.04%; -2.56%; -4.07%; 122%; 106%. 0.03%; -1.43%; -1.98%; 120%; 115%. -0.39%; -6.07%; -7.78%; 113%; 118%. -0.07%; -3.06%; -4.30%; 119%; 112%. 0.06%; -2.19%; -2.32%; 105%; 105%. -0.19%; -3.33%; -2.75%; 135%; 114%. |
| --- | --- |

# Conclusion

It is reported that for the BMS configuration SST yields an average BD rate improvement of −0.94%(Y)/−9.75%(U)/−9.93%(V) for all intra and −0.18%(Y)/−7.58%(U)/−7.55%(V) for random access. It is asserted that SST is especially beneficial for intra-slices and for sequences where inter-prediction fails for example CampFire (−0.44%(Y)/−4.18%(U)/−1.3%(V)) which has a fire burning on part of the picture, and Tango (0.01%(Y)/−10.31%(U)/−7.61%(V)) which has people rotating in and out of view. It is recommended that SST be adopted into the next version VTM/BMS software and Versatile Video Coding Working Draft.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).