| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SL, 10–20 July 2018 | Document: JVET-K0356 |
| --- | --- |

| Title: | Cross check of proposals for CE6 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Cross Check |  |  |
| Author(s) or Contact(s): | Yuriy Reznik | Tel: Email: | +1-206-265-0309 yreznik@brightcove.com |
| Source: | Yuriy Reznik |  |  |

_____________________________

# Abstract

This contribution is a review of design of transforms in VTM and BMS software, and cross-checks for the following CE6 (transform design) proposals:

- CE6-1.1.1 JVET-K0375

- CE6-1.2.1 JVET-K0272

- CE6-1.3.1 JVET-K0264

- CE6-1.5.1 JVET-K0171 [not complete yet]

- CE6-1.8.1 JVET-K0123 [not complete yet]

- CE6-1.10.1 JVET-K0250 [not complete yet]

- CE6-1.11.1 JVET-K0096

- CE6-1.13.1 JVET-K025 [not complete yet]

- CE6-2.2.1 JVET-K0734 [not complete yet]

- CE6-2.5.1 JVET-K0098 [not complete yet]

- CE6-3.1.1 JVET-K0085 [not complete yet]

- CE6-3.2.1 JVET-K0305 [not complete yet]

# Transforms in VTM and BMS software

## Design review

The VTM as well as BMS software includes two modules related to transforms:

- TrQuant_EMT.h/cpp – defining implementations of transforms of all types and sizes, and

- TrQuant.h/cpp – defining a TrQuant class, providing a high-level API for fused quantization + transform operations.

In VMT, the TrQuant_EMT module includes DCT-2 of dyadic sizes in range 2-128:

void fastForwardDCT2_B2  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B8  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B16 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B32 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B64 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT2_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

plus optional 4-point DST-7 transform:

void fastForwardDST7_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

In BMS, the set of transforms is further expanded to include DST-7, DCT-5, DCT-8, and DCT-1 or dyadic sizes from 4..128:

void fastForwardDST7_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST7_B8  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST7_B16 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST7_B32 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST7_B64 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST7_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B8  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B16 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B32 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B64 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT5_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B8  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B16 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B32 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B64 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDCT8_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B4  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B8  (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B16 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B32 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B64 (const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST1_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

In all cases, both forward and inverse transforms of each transform type and size are implemented as separate functions.

In TrQuant.h  or TrQuant class API, the transform functionality is defined by 2 main functions/methods:

Void invTransformNxN (TransformUnit &tu, ComponentID &compID, PelBuf &pResi, QpParam &cQPs);

Void transformNxN    (TransformUnit &tu, ComponentID &compID, QpParam &cQP, TCoeff &uiAbsSum, Ctx &ctx);

These functions extract transform sizes using TU and component ID parameters:

const CompArea &rect      = tu.blocks[compID];

const UInt uiWidth        = rect.width;

const UInt uiHeight       = rect.height;

and then redirect transform execution to the appropriate transform function defined in TrQuant_EMT.cpp. For example, in VTM, the execution of transforms is done as follows:

void xTrMxN(const int bitDepth, const Pel *residual, size_t stride, TCoeff *coeff, size_t width, size_t height, ...)

{

const int iWidth  = (int)width;

const int iHeight = (int)height;

[...]

TCoeff block[MAX_TU_SIZE * MAX_TU_SIZE];

TCoeff   tmp[MAX_TU_SIZE * MAX_TU_SIZE];

for(Int y = 0; y < iHeight; y++)

for(Int x = 0; x < iWidth; x++)

block[(y * iWidth) + x] = residual[(y * stride) + x];

switch (iWidth)

{

case 2:     fastForwardDCT2_B2(block, tmp, shift_1st, iHeight, 0, iSkipW, 0);  break;

case 4:     fastForwardDCT2_B4(block, tmp, shift_1st, iHeight, 0, iSkipW, 0);  break;

case 8:     fastForwardDCT2_B8  (block, tmp, shift_1st, iHeight, 0, iSkipW, 0);  break;

case 16:    fastForwardDCT2_B16 (block, tmp, shift_1st, iHeight, 0, iSkipW, 0);  break;

case 32:    fastForwardDCT2_B32 (block, tmp, shift_1st, iHeight, 0, iSkipW, 0);  break;

case 64:    fastForwardDCT2_B64 (block, tmp, shift_1st + COM16_C806_TRANS_PREC, iHeight, 0, iSkipW, 0);  break;

case 128:   fastForwardDCT2_B128(block, tmp, shift_1st + COM16_C806_TRANS_PREC, iHeight, 0, iSkipW, 0);  break;

}

switch (iHeight)

{

case 2:     fastForwardDCT2_B2(tmp, coeff, shift_2nd, iWidth, iSkipW, iSkipH, 0);  break;

case 4:     fastForwardDCT2_B4(tmp, coeff, shift_2nd, iWidth, iSkipW, iSkipH, 0);  break;

case 8:     fastForwardDCT2_B8  (tmp, coeff, shift_2nd, iWidth, iSkipW, iSkipH, 0);  break;

case 16:    fastForwardDCT2_B16 (tmp, coeff, shift_2nd, iWidth, iSkipW, iSkipH, 0);  break;

case 32:    fastForwardDCT2_B32 (tmp, coeff, shift_2nd, iWidth, iSkipW, iSkipH, 0);  break;

case 64:    fastForwardDCT2_B64 (tmp, coeff, shift_2nd + COM16_C806_TRANS_PREC, iWidth, iSkipW, iSkipH, 0);  break;

case 128:   fastForwardDCT2_B128(tmp, coeff, shift_2nd + COM16_C806_TRANS_PREC, iWidth, iSkipW, iSkipH, 0);  break;

}

}

where the functions fastForwardDCT2_*() are DCT-2 transforms implemented in TrQuant_EMT.cpp.

In BMS, the transform type & size selection logic is even more complex, as there are many additional transform types that are involved.

## Immediate problems: code bloat, instruction cache efficiency

The immediate issue that arises with described design – it that it requires a LOT of code! Since each transform of each size is declared and expected to be implemented as a separate function – this causes massive amounts of code replication, making the design less readable, more difficult to maintain, and less cache efficient. The use of templates as means for implementation only obscures the problem, and propagates it to the level of compiler-generated code.

To demonstrate the severity of this issue, I am attaching Release-mode generated object files for modules TrQuant_EMT and TrQuant in both VTM and BMS models, along with respective object file disassembly files, showing sizes of code and data segments. Below table summarizes code segment sizes.

Table 1.1 Code segment sizes required by transforms in VTM and BMS software

| Module | VTM | BMS |
| --- | --- | --- |
| TrQuant_EMT | 0x7FBF | 0xA8C1 |
| TrQuant | 0x41CB | 0x63F5 |
| Total | 0x918A (> 36Kbytes) | 0x10CB6 (~67 Kbytes) |

Based on this table, the transforms in VTM currently require over 36Kbytes of instruction cache memory and transforms in BMS – over 67kbps. Both numbers are considerably higher than 32Kbytes L1 instruction cache limit used in most recent Intel (mobile, desktop, and server-class) processors.

In other words, current designs of transforms in VTM/BMS are simply not fitting in the instruction cache in modern processors. When CPU receives L1 cache misses, it then needs to pull data from L2 cache or main memory, which are considerably latent operations, and which slows down execution of CPU dramatically. To be able to operate at full CPU speed, transform implementations shall be much more compact.

## Is this problem solvable in principle?

If set of transforms is reduced to so-called “even” types of DCT transforms (DCT/DST transforms of types I, II, III, and IV [4]), the issue can be easily avoided by using recursive structures of such transforms. That is, a factorization of DCT-II transform of size 128 would include (as its subsets) transforms of all smaller dyadic sizes. In practice, considering Intel platform, and common factorization techniques (e.g. Leffler, Chen, etc. [4]) such stack of transforms is easily implementable by using just about 5K..10Kbytes of code.

However, with “odd” transforms (DCT/DST or types V, VI, VII, and VIII) the situation is different.

For example, DCT-V of size n is effectively a real subset of DFT or size 2n-1, which makes it impossible to use dyadic decompositions. Instead for each number n, we must look at prime-factor decompositions of numbers 2n-1, to see if PFA (prime factor algorithm) factorization technique can be applied (see e.g. [6,8-10]). This, exercise produces numbers shown in Table 1.2.

Table 1.2 Analysis of PFA component transforms needed for factorizations of DCT-V

| N | 2n-1 | Prime factors | Remarks |
| --- | --- | --- | --- |
| 4 | 7 | 7 |  |
| 8 | 15 | 3,5 |  |
| 16 | 31 | 31 | DFT factorization of size 31 is needed |
| 32 | 63 | 3,3,7 |  |
| 64 | 127 | 127 | DFT factorization of size 127 is needed |
| 128 | 255 | 3,5,17 |  |

Based on this table, it follows that factorizations of DCT-V of sizes 16 and 64 will have to be direct factorizations, sharing no common elements with transforms of other sizes. DCT-V or length 64 is promising to be particularly large. DCT-V transforms of sizes 4, 8, 32, and 255 may have common sub-transforms, derivable as real subsets of DFTs of lengths 3 and/or 5 and/or 7. In other words, we can only expect a subset of DCT-V transforms to be implementable compactly, but not all.

Similarly, as showing in [5] DST-VII and DCT-VIII of size n can both be mapped to real subsets of DFT of size 2n+1. In this case, by repeating the same exercise, we arrive at numbers showing in Table 1.3.

Table 1.3 Analysis of PFA component transforms needed for factorizations of DST-VII / DCT-VIII

| N | 2n+1 | Prime factors | Remarks |
| --- | --- | --- | --- |
| 4 | 9 | 3,3 |  |
| 8 | 17 | 17 | DFT factorization of size 17 is needed |
| 16 | 33 | 3,11 |  |
| 32 | 65 | 5,13 |  |
| 64 | 129 | 3,43 | DFT factorization of size 43 is needed |
| 128 | 257 | 257 | DFT factorization of size 257 is needed |

As easily noticed, the biggest issue here are transforms of length 128. These are the largest transforms, and they will have to be factorized uniquely, sharing nothing in common with smaller sized transforms. Unique full factorizations will also be needed for transforms of sizes 8 and 32. Only transforms of lengths 4, 16 and 64 may have common sub-transforms, derivable as imaginary subsets of DFTs of lengths 3. In other words, DST-VII / DCT-VIIIs will require most code if they are to be properly factorized.

Moreover, considering that full set of PFA modules required for such transforms:

- DFT of length 257 (for DST-VII or DCT-VIII or length 128),

- DFT of length 127 (for DCT-V of length 64),

- DFT of length 43 (for 64-point DFT DST-VII or DCT-VIII)

- DFT of length 31 (for DCT-V of length 32),

- DFT of length 17 (for DCT-V of length 128, and DST-VII or DCT-VIII of length 8)

- DFT of length 13 (for DST-VII or DCT-VIII of length 32)

- etc..

this really does not look pretty. We are looking at about a factor of 5 increase in code size compared to a stack of DCT-II transforms, suggesting, that it will be extremely challenging to implement all such transforms within 32Kbytes memory bound.

This naturally brings a question: Do we really need all these “difficult” transforms (DCT-V , DST-VII, DCT-VIII) to be implemented exactly as they are defined mathematically, or it is sufficient to use somewhat different transforms but with some consistent structure – allowing them all to be implemented efficiently and compactly?

With this being my main observation and a question that I would like to pose to the CE6 BOG, I next perform analysis and cross checks of few specific CE6 proposals.

# Cross-check for CE6-1.1 / CE6-1.1.1

This CE relates to technology originally described in JVET-J0021, JVET-C0022, JVET-D0126 and subsequently finalized (as CE 6-1.1.1) in JVET-K0375.

## Brief technical description

The AMT transforms in BMS are replaced with a new set of transforms. In addition to transforms used in BMS, which are DCT-2, DCT-5, DCT-8, DST-1, and DST-7, two more one-dimensional are added for intra coded blocks. Namely, DST-4 and identity transform are added. It was proposed in JVET-C0022 and JVET-D0126 as well as in the CFP response JVET-J0021, respectively.

For intra coding, the following 7 transform sets are defined, where each set consist of 4 transform pairs:

T0, intra = { (DST-4, DST-4), (DST-7, DST-7), (DST-4, DCT-8), (DCT-8, DST-4) }

T1, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DCT-5) }

T2, intra = { (DST-7, DST-7), (DST-7, DCT-8), (DCT-8, DST-7), (DCT-5, DCT-5) }

T3, intra = { (DST-4, DST-4), (DST-4, DCT-5), (DCT-8, DST-4), (DST-1, DST-7) }

T4, intra = { (DST-4, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T5, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T6, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DST-7) }.

The first entry in the transform pair determines vertical transform, and the second entry specifies the horizontal transform.

A look-up table is defined to assign each of the 7 transform sets based on different intra prediction modes and block sizes.

Identity transform is applied for blocks that are not exceeding 16x16 and have intra modes within the proximity of horizontal and vertical intra directions, the proximity is defined by a block size-based threshold. If transform index is equal to 3 and block satisfies the above condition, the horizontal and/or vertical identity transform is applied.

For inter coding, the same transformation scheme in BMS is proposed.

Moreover, the proposed AMTs are implemented using TAS with 6-tap band matrices (see JVET-J0066 and for the details).

## Code review / analysis

Both modules TrQuant_EMT.h /cpp and TrQuant.h / TrQuant are modified. The following new transforms are added:

void fastForwardDST4_B4(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B4(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

Additionally, the submitted code includes #if ENABLE_AMT_COMPLEXITY_REDUCTION … #else … #endif sections with certain additional optimizations included. Such optimizations fall in 2 domains:

- the existing DCT-II transform implementations are replaced with something that looks like partially factorized DCT-II, wherein the even parts are being recursively reduced to butterflies, and odd parts are left to be performed as vector-matrix products.

- DST-7, DST-4, DCT-V, and other transforms are implemented by using DCT-IIs with certain adaptation steps added. Such adaptation is described in JVET-J0066.

The main issue that I see with adaptation technique described in JVET-J0066 is that it tries to achieve something that is impossible mathematically. DST-VII or DCT-V or DCT-VIII cannot be computed via DCT-II by using some magical band-diagonal matrices. The correct transformation to DCT-II must be a product of matrix of transform that needs to be converted by the inverse of DCT-II, and such product is not a diagonal matrix for any of the above-mentioned transforms. Therefore, in my understanding, JVET-J0066 effectively proposes a replacement of most DCT/DST-type transforms by some other transforms, defined as products of some given band-diagonal matrices and DCT-II. This is indeed one possible way of reducing the diversity of factorizations that I have described in Section 1.3, and it is indeed a much welcome contribution. But to call such adapted transforms DST-VII or DCT-V or DST-4 or DCT-VIII is a bit misleading.

The other comment that I have is on implementation. Instead of calling DCT-II as functions in cases when transforms are reduced to DCT-II, the provided implementation replicates DCT-II code all over! Hence instead of making the design more compact and manageable it makes it larger, messier, and less efficient considering CPU-based implementations.

The code size requirements by this implementation are studied by analysis of object files and summarized in below table.

Table 2.1. Code segment sizes in CE6-1.1.1

| Module | CE6-1.1.1 | CE6-1.1.1 + ENABLE_AMT_COMPLEXITY_REDUCTION |
| --- | --- | --- |
| TrQuant_EMT | 0xAF19 | 0x1E208 |
| TrQuant | 0x657C | 0x657C |
| Total | 0x11495 (>69Kbytes) | 0x24784 (> 145Kbytes) |

As evident, the amount of code used by CE6-1.1.1 is slightly larger compared to BMS, and over 2x larger when ENABLE_AMT_COMPLEXITY_REDUCTION flag is defined.

The document JVET-K0375 does not list complexity numbers, but they are self-evident.

Finally, I must note that the proposed implementation of DCT-II factorizes only even (DCT-II) parts of the transform, this achieving less than a factor of 2 speedup compared to full matrix multiplication. There are indeed much more efficient factorizations, and it is not clear to me why this particular implementation was selected. It adds much code but achieves only a modest speedup.

## Simulation results

Simulations were conducted using two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as described in JVET-J1010 [3]. BMS 1.1 software is used with VTM configuration files for the VTM configuration and with BMS configuration files for the BMS configuration. The CE software implementation was pulled from SVN, CE6-1.1 directory.

Table 2.2 shows the results of using VTM and BMS configurations of the proposed method.  Table 2.3 shows the results with ENABLE_AMT_COMPLEXITY_REDUCTION flag turned on.

Table 2.2. Results for CE6-1.1.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.40% | -1.26% | -1.07% | 202% | 142% | -0.24% | 0.08% | -0.09% | 99% | 101% |
| Class A2 | -3.26% | -1.43% | -1.36% | 210% | 129% | -0.21% | 0.05% | -0.02% | 99% | 99% |
| Class B | -3.55% | -1.49% | -1.47% | 215% | 129% | -0.21% | -0.11% | -0.15% | 100% | 101% |
| Class C | -3.39% | -1.11% | -1.43% | 220% | 114% | -0.42% | -0.42% | -0.43% | 101% | 100% |
| Class E | -4.50% | -2.91% | -2.91% | 197% | 126% | -0.48% | -0.42% | -0.61% | 98% | 99% |
| Overall | -3.60% | -1.59% | -1.62% | 210% | 127% | -0.31% | -0.17% | -0.26% | 100% | 100% |
| Class D | -3.38% | -0.87% | -0.91% | 232% | 110% | -0.50% | -0.51% | -0.55% | 100% | 100% |
| Class F (optional) | -2.43% | -1.23% | -1.20% | 211% | 111% | -1.33% | -1.59% | -1.80% | 102% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | 0.06% | 0.20% | 176% | 119% | -0.08% | 0.06% | -0.10% | 100% | 101% |
| Class A2 | -2.00% | 0.42% | 0.62% | 161% | 111% | -0.01% | 0.24% | 0.15% | 100% | 100% |
| Class B | -2.53% | -0.25% | -0.40% | 163% | 111% | -0.11% | 0.06% | 0.04% | 99% | 100% |
| Class C | -2.15% | 0.08% | -0.19% | 166% | 107% | -0.15% | -0.12% | -0.27% | 98% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.20% | 0.03% | -0.02% | 166% | 112% | -0.09% | 0.05% | -0.05% | 99% | 100% |
| Class D | -1.96% | 0.34% | 0.01% | 163% | 104% | -0.17% | -0.22% | -0.07% | 99% | 98% |
| Class F (optional) | -1.98% | -0.41% | -0.45% | 174% | 107% | -1.14% | -1.22% | -1.10% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.90% | 0.62% | 0.64% | 167% | 114% | -0.02% | 0.05% | -0.14% | 99% | 98% |
| Class C | -1.35% | 0.84% | 1.12% | 160% | 109% | -0.09% | 0.02% | -0.30% | 100% | 98% |
| Class E | -1.04% | 1.55% | 1.08% | 147% | 108% | -0.19% | -0.22% | 1.15% | 99% | 101% |
| Overall | -1.50% | 0.93% | 0.91% | 159% | 111% | -0.09% | -0.03% | 0.13% | 99% | 99% |
| Class D | -1.21% | 0.92% | 0.32% | 149% | 104% | -0.05% | -0.36% | 0.06% | 100% | 103% |
| Class F (optional) | -1.25% | -0.19% | -0.28% | 165% | 108% | -0.59% | -0.59% | -0.59% | 100% | 103% |

Table 2.3. Results for CE6-1.1.1 with ENABLE_AMT_COMPLEXITY_REDUCTION

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.39% | -1.26% | -1.07% | 202% | 116% | -0.24% | 0.06% | -0.01% | 98% | 89% |
| Class A2 | -3.20% | -1.45% | -1.36% | 209% | 113% | -0.18% | -0.04% | -0.05% | 97% | 89% |
| Class B | -3.50% | -1.46% | -1.45% | 211% | 111% | -0.19% | -0.02% | -0.06% | 98% | 92% |
| Class C | -3.36% | -1.17% | -1.43% | 220% | 107% | -0.43% | -0.31% | -0.34% | 100% | 98% |
| Class E | -4.43% | -2.98% | -2.92% | 197% | 111% | -0.49% | -0.36% | -0.50% | 99% | 95% |
| Overall | -3.56% | -1.61% | -1.61% | 209% | 111% | -0.30% | -0.13% | -0.19% | 98% | 93% |
| Class D | -3.33% | -1.01% | -1.13% | 231% | 106% | -0.48% | -0.34% | -0.55% | 100% | 98% |
| Class F (optional) | -2.41% | -1.19% | -1.23% | 209% | 103% | -1.35% | -1.59% | -1.89% | 101% | 96% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.90% | 0.01% | 0.26% | 161% | 106% | -0.09% | 0.12% | -0.01% | 93% | 95% |
| Class A2 | -1.96% | 0.31% | 0.66% | 151% | 104% | 0.02% | 0.20% | 0.19% | 95% | 98% |
| Class B | -2.48% | -0.17% | -0.37% | 153% | 103% | -0.08% | -0.04% | 0.07% | 95% | 98% |
| Class C | -2.14% | 0.19% | -0.27% | 159% | 103% | -0.19% | -0.04% | -0.28% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.17% | 0.06% | -0.01% | 156% | 104% | -0.09% | 0.04% | -0.02% | 95% | 97% |
| Class D | -1.93% | 0.02% | 0.09% | 157% | 103% | -0.24% | 0.07% | -0.17% | 98% | 100% |
| Class F (optional) | -1.96% | -0.32% | -0.56% | 166% | 102% | -1.17% | -1.29% | -1.22% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.90% | 0.50% | 0.85% | 155% | 104% | -0.01% | 0.05% | -0.19% | 94% | 94% |
| Class C | -1.36% | 0.74% | 0.90% | 153% | 106% | -0.10% | 0.04% | -0.19% | 96% | 96% |
| Class E | -1.13% | 1.31% | 1.74% | 139% | 105% | -0.18% | 0.71% | 0.85% | 95% | 98% |
| Overall | -1.53% | 0.78% | 1.09% | 150% | 105% | -0.08% | 0.21% | 0.07% | 95% | 96% |
| Class D | -1.22% | 0.51% | 0.55% | 145% | 102% | -0.03% | -0.11% | -0.31% | 97% | 99% |
| Class F (optional) | -1.36% | -0.20% | -0.17% | 156% | 102% | -0.61% | -0.72% | -1.33% | 96% | 98% |

These results are confirming ones reported by the proponents of this CE.

# Cross-check for CE6-1.2 / CE6-1.2.1

This CE relates to technology originally proposed in JVET-J0066, and then finalized to use in BMS software in JVET-K0272 (as CE 6-1.2.1) and in JVET-K0375 (as CE 6-1.1.1).

The cross check for CE6-1.1.1 considering ENABLE_AMT_COMPLEXITY_REDUCTION flag enabled serves the purpose of cross check for CE6-1.2 in the context of this technology being used jointly with CE6-1. Below we only focus on application defined in JVET-K0272.

## Brief technical description & Code analysis

Please refer to sections 2.1 and 2.3.

All comments provided for case when ENABLE_AMT_COMPLEXITY_REDUCTION flag is enabled are applicable in this case.

The code size requirements by implementation of CE6-1.2.1 are summarized in below table.

Table 3.1. Code segment sizes in CE6-1.2.1 vs BMS

| Module | CE6-1.2.1 | BMS |
| --- | --- | --- |
| TrQuant_EMT | 0x1845A | 0xA8C1 |
| TrQuant | 0x63F5 | 0x63F5 |
| Total | 0x1E84F (>122Kbytes) | 0x10CB6 (~67 Kbytes) |

As evident, the amount of code used by CE6-1.2.1 is almost 2x larger compared to BMS. This is indeed a consequence of replication of code of DCT-II factorization in all transforms.

## Simulation results

Simulations were conducted using two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as described in JVET-J1010 [3]. BMS 1.1 software is used with VTM configuration files for the VTM configuration and with BMS configuration files for the BMS configuration. The CE software implementation was pulled from SVN, CE6-1.2 directory.

Table 3.2 shows the results of using VTM and BMS configurations of the proposed method.

Table 3.2. Simulation results for CE6-1.2.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.27% | -1.34% | -1.18% | 201% | 114% | -0.02% | 0.10% | -0.04% | 97% | 86% |
| Class A2 | -3.10% | -1.67% | -1.45% | 207% | 109% | 0.02% | 0.06% | 0.03% | 99% | 89% |
| Class B | -3.40% | -1.44% | -1.47% | 211% | 110% | 0.02% | 0.16% | 0.08% | 98% | 92% |
| Class C | -3.09% | -0.96% | -1.20% | 217% | 106% | -0.01% | 0.05% | 0.02% | 99% | 96% |
| Class E | -4.29% | -2.68% | -2.71% | 197% | 109% | 0.01% | 0.02% | -0.07% | 99% | 91% |
| Overall | -3.41% | -1.56% | -1.57% | 208% | 110% | 0.00% | 0.09% | 0.01% | 98% | 91% |
| Class D | -3.00% | -0.60% | -0.69% | 231% | 106% | 0.02% | 0.20% | 0.03% | 99% | 98% |
| Class F (optional) | -1.87% | -0.78% | -0.79% | 207% | 104% | -0.01% | 0.09% | 0.00% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | -0.06% | 0.07% | 161% | 105% | 0.02% | 0.11% | -0.05% | 93% | 95% |
| Class A2 | -2.01% | 0.16% | 0.38% | 150% | 104% | 0.02% | 0.08% | -0.04% | 95% | 97% |
| Class B | -2.48% | -0.23% | -0.50% | 152% | 103% | 0.01% | 0.15% | 0.08% | 95% | 98% |
| Class C | -2.05% | 0.17% | -0.15% | 158% | 104% | 0.03% | 0.12% | -0.26% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.16% | -0.01% | -0.11% | 155% | 104% | 0.02% | 0.12% | -0.06% | 95% | 98% |
| Class D | -1.81% | 0.38% | -0.08% | 156% | 102% | 0.04% | 0.14% | 0.11% | 98% | 100% |
| Class F (optional) | -1.51% | -0.16% | -0.18% | 166% | 103% | 0.02% | 0.02% | -0.08% | 97% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.91% | 0.76% | 0.49% | 154% | 101% | 0.00% | -0.05% | -0.09% | 94% | 94% |
| Class C | -1.29% | 0.92% | 0.88% | 153% | 104% | -0.06% | 0.15% | 0.05% | 96% | 96% |
| Class E | -1.22% | 1.60% | 1.23% | 138% | 100% | -0.28% | -0.52% | 0.25% | 95% | 99% |
| Overall | -1.53% | 1.02% | 0.81% | 150% | 102% | -0.09% | -0.10% | 0.04% | 94% | 96% |
| Class D | -1.23% | 0.67% | 0.65% | 144% | 100% | 0.07% | 0.23% | 0.08% | 97% | 98% |
| Class F (optional) | -1.17% | -0.26% | -0.09% | 155% | 100% | -0.12% | 0.27% | -0.17% | 95% | 96% |

These results are confirming ones reported by the proponents of this CE.

# Cross-check for CE6-1.3 /CE6-1.3.1

This relates to technology originally described in JVET-J022 and finalized (as CE6-1.3.1) in JVET-K0264.

## Brief technical description

In JVET-J022, it is proposed to reduce the set of primary transforms to DCT-2, DCT-8, DST-4, DST-7. The AMT set thus is reduced to DCT-8, DST-4 and DST-7. Moreover, DST-7 is implemented based on the DCT-8 transform matrix, on which flipping, and sign changes operations are applied.

In JVET-K0264, the computation of DST-IV is obtained by exploiting the fact that DCT-VI is already present as odd portions of DCT-II computations, and then applying well known relation between DCT-IV and DST-IV, that involves only reordering and sign changing operations.

## Code review / analysis

Both modules TrQuant_EMT.h/cpp and TrQuant.cpp are modified. The following new transforms are added:

void fastForwardDST4_B4(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardDST4_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B4(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

void fastForwardIDTR_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use);

However, the implementations of DST-IVs, which were promised to be done via DCT-II are not really factorized in any way. Instead they are all implemented by direct matrix products:

#if TCH_EMT

void fastForwardDST4_B4(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 4 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr4[DST4][0]);

}

void fastForwardDST4_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 8 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr8[DST4][0]);

}

void fastForwardDST4_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 16 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr16[DST4][0]);

}

void fastForwardDST4_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 32 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr32[DST4][0]);

}

void fastForwardDST4_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 64 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr64[DST4][0]);

}

void fastForwardDST4_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM< 128 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr128[DST4][0]);

}

#endif

where matrix products of different sizes are generated by template:

template< Int uiTrSize >

inline void _fastForwardMM( const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use, const TMatrixCoeff* tc )

{

const int  rnd_factor  = 1 << (shift - 1);

const int  reducedLine = line - iSkipLine;

const int  cutoff      = uiTrSize - iSkipLine2;

TCoeff *pCoef;

for( int i = 0; i<reducedLine; i++ )

{

pCoef = dst;

const TMatrixCoeff* iT = tc;

for( int j = 0; j<cutoff; j++ )

{

int iSum = 0;

for( int k = 0; k<uiTrSize; k++ )

{

iSum += src[k] * iT[k];

}

pCoef[i] = (iSum + rnd_factor) >> shift;

pCoef += line;

iT += uiTrSize;

}

src += uiTrSize;

}

//...

}

The inner matrix product loop is highlighted.

The computations of DST-VII via DCT-VIII as suggested by JVET-J022 are implemented as follows

#if TCH_EMT

void fastForwardDST7_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM_STR< 8 >( src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr8[DCT8][0] );

}

void fastForwardDST7_B16(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM_STR< 16 >( src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr16[DCT8][0] );

}

void fastForwardDST7_B32(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM_STR< 32 >( src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr32[DCT8][0] );

}

void fastForwardDST7_B64(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM_STR< 64 >( src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr64[DCT8][0] );

}

void fastForwardDST7_B128(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

_fastForwardMM_STR< 128 >( src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr128[DCT8][0] );

}

#else

where template _fastForwardMM_STR<>() is defined as follows:

template< Int uiTrSize >

inline void _fastForwardMM_STR(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use, const TMatrixCoeff* tc) // reverse the order R then transform T then sign change S

{

const int  rnd_factor = 1 << (shift - 1);

const int  reducedLine = line - iSkipLine;

const int  cutoff = uiTrSize - iSkipLine2;

TCoeff *pCoef;

for (int i = 0; i<reducedLine; i++)

{

pCoef = dst;

const TMatrixCoeff* iT = tc;

int ss = 1;

for (int j = 0; j<cutoff; j++)

{

int iSum = 0;

for (int k = 0; k<uiTrSize; k++)

{

iSum += src[k] * iT[uiTrSize-k-1];// reverse ordering

}

pCoef[i] = (ss*iSum + rnd_factor) >> shift;// sign changing

ss *= -1;

pCoef += line;

iT += uiTrSize;

}

src += uiTrSize;

}

//...

}

Compared to direct matrix multiply, this function does include sign changes (marked as green) and reordering (marked as blue) operations. In other words, technically, the implementations of DST-VII is consistent with description provided by the proponent.

However, if DCT-VIII transform is not factorized at all – this implementation brings no advantages in terms of computational complexity or compactness of implementation. It only saves the number of transform coefficients that need to be stored, which is a small number relative to the amount of memory needed to store the code (CPU instructions) for such transforms.

The code used by implementation of CE6-1.3.1 is analyzed by producing dump of object files. The results of this analysis are summarized in below table.

Table 4.1 Code segment sizes in CE6-1.3.1 vs BMS

| Module | CE6-1.3.1 | CE6-1.3.1, no DST4, IDTR | BMS |
| --- | --- | --- | --- |
| TrQuant_EMT | 0xB831 | 0xAED3 | 0xA8C1 |
| TrQuant | 0x657C | 0x63F5 | 0x63F5 |
| Total | 0x11DAD (~71Kbytes) | 0x112C8 (~68Kbytes) | 0x10CB6 (~67 Kbytes) |

For clarity, in addition to analyzing full implementation of CE6-1.3.1 we also looked at reduced version produced by removing DST-VI and identity transforms (done by replacing #if TCH_EMT in line 1953 of module TrQuant_EMT.cpp with #if 0 and using BMS version of TrQuant.cpp). This reduced version only tests effect of replacing direct DST-IIV implementations with conversions to DCT-VIII.

As evident, the amount of code used by CE6-1.3.1 is by about 4Kbytes larger than implementation in BMS.

The reduced implementation of CE6-1.3.1, which only replaces DST-VII with DCT-VIII is by about 1Kbyte (or 1,554 bytes to be exact) larger than implementation in BMS. On the other hand, the amount memory saved for storing coefficients is about (128 + 64 + 32 + 16 + 8 + 4) * 4 = 236*4 = 944 bytes. In other words, there is more memory added than saved when proposed technique is applied to non-factorized transforms.

Summarizing the above, I can only say that I like the idea of exploiting commonalities between DST-VI and DCT-II and DST-VII and DCT-VIII. Furthermore, I think that it absolutely makes sense, especially when coupled with factorizations of transforms, allowing common core parts of such transforms to be coded only once. This approach should, in theory, produce code that is more compact and more efficient.

However, the supplied implementation of this idea in BMS, relying on matrix vector products that are replicated for most transforms does not seem to demonstrate such advantages.

## Simulation results

Simulations were conducted using two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as described in JVET-J1010 [3]. BMS 1.1 software is used with VTM configuration files for the VTM configuration and with BMS configuration files for the BMS configuration. The CE software implementation was pulled from SVN, CE6-1.3.1 branch.

Table 4.2 shows the results of using VTM and BMS configurations of the proposed method.

Table 4.2. Simulation results for CE6-1.3.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.44% | -1.35% | -1.24% | 194% | 137% | -0.13% | 0.04% | -0.11% | 98% | 102% |
| Class A2 | -3.29% | -1.41% | -1.35% | 200% | 129% | -0.10% | 0.18% | 0.08% | 102% | 102% |
| Class B | -3.52% | -1.49% | -1.56% | 209% | 126% | -0.09% | 0.02% | -0.07% | 100% | 101% |
| Class C | -3.39% | -1.17% | -1.43% | 222% | 114% | -0.38% | -0.30% | -0.33% | 105% | 98% |
| Class E | -4.46% | -2.95% | -3.04% | 197% | 123% | -0.35% | -0.31% | -0.44% | 101% | 99% |
| Overall | -3.60% | -1.63% | -1.69% | 206% | 125% | -0.20% | -0.08% | -0.17% | 101% | 100% |
| Class D | -3.34% | -1.07% | -1.01% | 233% | 121% | -0.47% | -0.39% | -0.42% | 102% | 101% |
| Class F (optional) | -3.44% | -1.35% | -1.24% | 194% | 137% | -0.13% | 0.04% | -0.11% | 98% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.97% | -0.05% | 0.19% | 168% | 125% | -0.04% | -0.18% | -0.14% | 101% | 105% |
| Class A2 | -2.04% | 0.39% | 0.55% | 152% | 118% | 0.02% | 0.13% | 0.21% | 99% | 106% |
| Class B | -2.51% | -0.17% | -0.43% | 161% | 117% | -0.03% | 0.05% | -0.06% | 102% | 103% |
| Class C | -2.15% | -0.11% | -0.28% | 169% | 113% | -0.12% | -0.10% | -0.22% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.21% | -0.02% | -0.07% | 163% | 118% | -0.05% | -0.02% | -0.06% | 101% | 103% |
| Class D | -1.97% | 0.52% | -0.31% | 168% | 110% | -0.19% | -0.35% | -0.16% | 107% | 96% |
| Class F (optional) | -1.97% | -0.05% | 0.19% | 168% | 125% | -0.04% | -0.18% | -0.14% | 101% | 105% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.94% | 0.89% | 0.98% | 170% | 120% | -0.03% | -0.01% | 0.12% | 104% | 104% |
| Class C | -1.36% | 0.85% | 0.91% | 161% | 120% | -0.06% | 0.20% | 0.06% | 102% | 101% |
| Class E | -1.18% | 0.48% | 1.30% | 153% | 120% | -0.20% | -0.49% | 0.44% | 103% | 103% |
| Overall | -1.56% | 0.77% | 1.04% | 163% | 120% | -0.08% | -0.06% | 0.18% | 103% | 103% |
| Class D | -1.24% | 1.37% | 0.31% | 146% | 115% | -0.01% | -0.19% | 0.20% | 101% | 100% |
| Class F (optional) | -1.94% | 0.89% | 0.98% | 170% | 120% | -0.03% | -0.01% | 0.12% | 104% | 104% |

The results are confirming ones reported by the proponents.

# Cross-check for CE6-1.5 /CE6-1.5.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-1.8 /CE6-1.8.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-1.10 /CE6-1.10.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-1.11 /CE6-1.11.1

This relates to technology originally described in JVET-J017 and finalized (as CE6-1.3.1) in JVET-K0096.

## Brief technical description

For core transform, simplified Adaptive multiple core transform is utilized by adding two transforms (DST7 and DCT8) on top of HEVC, and maximum AMT is restricted to a size of 32. A forward N×N DST7 with 2N + 1 length Discrete Fourier Transform (DFT) is implemented to achieve the N×N DST7. 2N+1 FFT has been reformulated to a two dimensional FFT. DCT8 can be derived from DST7 with sign changes and reordering just before and after the DST7 computation. Hence, DST7 is reused in this implementation for DCT8.

The document JVET-K0096 offers the following details on actual DST-7 factorizations that are proposed:

Fig 8.1 16-point DST-VII via 33-point prime-factor split DFT.

Fig 8.2 32-point DST-VII via 65-point prime-factor split DFT.

This is indeed all logical and derived by using existing factorization techniques [5-10].

## Review of code / Analysis

Both modules TrQuant_EMT.cpp and TrQuant.cpp are modified, however the bulk of changes resides in TrQuant_EMT.cpp. The changes related to CE6-1.11.1 are localized by using preprocessor blocks: #if DST7_IMPL_WITH_FFT...#else ... #endif.

The code does implement fast DST-VII factorizations exactly as described in the proposal.

For example, DST-VII of length 8 is implemented via FFT as follows:

void fastForwardDST7_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

#if DST7_IMPL_WITH_FFT

Int rnd_factor = 1 << (shift - 1);

const Int uiTrSize = 8;

TCoeff *pCoef;

const Int  reducedLine = line - iSkipLine;

const Int  cutoff = uiTrSize - iSkipLine2;

for (Int i = 0; i < reducedLine; i++)

{

pCoef = dst;

Int FFTIn[8], FFTOut[8];

Int *pFFTIn = FFTIn, *pFFTOut = FFTOut;

*pFFTIn++ = src[1]; *pFFTIn++ = src[3]; *pFFTIn++ = src[5]; *pFFTIn++ = src[7];

*pFFTIn++ = src[6]; *pFFTIn++ = src[4]; *pFFTIn++ = src[2]; *pFFTIn = src[0];

xDST7_FFT_B8(FFTIn, FFTOut);

for (Int j = 0; j < uiTrSize; j++) {

pCoef[i] = (Int)(((*pFFTOut++) + rnd_factor) >> shift);

pCoef += line;

}

src += uiTrSize;

}

//...

#else

_fastForwardMM< 8 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr8[DST7][0]);

#endif

}

Similarly, the implementation of DCT-VIII is implemented by calling DST-VII:

void fastForwardDCT8_B8(const TCoeff *src, TCoeff *dst, Int shift, Int line, Int iSkipLine, Int iSkipLine2, Int use)

{

#if DST7_IMPL_WITH_FFT

const Int uiTrSize = 8;

TCoeff *pBlock = block_buf;

TCoeff *pCoef = dst;

//Pre-processing

const Int  reducedLine = line - iSkipLine;

src += uiTrSize;

for (Int i = 0; i < reducedLine; i++) {

for (Int k = 0; k < uiTrSize; k++) {

*pBlock++ = *--src;

}

src += (uiTrSize << 1);

}

pBlock = block_buf;

fastForwardDST7_B8(pBlock, dst, shift, line, iSkipLine, iSkipLine2, use);

//Post-processing

const Int  cutoff = uiTrSize - iSkipLine2;

for (Int i = 0; i < reducedLine; i++) {

pCoef = dst + i;

for (Int j = 0; j < cutoff; j++) {

if (j & 0x1) {

*pCoef = -*pCoef;

}

pCoef += line;

}

}

// ...

#else

_fastForwardMM< 8 >(src, dst, shift, line, iSkipLine, iSkipLine2, use, g_aiTr8[DCT8][0]);

#endif

}

The code used by implementation of CE6-1.3.1 is analyzed by producing dump of object files. The results of this analysis are summarized in below table.

Table 8.1 Code segment sizes in CE6-1.11.1 vs BMS

| Module | CE6-1.11.1 | BMS |
| --- | --- | --- |
| TrQuant_EMT | 0xCD9D | 0xA8C1 |
| TrQuant | 0x634F | 0x63F5 |
| Total | 0x130EC (~76Kbytes) | 0x10CB6 (~67 Kbytes) |

As evident, the amount of code used by CE6-1.3.1 is by about 9Kbytes larger than implementation in BMS. This increase is expected as added factorizations generally need more code than matrix multiplies. But the advantage of factorizations is significant reduction in computational complexity.

## Simulation results

Simulations were conducted using two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as described in JVET-J1010 [3]. The CE software implementation was pulled from SVN, CE6-1.11.1 branch. Only tests of BMS-1.1 vs CE6-11.1 and VTM-1.1 vs CE6-11.1 are performed.

Table 4.2 shows the results of using VTM and BMS configurations of the proposed method.

Table 8.2. Simulation results for CE6-1.11.1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.11% | -1.10% | -0.98% | 193% | 106% | 0.29% | 0.64% | 0.48% | 97% | 92% |
| Class A2 | -3.02% | -1.64% | -1.52% | 199% | 105% | 0.20% | 0.24% | 0.02% | 97% | 94% |
| Class B | -3.29% | -1.51% | -1.51% | 206% | 105% | 0.19% | 0.15% | 0.19% | 97% | 93% |
| Class C | -3.04% | -1.01% | -1.31% | 213% | 100% | 0.05% | 0.00% | 0.10% | 96% | 94% |
| Class E | -4.21% | -2.94% | -2.82% | 190% | 103% | 0.16% | 0.01% | 0.06% | 95% | 93% |
| Overall | -3.31% | -1.59% | -1.59% | 201% | 104% | 0.17% | 0.19% | 0.17% | 97% | 93% |
| Class D | -2.93% | -0.86% | -0.86% | 224% | 102% | 0.08% | 0.05% | 0.00% | 96% | 96% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.51% | 0.09% | 0.13% | 143% | 104% | 0.46% | 0.65% | 0.34% | 80% | 97% |
| Class A2 | -1.86% | 0.33% | 0.45% | 136% | 102% | 0.23% | 0.33% | -0.01% | 87% | 98% |
| Class B | -2.32% | -0.26% | -0.42% | 142% | 102% | 0.20% | 0.45% | 0.20% | 87% | 98% |
| Class C | -2.01% | 0.05% | -0.14% | 146% | 96% | 0.03% | 0.23% | -0.15% | 89% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -1.98% | 0.01% | -0.06% | 142% | 101% | 0.21% | 0.41% | 0.09% | 86% | 98% |
| Class D | -1.79% | 0.25% | -0.15% | 146% | 95% | 0.05% | 0.24% | 0.01% | 92% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.1 |  |  |  |  | Over BMS-1.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.70% | 0.73% | 1.04% | 139% | 102% | 0.22% | 0.16% | 0.02% | 86% | 97% |
| Class C | -1.28% | 0.75% | 0.85% | 142% | 99% | 0.01% | -0.15% | -0.40% | 89% | 95% |
| Class E | -0.93% | 1.38% | 1.54% | 122% | 100% | 0.19% | 0.03% | -1.94% | 88% | 97% |
| Overall | -1.37% | 0.90% | 1.10% | 135% | 101% | 0.14% | 0.03% | -0.61% | 87% | 96% |
| Class D | -1.16% | 0.60% | 0.16% | 137% | 101% | 0.13% | -0.12% | -0.40% | 92% | 98% |

The results are confirming ones reported by the proponents.

We note that the CE6-11.1 proposal also reports results for several additional cases and modes (e.g. AMT + disabling secondary transform, low QP conditions, etc). If deemed essential, I will try to reproduce those as and report in subsequent update to this document.

# Cross-check for CE6-1.13 /CE6-1.13.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-2.2 /CE6-2.2.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-2.5 /CE6-2.5.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-3.1 /CE6-3.1.1

In progress. Will be included in next revision of this document.

# Cross-check for CE6-3.2 /CE6-3.2.1

In progress. Will be included in next revision of this document.

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- V. Britanak, K. R. Rao, and P. Yip, “Discrete Cosine and Sine Transforms: General Properties, Fast Algorithms and Integer Approximations”, Academic Press - Elsevier, Oxford, UK, 2007.

- R. K. Chivukula and Y. Reznik, "Fast Computing of Discrete Cosine and Sine Transforms of Types VI and VII," Proc. SPIE Vol. 8135, Applications of Digital Image Processing, San Diego, CA, August 22-25, 2011

- Y. Reznik and R. K. Chivukula, "Design of fast transforms for high-resolution image and video coding", Proc. SPIE Vol. 7443, Applications of Digital Image Processing, pp. 744312-1--18, 2009.

- M.T. Heideman, “Computation of an Odd-Length DCT from a Real-Valued DFT of the Same Length”, IEEE Trans. Signal Proc., vol. 40, no. 1, Jan 1992.

- D.P. Kolba, T.W. Parks, “A Prime Factor FFT Algorithm Using High-Speed Convolution”, IEEE Trans. ASSP, vol. 25, Aug 1977.

- C.S. Burrus, T.W. Parks, “DFT/FFT and Convolution Algorithms – Theory and Implementation”, Wiley, New York, 1985.

- H.J. Nussbaumer, “Fast Fourier Transform and Convolution Algorithms”, Springer, 1981.

# Patent rights declaration(s)

Yuriy Reznik may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).