| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0357 |
| --- | --- |

| Title: | CE4.3.3: Locally adaptive motion vector resolution and MVD coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang, Yu Han, Chun-Chi Chen, Chao-Hsiung Hung, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +86-10-59233584 yzh@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes locally adaptive motion vector resolution (LAMVR) which is used in BMS. On top of that, an MVD coding method is also introduced. The test results of CE4 test 4.3.3 including 2 sub tests are presented. For the “Tool on case 1 (VTM as reference)” test in RA case, the sub test a) “LAMVR” achieves objective gain of 1.75% in terms of average luma BD-rate reduction and sub test b) “LAMVR + MVD coding” achieves 1.79% BD-rate reduction.

# Introduction

LAMVR in this CE4 test is identical to the one used BMS and JEM. This contribution provides the test results of LAMVR coding tool in both VTM and BMS. Based on LAMVR, an MVD coding method is also tested for both reference softwares.

# Technical description

The details of algorithm description of LAMVR can be found in [1].

Motion vector differences (MVDs) can be signalled in different precision to allow flexible MVD coding for video sequences with different resolution. In JEM and BMS, MVD could be signalled either in units of quarter luma sample, integer luma sample or four luma samples. A variable length MVD resolution flag (0 to 2 bits) is conditionally signalled in CU level for those that have at least one non-zero MVD components with the first bit identifying whether quarter luma sample MVD precision is used. When the first bit (equal to 1) indicates that quarter luma sample MVD precision is not used, a second bit is signalled to indicate if integer luma sample MVD precision or four luma samples MVD precision is used.

When a zero is signalled for the first bit of the MVD resolution flag, quarter luma sample MVD resolution is used. When the MVD resolution flag is not signalled (which means both MVDs for reference list 0 and reference list 1 are zero), quarter luma sample MVD resolution is inferred. In the other cases when integer-luma sample MVD precision or four luma samples MVD precision is adopted, the MVP candidates in the AMVP candidate list is rounded to the corresponding precision.

The MVD resolution is decided for each CU according to the RD check result in the encoder side. To avoid performing RD checks three times for all the three MVD resolutions in each CU, the following schemes are applied to speedup the encoding process.

- When performing RD check of a CU with quarter luma sample MVD precision, the motion information of the current CU is stored (with quarter luma sample accuracy). The stored motion information is then rounded to the corresponding precision before it is used as the starting point for further small range motion vector refinement during the RD check for the same CU with integer luma sample and four luma samples MVD resolution. By doing so, the time consumed in the motion estimation process is not duplicated three times.

- The RD check for 4 luma samples MVD resolution is only invoked conditionally. The RD cost for integer luma sample MVD precision and quarter luma sample MVD precision is compared to decide whether it is necessary to proceed to check four luma samples MVD precision for the same CU. When the RD cost for quarter luma sample MVD precision is much smaller than that of the integer luma sample MVD precision, the RD check for four luma samples MVD precision is skipped.

In the following part, the proposed MVD coding scheme is illustrated, the details of algorithm description can also be found in [2].

For MVD signaling, the MVD context modelling and binarization design in the original LAMVR tool can be found in Table 1.

Table 1. Context modeling and binarization parameter in CE4.3.3 test (a)

| Syntax element | Context model/Binarization parmeter | Integer or four luma sample precision, or POC distance =1 | Quarter pel and POC distance > 1 |
| --- | --- | --- | --- |
| abs_mvd_greater0_flag | CtxIdx | 0 | 0 |
| abs_mvd_greater1_flag | CtxIdx | 1 | 1 |
| abs_mvd_remaining_value | Exp-Golomb Code Parameter | 1 | 1 |

The scheme is modified in the proposed MVD coding method so that the binarization and context modeling are dependent on the MVD precision and the POC distance between the current frame and the reference frame. Table 2 shows the context modeling and binarization parameter after the proposed modification.

Table 2. Context modeling and binarization parameter in CE4.3.3 test (b)

| Syntax element | Context model/Binarization parmeter | Integer or four luma sample precision, or POC distance =1 | Quarter pel and POC distance > 1 |
| --- | --- | --- | --- |
| abs_mvd_greater0_flag | CtxIdx | 0 | 0 |
| abs_mvd_greater1_flag | CtxIdx | 1 | 2 |
| abs_mvd_remaining_value (unary portion) | CtxIdx | 3 | 4 |
| abs_mvd_remaining_value | Exp-Golomb Code Parameter | 1 | 2 |

# Simulation results

The common test conditions specified in [3] are followed. As descripted in CE4 [4], the test specification is as below, and the “Tool on case 1 (VTM as reference)” with default encoder configuration and “Tool off case 2 (BMS LAMVR off as reference)” with IMV set to 0 in the BMS encoder configuration are used as anchors.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.3.3 | LAMVR and MVD coding a) Adaptive MV resolution (1/4 or 1 or 4). b) a) + MVD coding. | Yan Zhang (Qualcomm) | Xiaoyu Xiu (InterDigital) |

## Test4.3.3.a LAMVR

Configuration as below: both enable LAMVR in tool on case 1 and tool off case 2 tests.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| a) LAMVR | IMV : 2 | IMV : 2 |

Simulation result is as below:

## Test4.3.3.b LAMVR + MVD coding

Configuration as below: both enable LAMVR and MVD coding in tool on case 1 and tool off case 2 tests.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| b) LAMVR + MVD coding | IMV : 2 MVD coding macro :1 | IMV : 2 MVD coding macro :1 |

Simulation result is as below:

# Conclusion

This contribution presents LAMVR and MVD coding technologies which are studied in CE4. This contribution is CE4 test 4.3.3 includes 2 sub tests. For the “Tool on case 1 (VTM as reference)” tests in RA configuration, the sub test a) “LAMVR” achieves objective gain of 1.75% in terms of average luma BD-rate reduction; the sub test b) “LAMVR + efficient MVD coding” achieves 1.79% BD-rate reduction.

# References

- JVET-G1001, “Algorithm description of Joint Exploration Test Model 7 (JEM7),” Torino, IT, 13-21 Jul. 2017.

- JVET-D0123, “Enhanced Motion Vector Difference Coding,” Chengdu, CN, 15–21 October 2016.

- JVET-J1010, “JVET common test conditions and software reference configurations,” San Diego, US, 10-20 Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).