| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0358-v1 |
| --- | --- |

| Title: | CE9.1.5: MVD-based Early-skip Condition for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 {chunchic,wchien,martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposed a method for Decoder-side Motion Vector Refinement (DMVR) to adaptively skip its refinement process when the motion vector difference (MVD) between the selected candidate and any previous ones in the same merge list falls within a pre-defined threshold. Specifically, when the x- and y-component of a MVD between the selected candidate and a previous one are both smaller than a 1-pixel-wide interval, the refinement process is not performed. Same as the DMVR in BMS-1.0, this CE performs integer-pel search within a local search range [-1,1] without half-pel refinement. Experiments were conducted on top of the CE-9 anchors (i.e. BMS-1.0 w/ VTM cfg. and BMS-1.0 w/ BMS cfg.) for Random Access case. The results show 2.7% BD-rate reduction against VTM, with 8% and 27% encoding and decoding time inflation, respectively, and almost no performance variation against BMS. Additional comparison with DMVR on VTM shows this CE reduces 9% decoding time without BD-rate lose. Detailed performance results are summarized as follows.

CE9.1.5 vs. BMS: (Y)  0.0%, (U)  0.0%, (V)  0.0%, (Enc.) 100%, (Dec.) 99%.

CE9.1.5 vs. VTM: (Y) -2.7%, (U) -2.5%, (V) -2.7%, (Enc.) 108%, (Dec.) 127%.

DMVR  vs. VTM: (Y) -2.6%, (U) -2.5%, (V) -2.7%, (Enc.) 109%, (Dec.) 136%.

# Introduction

Decoder-side Motion Vector Refinement (DMVR) [1][2] is a bi-prediction technique with two initially signalled motion vectors (MV) that can be further refined by using bilateral template matching prediction. A bilateral template is formed based on the original bi-prediction reference block and is used to find the best matching block, in terms of a distortion cost, from its reference pictures in reference list 0 (L0) and 1 (L1), respectively. Then, the resulting MVs pointing to the best matching positions respectively in L0 and L1 are taken to form the final bi-prediction reference block. In BMS-1.0 [3], DMVR can be switched on and off adaptively without signalling overhead. Its MV refinement process operates only when the following conditions are all satisfied.

- The coding unit is a merge block.

- Uni-directional, ATMVP, STMVP, affine and merge candidates using IC mode are all excluded.

- Only one of (POCref0 – POCcur) and (POCref1 – POCcur) values is negative.

# Technical Description

In this contribution, we imposed an additional condition on DMVR to confine its MV refinement process. DMVR starts its MV refinement process only when the three conditions aforementioned in Section 1 and the proposed one below are all met.

- The MV difference between the selected candidate and any of the previous candidates in the same merge list is larger than a pre-defined threshold (that is, 1-pixel-wide interval).

# Experimental Results

The experiments for DMVR with the proposed condition imposed were conducted using the BMS-1.0 software [3] and the common test conditions [4] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. The BMS and VTM anchors were generated using the BMS-1.0 software with BMS and VTM configurations, respectively. Macro settings specified in the BMS-1.0 software are not changed. Parallel encoding and decoding were applied to Random Access case for anchors and tests. The search configurations of DMVR as recorded in Table 1 is the same as that in the BMS-1.0 software. The performance results generated on top of the VTM and BMS anchors were summarized in Table 2.

It is reported that the BD-rate variation against the BMS anchor is less than 0.1%, with 1% decoding time reduction. In Table 3, we additionally recorded the crosscheck results of DMVR from AHG 13 [5] for comparison. When the proposed method is compared with the original DMVR on top of VTM, the BD-rate difference between them is less than 0.1%, with 9% reduction in decoding time.

- Search configurations of DMVR.

| Initial MV signaled | Sub-CU refinement | Neighboring recon. samples used | Max # of SAD calculation | Max. SR | Cost Function | Interpolation filter/tap no. | Search Patterns |
| --- | --- | --- | --- | --- | --- | --- | --- |
| yes | no | no | 9 | 1 | SAD | DCTIF/8 | square for int-pel search no half-pel search |

- BD-rate results and processing time ratios relative to VTM and BMS anchors.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.1.5 vs. VTM anchor |  |  |  |  | CE9.1.5 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.12% | -2.35% | -2.68% | 107% | 126% | -0.01% | -0.01% | 0.06% | 100% | 99% |
| Class A2 | -4.78% | -4.70% | -4.72% | 108% | 134% | 0.04% | -0.03% | 0.01% | 100% | 100% |
| Class B | -2.15% | -2.03% | -2.09% | 109% | 125% | -0.05% | -0.03% | -0.05% | 100% | 99% |
| Class C | -1.37% | -1.69% | -1.84% | 108% | 125% | -0.02% | 0.07% | -0.03% | 99% | 99% |
| Overall | -2.66% | -2.54% | -2.67% | 108% | 127% | -0.02% | 0.00% | -0.01% | 100% | 99% |
| Class D | -1.27% | -1.45% | -1.70% | 110% | 128% | 0.03% | 0.03% | 0.05% | 99% | 98% |

- Performance comparison of CE9.1.5 and DMVR on top of VTM anchor.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.1.5 vs. VTM anchor |  |  |  |  | Original DMVR vs. VTM anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.12% | -2.35% | -2.68% | 107% | 126% | -3.11% | -2.37% | -2.70% | 108% | 135% |
| Class A2 | -4.78% | -4.70% | -4.72% | 108% | 134% | -4.82% | -4.72% | -4.75% | 109% | 143% |
| Class B | -2.15% | -2.03% | -2.09% | 109% | 125% | -2.08% | -2.01% | -2.11% | 110% | 134% |
| Class C | -1.37% | -1.69% | -1.84% | 108% | 125% | -1.38% | -1.69% | -1.80% | 111% | 136% |
| Overall | -2.66% | -2.54% | -2.67% | 108% | 127% | -2.65% | -2.54% | -2.67% | 109% | 136% |
| Class D | -1.27% | -1.45% | -1.70% | 110% | 128% | -1.25% | -1.56% | -1.77% | 114% | 139% |

# Conclusion

This contribution presented a MVD-threshold-based constraint imposed on DMVR and was tested in the context of CE 9. It is reported that the BD-rate variation introduced from the proposed method is less than 0.1% when compared with the original DMVR. The report also recorded an average decoding-time reduction of 9% when the proposed method is compared with the original DMVR on top of VTM. In view of these, we would recommend the committee to consider integrating the proposed technique into the next release of both BMS and VTM software.

# References

- X. Chen, J. An and J. Zheng, “Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0029, Oct. 2016.

- J. Chen, E. Alshina and et. al., “Algorithm Description of Joint Exploration Test Model 7 (JEM7),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1001, July 2017.

- BMS-1.0 software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, and et. al., “JVET Common Test Conditions and Software Reference Configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, Apr. 2018.

- W.-J. Chien, J. Boyce and et. al., “JVET AHG Report: Tool Reporting Procedure (AHG13),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0013, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).