| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0359-v2 |
| --- | --- |

| Title: | CE9.2.5/9.2.6: DMVR with Template-free Bilateral Matching |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 {chunchic,wchien,martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposed a template-free modification on the bilateral template matching process in Decoder-side Motion Vector Refinement (DMVR). In BMS-1.0, when the motion vectors (MVs) of a merge block point respectively to a past and a future reference picture, DMVR takes its bi-prediction reference block as its template to find two matching blocks separately from the two reference pictures. Instead, this contribution computes a matching cost between two uni-prediction reference blocks pointed to by the merge block MVs. Then a local search based on bilateral matching around these MVs is performed. Resulting MVs are further refined at half-pel precision. Conventional 8-point square search is applied during the whole search process. The matching cost is a block-adaptive Mean Removed Sum of Absolute Difference (MRSAD) plus motion cost. In addition, the MV refinement process is switchable adaptively based on the MV difference between merge candidates. Experiments were conducted on top of the anchors (i.e. BMS-1.0 w/ VTM cfg. and BMS-1.0 w/ BMS cfg.) for Random Access case. Two CE tests are evaluated: (CE9.2.5) DCTIF for search with search range [-8,8], (CE9.2.6) Bilinear filter for search with search range [-2,2]. Detailed performance results are summarized as follows.

CE9.2.5 vs. BMS: (Y) -1.8%, (U) -1.6%, (V) -1.6%, (Enc.) 102%, (Dec.) 112%.

CE9.2.6 vs. BMS: (Y) -1.4%, (U) -1.1%, (V) -1.2%, (Enc.) 100%, (Dec.) 108%.

CE9.2.5 vs. VTM: (Y) -5.3%, (U) -4.7%, (V) -4.9%, (Enc.) 116%, (Dec.) 164%.

CE9.2.6 vs. VTM: (Y) -4.8%, (U) -4.1%, (V) -4.3%, (Enc.) 112%, (Dec.) 150%.

# Introduction

Decoder-side Motion Vector Refinement (DMVR) [1][2] is a template-based bi-prediction technique. A bilateral template is formed based on the bi-prediction reference block of a merge block with motion vectors (MVs) pointing respectively to a past and a future reference picture. Then, as shown in Figure 1, it is used to find two matching blocks around the initial prediction reference blocks separately from the two reference pictures. In BMS-1.0 [3], each matching block is found at 9 positions, including the one pointed to by the initial MV and its 8 immediately neighboring positions with identical phase shift on the reference picture. The sum of absolute differences (SAD) is used as the cost function. The resulting MVs (e.g. v0’ and v1’ in Figure 1) pointing respectively to the two matching blocks are taken to replace the original MVs of the merge block. At the last, motion compensation is performed based on the refined MVs. DMVR is switchable adaptively between on and off from checking following three conditions.

- The coding unit is a merge block.

- Uni-directional, ATMVP, STMVP, affine and merge candidates using IC mode are all excluded.

- Only one of (POCref0 – POCcur) and (POCref1 – POCcur) values is negative.

The MV refinement process operate only when all the conditions are satisfied.

I0: Past Ref.

Current Pic.

I1: Future Ref.

Cur. Block

Ref. 0

Ref. 1

v0

v1

1/2

1/2

Bilateral Template = (Ref. 0 + Ref. 1) / 2

Cur. Block

Ref. 0’

v0

v1’

Bilateral Template

v0’

Ref. 1’

v1

Find Matching Block

Find Matching Block

I0: Past Ref.

Current Pic.

I1: Future Ref.

x

x

- Bilateral template matching.

# Technical Description

Template-free bilateral matching is proposed for DMVR to avoid its template-based refinement process. This contribution directly computes the bilateral matching cost between the uni-prediction reference blocks (denoted as I0(x+v0) and I1(x+v1) and x as the coordinate of a pixel within the current block) pointed to by the initial bi-prediction MVs (e.g. v0 and v1 in Figure 1). Then, the conventional 9-point square search is applied to perform a local search based on bilateral matching within search range [-8, 8] around the initial bi-prediction MVs. Specifically, supposing the initial MVs are v0(0) and v1(0), at the first search iteration, nine MV pairs (i.e. v0(0)+∆ and v1(0)-∆ where ∆∈{(0,0), (-1,1), (0,1), (1,1), (1,0), (1,-1), (0,-1), (-1,-1), (-1,0)}) and find out the optimal ∆* which can lead to the lowest bilateral matching cost. In this contribution, the cost function is defined as the distortion between I0(x+v0(0)+∆) and I1(x+v1(0)-∆) plus motion cost. The distortion function can be either Mean Removed SAD (MRSAD) or SAD, defined respectively in Eqs. (1) and (2). MRSAD is taken only when the number of block pixels is larger than 64. As specified in Eq. (3), the motion-cost function is 4 times the 1-norm of the scaled ∆, where the scaling factor is determined by the MV precision stored in the motion field (e.g. 2 for quarter-pel precision and 4 for 16th-pel precision).

| SAD(0)(∆) = ∑x ¦ I0(x+v0(0)+∆) - I1(x+v1(0)-∆)) ¦ |  |
| --- | --- |
| MRSAD(0)(∆) = ∑x ¦ (I0(x+v0(0)+∆) - µ0) - (I1(x+v1(0)-∆) - µ1) ¦ |  |
| R(∆) = 4 * ¦ ∆ << scale ¦ |  |

After the optimal ∆* is found, the iteration process updates the initial MVs (v0(0) and v1(0)) by using ∆*. Specifically, we have v0(1)=v0(0)+∆ and v1(1)=v1(0)-∆). Then, after advancing all the superscripts in the above description by 1, the same iteration process repeats until ∆* is equal to (0,0) or the 26th iteration is reached. The output MV pair (denoted as v0(n) and v1(n), n≥1) are then refined again with one additional iteration at half-pel precision. The resulting MV par is then taken to replace the original MVs (v0(0) and v1(0)) of the merge block. At the last, motion compensation is performed based on the refined MVs.

In addition, we imposed an additional condition [6] on DMVR to confine its MV refinement process. DMVR starts its MV refinement process only when the three conditions as mentioned in Section 1 and the proposed one below are all satisfied.

- The MV difference between the selected candidate and any of the previous candidates is not less than a pre-defined threshold (i.e. ¼-, ½- and 1-pixel-wide intervals for blocks with less than 64, less than 256 and more than 256 pixels, respectively).

# Experimental Results

The experiments for the proposed template-free DMVR were conducted using the BMS-1.0 software [3] and the common test conditions [4] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. BMS and VTM anchors were generated using the BMS-1.0 software with BMS and VTM configurations, respectively. None of the original macros in the software was changed. Parallel encoding and decoding were applied to Random Access cases for anchors and tests. The search configurations of DMVR and the two CE tests were recorded in Table 1. The performance results generated on top of the VTM and BMS anchors were summarized in Table 2.

For CE9.2.5, the report shows an average Y BD-rate of -1.76%, with 2% and 12% of encoding and decoding time increase, respectively, when the proposed method is compared with BSM anchor. The gain is even higher on top of VTM anchor. It is recorded that the average Y BD-rate reaches -5.82%, with 16% and 64 of encoding/decoding time increase, respectively.

For CE9.2.6, it is reported that every number presented above drops when bilinear interpolation filter and small search range are introduced. Around 0.3-0.5% of Y BD-rate is not collectable. Both encoder and decoder run faster by 4% and 14%, respectively, on VTM and 2% and 4%, respectively, on BMS.

We also quoted the performance results of DMVR relative to VTM anchor from AHG 13 [5]. When compared the proposed method with the original DMVR, CE9.2.5 can further improve the Y BD-rate by 2.63%, with an increase of encoding and decoding time by 7% and 28%, respectively. CE9.2.6 can improve the Y BD-rate by 2.11%, with an increase of encoding and decoding time by 3% and 14%, respectively.

- Search configuration used in this CE in comparison with that of DMVR

|  | Initial MV signaled | Sub-CU refinement | Neighboring recon. samples used | Max # of SAD calculation | Max. SR | Cost Function | Interpolation filter/tap no. | Search Patterns |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DMVR | yes | no | no | 9 | 1 | SAD | DCTIF/8 | square for int-pel search no half-pel search |
| CE9.2.5 | yes | no | no | less than 9 + 25*5 + 8 = 142 | 8 | >64: MRSAD ≤64: SAD | DCTIF/8 | square for int-pel search square for half-pel search |
| CE9.2.6 | yes | no | no | less than 9 + 5 + 11*2 + 8 = 44 | 2 | >64: MRSAD ≤64: SAD | ME: Bilinear/2 MC: DCTIF/8 | square for int-pel search square for half-pel search |

- BD-rate results and processing time ratios relative to VTM and BMS anchors.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.5 vs. VTM anchor |  |  |  |  | CE9.2.5 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.60% | -4.47% | -5.10% | 112% | 159% | -1.84% | -1.54% | -1.84% | 100% | 115% |
| Class A2 | -8.86% | -7.87% | -7.96% | 115% | 180% | -2.65% | -2.22% | -2.31% | 102% | 112% |
| Class B | -4.62% | -3.97% | -4.04% | 117% | 158% | -1.57% | -1.42% | -1.36% | 103% | 110% |
| Class C | -3.18% | -3.39% | -3.47% | 118% | 165% | -1.29% | -1.23% | -1.27% | 101% | 113% |
| Overall | -5.28% | -4.70% | -4.88% | 116% | 164% | -1.76% | -1.55% | -1.62% | 102% | 112% |
| Class D | -3.36% | -3.25% | -3.40% | 121% | 167% | -1.55% | -1.39% | -1.22% | 101% | 112% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.6 vs. VTM anchor |  |  |  |  | CE9.2.6 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.88% | -3.73% | -4.21% | 108% | 141% | -1.32% | -0.91% | -1.15% | 99% | 108% |
| Class A2 | -8.07% | -7.13% | -7.30% | 110% | 159% | -2.10% | -1.63% | -1.83% | 100% | 107% |
| Class B | -4.30% | -3.56% | -3.66% | 114% | 152% | -1.35% | -1.13% | -1.12% | 101% | 108% |
| Class C | -2.77% | -2.95% | -3.00% | 113% | 150% | -0.99% | -0.93% | -0.94% | 100% | 107% |
| Overall | -4.76% | -4.15% | -4.32% | 112% | 150% | -1.40% | -1.13% | -1.22% | 100% | 108% |
| Class D | -3.05% | -3.01% | -3.24% | 115% | 153% | -1.32% | -1.15% | -1.11% | 99% | 106% |

- Performance comparison of CE9.2.5/6 and DMVR on top of VTM anchor

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.5 vs. VTM anchor |  |  |  |  | Original DMVR vs. VTM anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.60% | -4.47% | -5.10% | 112% | 159% | -3.11% | -2.37% | -2.70% | 108% | 135% |
| Class A2 | -8.86% | -7.87% | -7.96% | 115% | 180% | -4.82% | -4.72% | -4.75% | 109% | 143% |
| Class B | -4.62% | -3.97% | -4.04% | 117% | 158% | -2.08% | -2.01% | -2.11% | 110% | 134% |
| Class C | -3.18% | -3.39% | -3.47% | 118% | 165% | -1.38% | -1.69% | -1.80% | 111% | 136% |
| Overall | -5.28% | -4.70% | -4.88% | 116% | 164% | -2.65% | -2.54% | -2.67% | 109% | 136% |
| Class D | -3.36% | -3.25% | -3.40% | 121% | 167% | -1.25% | -1.56% | -1.77% | 114% | 139% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.6 vs. VTM anchor |  |  |  |  | Original DMVR vs. VTM anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.88% | -3.73% | -4.21% | 108% | 141% | -3.11% | -2.37% | -2.70% | 108% | 135% |
| Class A2 | -8.07% | -7.13% | -7.30% | 110% | 159% | -4.82% | -4.72% | -4.75% | 109% | 143% |
| Class B | -4.30% | -3.56% | -3.66% | 114% | 152% | -2.08% | -2.01% | -2.11% | 110% | 134% |
| Class C | -2.77% | -2.95% | -3.00% | 113% | 150% | -1.38% | -1.69% | -1.80% | 111% | 136% |
| Overall | -4.76% | -4.15% | -4.32% | 112% | 150% | -2.65% | -2.54% | -2.67% | 109% | 136% |
| Class D | -3.05% | -3.01% | -3.24% | 115% | 153% | -1.25% | -1.56% | -1.77% | 114% | 139% |

# Conclusion

This contribution presented a template-free bilateral matching technique for DMVR and was evaluated in the context of CE 9. Reported results show that at least 2% additional gain on average Y BD-rate numbers is collectable when we compared the two CE tests with the original DMVR in BMS-1.0 software. The results also show at least 1.4% of Y BD-rate improvement on top of BMS anchor with less than 2% encoding-time increase and around 10% decoding-time inflation. In the conclusion, we would recommend the committee to consider integrating the proposed technique into the next release of BMS/VTM software and working draft.

# References

- X. Chen, J. An and J. Zheng, “Decoder-Side Motion Vector Refinement Based on Bilateral Template Matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-D0029, Oct. 2016.

- J. Chen, E. Alshina and et. al., “Algorithm Description of Joint Exploration Test Model 7 (JEM7),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-G1001, July 2017.

- BMS-1.0 software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, and et. al., “JVET Common Test Conditions and Software Reference Configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, Apr. 2018.

- W.-J. Chien, J. Boyce and et. al., “JVET AHG Report: Tool Reporting Procedure (AHG13),” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0013, July 2018.

- C.-C. Chen, W.-J. Chien and M. Karczewicz, “CE9.1.5: MVD-based Early-skip Condition for DMVR,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0358, July 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).