| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0360-v1 |
| --- | --- |

| Title: | CE9-related: Bilateral Matching with Constrained Motion Vector Storage |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +886-3-566-1980 {chunchic,wchien,martak}@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution introduced a constraint on the Template-free DMVR (CE9.2.5/9.2.6, JVET-K0359) to prohibit using refined motion vectors (MVs) for MV prediction and boundary strength (BS) calculation in deblocking filter. Specifically, refined MVs are taken only for motion compensation, but not for storage. None of them is kept in the motion field. Thus, only un-refined motion field is available for MV prediction and BS calculation. Experiments were conducted on top of the BMS (BMS-1.0 w/ BMS cfg.) and VTM anchors (BMS-1.0 w/ VTM cfg.) with Random Access configuration. Detailed performance results are summarized as follows.

Modified CE9.2.5 vs. BMS: (Y) -0.6%, (U) -0.7%, (V) -0.6%, (Enc.) 103%, (Dec.) 113%.

Modified CE9.2.6 vs. BMS: (Y) -0.3%, (U) -0.3%, (V) -0.3%, (Enc.) 100%, (Dec.) 103%.

Modified CE9.2.5 vs. VTM: (Y) -3.6%, (U) -3.4%, (V) -3.5%, (Enc.) 118%, (Dec.) 165%.

Modified CE9.2.6 vs. VTM: (Y) -3.0%, (U) -2.7%, (V) -2.8%, (Enc.) 109%, (Dec.) 142%.

# Introduction and Technical Description

In CE 9.2.5 and 9.2.6 [1], a template-free modification on the bilateral template matching process in Decoder-side Motion Vector Refinement (DMVR) was proposed. When DMVR is applied, the bi-prediction MVs of a merge block are refined and stored in the motion field to replace of the original merge block motion. Then, the refined MVs are used for performing motion compensation for the merge block. As the refined MVs are stored in the motion field, they can be taken as reference when spatial MV prediction, temporal MV prediction and boundary strength (BS) calculation perform.

# Technical Description

In this contribution, when DMVR is applied, refined MVs are taken only for motion compensation, but not for storage. None of them is allowed to update the motion field, so the motion filed is kept the same as it was before the MV refinement process is performed. Accordingly, only the un-refined motion field is available for later use, including spatial MV prediction, temporal MV prediction and BS calculation in deblocking filter.

# Experimental Results

The experiments of the proposed method were conducted using the BMS-1.0 software [2] and the common test conditions [3] to collect BD-rate numbers, encoding-time ratio and decoding-time ratio. BMS and VTM anchors were generated using the same BMS-1.0 software but with BMS and VTM configurations, respectively. None of the original macros in the software was changed. Parallel encoding and decoding were applied to Random Access cases for anchors and tests. The search configurations are the same as CE9.2.5 or CE 9.2.6. The performance results generated on top of the VTM and BMS anchors were summarized in Table 1. The performance results of CE9.2.5 and CE 9.2.6 [1] were also recorded in Table 2 for comparison.

- BD-rate results and processing time ratios relative to VTM and BMS anchors.

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modified CE9.2.5 vs. VTM anchor |  |  |  |  | Modified CE9.2.5 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.62% | -3.10% | -3.41% | 115% | 158% | -0.58% | -0.66% | -0.71% | 101% | 109% |
| Class A2 | -6.04% | -5.57% | -5.66% | 118% | 177% | -0.84% | -0.75% | -0.79% | 104% | 113% |
| Class B | -3.12% | -2.95% | -2.90% | 119% | 167% | -0.64% | -0.68% | -0.62% | 105% | 117% |
| Class C | -2.24% | -2.53% | -2.54% | 119% | 159% | -0.51% | -0.55% | -0.43% | 101% | 112% |
| Overall | -3.57% | -3.39% | -3.46% | 118% | 165% | -0.63% | -0.65% | -0.62% | 103% | 113% |
| Class D | -2.44% | -2.58% | -2.86% | 121% | 162% | -0.73% | -0.65% | -0.58% | 102% | 113% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Modified CE9.2.6 vs. VTM anchor |  |  |  |  | Modified CE9.2.6 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.79% | -2.25% | -2.51% | 107% | 136% | -0.01% | -0.06% | -0.06% | 100% | 103% |
| Class A2 | -4.89% | -4.46% | -4.58% | 109% | 148% | -0.27% | -0.20% | -0.23% | 101% | 103% |
| Class B | -2.80% | -2.46% | -2.48% | 109% | 142% | -0.45% | -0.46% | -0.41% | 100% | 102% |
| Class C | -1.93% | -2.13% | -2.17% | 111% | 144% | -0.31% | -0.25% | -0.25% | 100% | 103% |
| Overall | -2.98% | -2.73% | -2.82% | 109% | 142% | -0.29% | -0.27% | -0.26% | 100% | 103% |
| Class D | -2.23% | -2.18% | -2.38% | 113% | 147% | -0.60% | -0.50% | -0.44% | 100% | 103% |

- Performance results of CE9.2.5 and CE9.2.6 [1].

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.5 vs. VTM anchor |  |  |  |  | CE9.2.5 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -5.60% | -4.47% | -5.10% | 112% | 159% | -1.84% | -1.54% | -1.84% | 100% | 115% |
| Class A2 | -8.86% | -7.87% | -7.96% | 115% | 180% | -2.65% | -2.22% | -2.31% | 102% | 112% |
| Class B | -4.62% | -3.97% | -4.04% | 117% | 158% | -1.57% | -1.42% | -1.36% | 103% | 110% |
| Class C | -3.18% | -3.39% | -3.47% | 118% | 165% | -1.29% | -1.23% | -1.27% | 101% | 113% |
| Overall | -5.28% | -4.70% | -4.88% | 116% | 164% | -1.76% | -1.55% | -1.62% | 102% | 112% |
| Class D | -3.36% | -3.25% | -3.40% | 121% | 167% | -1.55% | -1.39% | -1.22% | 101% | 112% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | CE9.2.6 vs. VTM anchor |  |  |  |  | CE9.2.6 vs. BMS anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.88% | -3.73% | -4.21% | 108% | 141% | -1.32% | -0.91% | -1.15% | 99% | 108% |
| Class A2 | -8.07% | -7.13% | -7.30% | 110% | 159% | -2.10% | -1.63% | -1.83% | 100% | 107% |
| Class B | -4.30% | -3.56% | -3.66% | 114% | 152% | -1.35% | -1.13% | -1.12% | 101% | 108% |
| Class C | -2.77% | -2.95% | -3.00% | 113% | 150% | -0.99% | -0.93% | -0.94% | 100% | 107% |
| Overall | -4.76% | -4.15% | -4.32% | 112% | 150% | -1.40% | -1.13% | -1.22% | 100% | 108% |
| Class D | -3.05% | -3.01% | -3.24% | 115% | 153% | -1.32% | -1.15% | -1.11% | 99% | 106% |

# Conclusion

This contribution presented a modification on the Template-free DMVR that refined MVs are prohibited for spatial and temporal MV prediction and BS calculation. Reported results show around 3% and 0.3-0.6% Y BD-rate reductions are achievable against VTM and BMS anchors, respectively. When compared with the original CE 9.2.5 and 9.2.6, this proposal shows less than 5% difference in encoding and decoding runtime. We would recommend the committee to consider integrating the proposed technique into the next software release of BMS/VTM software and working draft or further study in a context of a CE.

# References

- C.-C. Chen, W.-J. Chien and M. Karczewicz, “CE9.2.5/9.2.6: DMVR with Template-free Bilateral Matching,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0359, July 2018.

- BMS-1.0 software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- J. Boyce, K. Suehring, and et. al., “JVET Common Test Conditions and Software Reference Configurations,” Joint Video Exploration Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-J1010, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).