| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0363 |
| --- | --- |

| Title: | CE4.5.2: Motion compensated boundary pixel padding |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yan Zhang, Yu Han, Chun-Chi Chen, Chao-Hsiung Hung, Wei-Jung Chien, Marta Karczewicz 5775 Morehouse Drive, San Diego, CA, U.S.A. 92121 | Tel: Email: | +86-10-59233584 yzh@qti.qualcomm.com wchien@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution describes motion compensated boundary pixel padding (MC Padding) which is introduced on top of BMS and VTM to replace repetitive boundary padding. The test results of CE test 4.5.2 are presented in this contribution. For the “Tool on case 1 (VTM as reference)” tests in RA case, the simulation shows the proposed tool can achieve BD-rate reduction of 0.22%.

# Introduction

Motion compensated boundary padding is proposed to replace the repetitive boundary padding method. This contribution briefly introduces the MC Padding algorithm and provides the test results of MC Padding tool in both VTM and BMS.

# Technical description

The details of algorithm description of MC Padding can also be found in [1].

When motion compensation is performed in the decoder side, it is possible that the motion vector point to a reference block that is partially or entirely located outside the reference slice. Without boundary padding, these pixels will be unavailable. Traditionally, the reference slice is padded using repetitive padding method which repeats the outer most pixel in each of the four directions for a certain amount of times depending on the padding size. These padded pixels can only provide very limited information since it is very likely that the padded area does not contain any meaningful content compare to those that lie inside the boundary.

In this proposal, a new boundary pixel padding method is introduced so that more information can be provided by the padded areas in the reference slice. A motion vector is first derived from the boundary 4x4 block inside the current frame. If the boundary 4x4 block is intra coded or the motion vector is not available, repetitive padding will be used. If the boundary 4x4 block is predicted using uni-directional inter prediction. The only motion vector within the block will be used for motion compensated boundary pixel padding. Using the position of the boundary 4x4 block and its motion vector, a corresponding starting position can be computed in the reference frame. From this starting position till the boundary of the reference slice in the given padding direction, a 4xM or Mx4 image slip can be fetched where M is the distance between the horizontal/vertical coordinate of the boundary pixel position and the starting position depending on the padding direction. Here in this CE test, M is forced to be smaller that 64. In case of bi-directional inter prediction, only the motion vector, which points to the pixel position farther away from the frame boundary in the reference slice in terms of the padding direction, is used in motion compensated boundary pixel padding. The difference between the DC values of the boundary 4x4 block in the current slice and its corresponding reference 4x4 block in the reference slice is used as the offset to filter the fetched motion compensated image slip before it is copied to the padding area beyond the image boundary.

# Simulation results

The common test conditions specified in [2] are followed. As descripted in CE4 [3], the test specification is as below, the “Tool on case 1 (VTM as reference)” and “Tool off case 1 (BMS as reference)” with default encoder configuration are used as anchors.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.5.2 | MC Padding | Yan Zhang (Qualcomm) | Minsoo Park (Samsung) |

## Test4.5.2 MC Padding

Configuration as below: both enable MC Padding in tool on case 1 and tool off case 1 tests.

| Test | Configuration of Tool On Test | Configuration of Tool Off Test |
| --- | --- | --- |
| MC Padding | MCFramePad : true | MCFramePad : true |

Simulation results are as below:

# Conclusion

This contribution presents MC Padding technology which is studied in CE4. This contribution is CE4 test 4.5.2. For the “Tool on case 1 (VTM as reference)” tests in RA case, the test “MC Padding” achieved objective gain of 0.22% in terms of average luma BD-rate reduction.

# References

- JVET-J0021, “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, San Diego, US, 10–20 Apr. 2018.

- JVET-J1010, “JVET common test conditions and software reference configurations,” San Diego, US, 10-20 Apr. 2018.

- H. Yang, S. Liu, K. Zhang, JVET-J1024, “Description of CE4: Inter prediction and motion vector coding”, Apr. 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).