| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0364-v1 |
| --- | --- |

| Title: | Non-CE4: separate merge candidate list for sub-block modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ting Fu Huanbang Chen Haitao Yang Jianle Chen | Email: | futing5@huawei.com chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

Currently, several sub-block based motion vector prediction (MVP) methods has been proposed, e.g., ATMVP, STMVP, Affine Merge and Planar MVP [1]-[3]. All of these methods generate fine granularity motion fields for CUs. Because of the commonality among these methods, this contribution presents a separate merge candidate list for sub-block based MVPs. The candidate list is constructed by integrating two or more sub-block based MVPs into a separate merge candidate list.

Simulation results reportedly show on average x%, x% luma BD-rate gain for RA and LB configuration over VTM1.0, and x%, x% luma BD-rate gain for RA and LB configuration over BMS1.0.

Proposed method

In the affine merge mode of BMS1.0, only the first available affine neighbour can be used to derive motion information of affine merge mode [2]. In JVET-J0024, a candidate list for affine merge mode is constructed by searching valid affine neighbours and combining the neighbor motion information of each control point. [1]. JVET-J0061 presents a planar motion vector prediction (PMVP) method by averaging a horizontal and vertical linear interpolation on 4x4 block basis [3]. In ATMVP, each CU fetches multiple sets of motion information from multiple blocks smaller than the current CU in the collocated reference picture. All of these methods are sub-block based and aimed at generating a fine granularity motion field. To take advantage of these methods while remove the redundant check, a separate merge candidate list for sub-block based MVPs is presented in this contribution where the candidate list includes affine motion information, planar motion information and ATMVP motion information.

The separate merge candidate list is constructed as following steps:

- Insert the model based affine candidates, as depicted in JVET-J0024.

- If the current CU is not in the left top position of picture, and both width and height for the CU are larger than 16, the motion information derived from PMVP method is appended to the candidate list. Derivation for PMVP is illustrated in JVET-J0061.

- Insert the ATMVP (STMVP) candidates.

- Insert the control point based affine candidates, as depicted in JVET-J0024.

Experimental results

The test condition descripted in JVET-J1024 [4] is used for verifying the contribution. The results are summarized in Table 1.

Table 1 Summary of overall performance in RA/LB configuration

| Test a | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA |  |  |  |  |  |  |  |  |  |  |
| LB |  |  |  |  |  |  |  |  |  |  |
| Test b | Over VTM |  |  |  |  | Over BMS – AFFINE |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA |  |  |  |  |  |  |  |  |  |  |
| LB |  |  |  |  |  |  |  |  |  |  |

Conclusions

This contribution describes a separate merge candidate list for sub-block based MVPs, which improves coding efficiency by around xxx luma BD-rate in RA/LB configurations.

References

- “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, April 2018

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- N. Zhang, J. An, J. Zheng, “Planar Motion Vector Prediction”, JVET-J0061, April 2018

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).