| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0364-v2 |
| --- | --- |

| Title: | Non-CE4: separate merge candidate list for sub-block modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ting Fu Huanbang Chen Haitao Yang Jianle Chen | Email: | futing5@huawei.com chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

- Abstract

Currently, several sub-block based motion vector prediction (MVP) methods has been proposed, e.g., ATMVP, STMVP, Affine Merge enhancement in JVET-J0024 and Planar MVP in JVET-J0061. All of these methods generate fine granularity motion fields for CUs. Because of the commonality among these methods, this contribution presents a separate merge candidate list for sub-block based MVPs. The candidate list is constructed by integrating two or more sub-block based MVPs into a separate merge candidate list.

Simulation results reportedly show on average -5.22%, -3.96% luma BD-rate gain for RA and LB configuration over VTM1.0, and x%, x% luma BD-rate gain for RA and LB configuration over BMS1.0.

Proposed method

In the affine merge mode of BMS1.0, only the first available affine neighbour can be used to derive motion information of affine merge mode [2]. In JVET-J0024, a candidate list for affine merge mode is constructed by searching valid affine neighbours and combining the neighbor motion information of each control point. [1]. JVET-J0061 presents a planar motion vector prediction (PMVP) method by averaging a horizontal and vertical linear interpolation on 4x4 block basis [3]. In ATMVP, each CU fetches multiple sets of motion information from multiple blocks smaller than the current CU in the collocated reference picture. All of these methods are sub-block based and aimed at generating a fine granularity motion field. To take advantage of these methods while remove the redundant check, a separate merge candidate list for sub-block based MVPs is presented in this contribution where the candidate list contains affine motion information, planar motion information and ATMVP motion information. The max number of the candidates list is 7.

The separate merge candidate list is constructed as following steps:

- Insert the model based affine candidates, as depicted in JVET-J0024.

- If the current CU is not in the left edge or the top edge of the picture, and both width and height of the CU are larger than 16, the motion information derived from PMVP method is appended to the candidate list. Derivation for PMVP is illustrated in JVET-J0061.

- Insert the ATMVP (STMVP) candidates.

- Insert the control point based affine candidates, as depicted in JVET-J0024.

Experimental results

The test condition descripted in JVET-J1024 [4] is used for verifying the contribution. The proposed method has been implemented in BMS1.0, and the results are summarized in Table 1, Test b. Besides, results of ATMVP+ JVET-J0024+ JVET-J0061 (Test a) are tabulated in Table 1 as well. Test c in Table 1 is conducted by removing ATMVP from the proposed separate merge candidate list for sub-block based MVPs.

For Test a) “JVET-J0024 + JVET-J0061 + ATMVP”, there are three kinds of merge mode in total. The first merge mode is affine merge mode as described in JVET-J0024, and the max number of the affine merge candidates is 5. The second merge mode is planar merge mode. The planar merge mode contains only one candidate that illustrated in JVET-J0061. The third merge mode is normal merge mode. The max number of the normal merge candidates is 6 and the candidates list is as below:

- Spatial candidates for blocks A0->B0->B1->A1, as shown in Figure 1.

- ATMVP.

- Spatial candidate for block B2.

- Temporal candidate (derived as in HEVC).

- Combined candidates.

- Zero candidates.

Figure 1: Spatial candidates for normal merge mode.

For test b) “The separate merge candidate list for sub-block based MVPs”, there are two kinds of merge mode in total. The first one is the sub-block merge mode whose candidates list is described as Section 1. The max number of the sub-block merge candidates is 7. The second one is the normal merge mode. The max number of the normal merge candidates is 5 and the candidates list is as below:

- Spatial candidates for blocks A0->B0->B1->A1, as shown in Figure 1.

- Spatial candidate for block B2.

- Temporal candidate (derived as in HEVC).

- Combined candidates.

- Zero candidates.

For test c) “ATMVP + Affine-planar”, there are two kinds of merge mode in total. The first one is the normal merge mode in Test a. The second one is the affine-planar merge mode. The max number of the affine-planar merge candidates is 6, and the candidates list is as below:

- Insert the model based affine candidates, as described in JVET-J0024.

- If the current CU is not in the left edge or the top edge of the picture, and both width and height of the CU are larger than 16, the motion information derived from PMVP method is appended to the candidate list. Derivation for PMVP is illustrated in JVET-J0061.

- Insert the control point based affine candidates, as depicted in JVET-J0024.

Table 1 Summary of overall performance in RA/LB configuration

| Test a | Over VTM |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -5.17% | -4.23% | -4.33% | 171% | 150% |  |  |  |  |  |
| LB | -3.82% | -3.24% | -3.37% | 215% | 133% |  |  |  |  |  |
| Test b | Over VTM |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -5.22% | -4.29% | -4.41% | 164% | 152% |  |  |  |  |  |
| LB |  | -3.96% |  | -3.39% | -3.56% |  | 206% |  | 140% |  |  |  |  |  |  |  |  |  |  |
| Test c | Over VTM |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -5.13% | -4.19% | -4.34% | 164% | 143% |  |  |  |  |  |
| LB | -3.91% | -3.33% | -3.45% | 206% | 129% |  |  |  |  |  |

In Table 2, the summary results for CE4.2.10, CE4.2.14 and CE4.2.16 are provided for the comparison of different sub-block based modes.

Table 2 Summary of overall performance in RA/LB configuration for CE4.2.10, CE4.2.14 and CE4.2.16

| CE4.2.10 | Over VTM |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -4.08% | -3.22% | -3.29% | 155% | 117% | -0.86% | -0.69% | -0.79% | 111% | 101% |
| LB | -2.67% | -1.99% | -1.96% | 194% | 113% | -0.54% | -0.34% | -0.57% | 117% | 101% |
| CE4.2.14 | Over VTM |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.91% | -0.94% | -0.94% | 108% | 119% | -0.49% | -0.46% | -0.43% | 104% | 107% |
| LB | -0.70% | -0.64% | -0.75% | 109% | 112% | -0.51% | -0.50% | -0.42% | 105% | 104% |
| CE4.2.16 | Over VTM |  |  |  |  | Over BMS - ATMVP |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | -0.87% | -0.81% | -0.83% | 103% | 115% | -0.74% | -0.62% | -0.61% | 100% | 105% |
| LB | -1.08% | -1.27% | -1.23% | 104% | 112% | -1.02% | -1.08% | -1.25% | 99% | 106% |

Conclusions

This contribution describes a separate merge candidate list for sub-block based MVPs, which achives coding efficiency by around -5.22%/-3.96% luma BD-rate in RA/LB configurations over VTM1.0.

References

- “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, April 2018

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- N. Zhang, J. An, J. Zheng, “Planar Motion Vector Prediction”, JVET-J0061, April 2018

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).