| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0365 |
| --- | --- |

| Title: | CE3 3.2.2: Intra mode signaling with priority based MPM and non-MPM list construction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Zhijie Zhao, Jianle Chen | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

Two main modifications are proposed with respect to the Intra mode signaling on top of BMS. The first one relates to the MPM list construction which includes three main changes: The first change being, Intra modes belonging to the second tier neighbors (non-adjacent neighbors) are inserted into the MPM list after the above left mode. The second change being, that the DC mode is inserted after the second tier neighbors intra modes are inserted. The third change replaces DIA (diagonal mode) with VDIA (vertical diagonal) mode in the default list which is used to complete the MPM list.

The second modification fills the 16 selected modes and the first 14 non-selected modes with offsets to the first two angular modes from the MPM list and 5 more non-selected modes are included from a default intra mode list which is ordered based on the intra mode probability occurrence. Experimental results of the combined method are as follows:

Over VTM Anchor (AI, RA): -1.48%, -0.68% Luma coding gain with 112%, 103% encoding time and 104%, 100% decoding time.

Over BMS Anchor (AI, RA): -0.25%, -0.11% Luma coding gain with 102%, 100%, encoding time and 102%, 100% decoding time.

# Introduction

The proposed method was first introduced in the Huawei’s CfP response [1], and is evaluated as part of the CE3. The method addresses to improve the existing intra mode coding scheme in BMS with respect to BD-Rate gain without introducing any additional computational complexity.

# Background

BMS uses a total of 67 intra prediction modes. The intra mode is signaled by dividing the available intra modes into three different lists: MPM modes list, selected modes list and non-selected modes set. The 6 MPM modes are derived from five immediate spatial neighbors as shown in Fig. 1.

Fig. 1 Neighbour intra modes

The order of insertion of intra modes into the MPM list is as follows:  “Left (L)”, “Above (A)”, “Planar”, “DC”, “Below left (BL)”, “above right (AR)”, and “above left (AL)”. The 16 selected modes are chosen from the remaining 61 modes with a fixed interval of value “4”. The rest 45 modes are assigned to the non-selected modes set. The MPM mode is coded with a unary code, the selected mode is signaled with a 4-bit fixed length code, and the non-selected mode is coded with a truncated binary code. The binarization for the syntax element intra prediction mode is specified in Table 1.

| Intra prediction modes | MPM flag | Selected flag | Bin string |
| --- | --- | --- | --- |
| MPM modes (6) | 1 |  | 0 |
|  |  |  | 10 |
|  |  |  | 110 |
|  |  |  | 1110 |
|  |  |  | 11110 |
|  |  |  | 11111 |
| Selected modes (16) | 0 | 1 | 4 bits fixed length code |
| Non-selected modes (45) | 0 | 0 | Truncated binary code |

Table 1. Intra prediction mode coding in BMS

# Proposed Method

The proposed method consist of two main aspects. The first aspect relates to the modified MPM list construction which includes three main changes. The first change allows to include intra modes of “Second tier (non-adjacent)” spatial neighbors into the MPM list.  As shown in Fig. 2, the intra modes of “Second tier spatial neighbors” are also considered for inclusion in the MPM list. They are inserted after the “Bottom Left (BL)” intra mode is inserted into the MPM list.

Second Tier Neighbors

Current Coding Unit

L

BL

AL

AR

A

L

A

L

A

A

Second Tier Neighbors

L

Second Tier

Neighbors

L

A

Fig. 2 Second tier (non-adjacent) intra modes considered in MPM list construction

The second change suggests to move the inclusion of DC mode after the “Second Tier” spatial neighbor’s intra mode insertion. The third change suggests to use vertical diagonal (VDIA) mode instead of diagonal mode (DIA) mode in the default list of intra modes which are used to complete the MPM list.

The second aspect focuses on deriving the set of selected modes and non-selected modes based on the list of intra modes already included in the MPM list. Moreover the non-selected modes are truncated binarization coded: Out of the total 45 non-selected modes, the first 19 modes use 5 bits to signal the intra mode value, whereas the remaining 29 modes require 6 bits to signal the intra mode. To leverage this property, the proposed method also derives the first 19 non-selected modes based on the probability of intra mode occurrence.

The list of 16 selected modes and 14 out of the first 19 non-selected modes are generated by adding offsets of value “1” to the first 2 angular modes present in the MPM list. The remaining 5 out of first 19 non-selected modes are filled from a default intra mode list which is ordered based on the intra mode probability occurrence,

# Experimental Results

The table below shows the coding gain and processing time impact of the suggested modifications which is the combination of (1) MPM list construction modifications and (2) Selected and non-selected mode list modifications.

- BMS simulations

  - Anchor: BMS

  - Test: Anchor with Proposal

- VTM simulations

  - Anchor: VTM

  - Test: VTM + Intra65Ang == 1 + Proposal

# Conclusion

Two modifications have been proposed for the intra mode signaling of BMS.

(1) MPM list modifications which are:

- Inclusion of second tier spatial neighbor’s intra mode in the MPM list after the “above left” neighbors Intra mode.

- DC mode is included after the second tier neighbor’s intra modes are included in the MPM list.

- DIA mode is replaced with VDIA mode in the list of default intra modes which are used to complete the MPM list.

(2) Selected and non-selected list modification:

Selected modes and first few non-selected modes are filled by adding offsets of value “1” to the first two angular modes from the MPM list.

The suggested modifications results in a luma BD-Rate gain of -0.25% and -0.11% on top of BMS for AI and RA configurations respectively.  The suggested modifications are proposed to be adopted in BMS.

# Reference

[1] „Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario“, A. Alshin et. al.,  doc. JVET-J0024, April2018.

[2] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-J1023, April 2018

[4] “Non-EE1: Priority List Based Intra Mode Coding with 5 MPM”, Y.Yu et al, doc. JVET-H0051, October 2017

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).