| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0367-v1 |
| --- | --- |

| Title: | Non-CE4: BMS affine improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, affine prediction in BMS10 is improved by 4 aspects: 1) bit-exact SIMD implementation for affine ME; 2) optimize for adaptive sub-block in affine MC; 3) division free for affine merge candidate derivation; 4) modify the condition check for affine merge mode.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM anchor, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor. For other aspects, experimental results reportedly show on average x%/x% luma coding gain in RA/LB configurations over VTM anchor.

Introduction

Affine prediction in BMS is improved by following aspects:

- bit-exact SIMD implementation for affine ME

- optimize for adaptive sub-block in affine MC

In BMS1.0, in order to simplify the motion compensation prediction, sub-block based affine transform prediction is applied. The sub-block size M×N is derived as in Equation 1, where MvPre is the motion vector fraction accuracy (1/16), (v2x, v2y) is motion vector of the bottom-left control point, calculated according to 4-parameter motion model. After derived by Equation 1, M and N should be adjusted downward if necessary to make it a divisor of w and h, respectively [1]. The minimum sub-block size of affine block is 4x4 in BMS1.0.

M=clip3(4,w,w×MvPremaxabsv1x-v0x,absv1y-v0y)N=clip3(4,h,h×MvPremaxabsv2x-v0x,absv2y-v0y)	            (1)

In this contribution, a SPS syntax is added to signal the minimum sub-block size for affine block. Meanwhile, a bug in affine sub-block size calculation is detected and fixed.

- division free for affine merge candidate derivation

- modify the condition check for affine merge mode

In BMS affine, the control point motion vectors (CPMV) are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. When the affine block size is 4 in width, the top-left CPMV will be rewrite by the top-right CPMV, when the affine block size is 4 in height, the top-left CPMV will be rewrite by the bottom-left CPMV. To avoid such situation, the condition check for affine merge mode is modified from “W*H >= 64” to “W >= 8 && H >=8”.

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test a | Affine SIMD implementation |
| Test b | optimize for adaptive sub-block in affine MC |
| Test c | Aspects 2) to 4) |

Simulation results

The contribution has been implemented on top of BMS1.0 [2]. The test condition descripted in JVET-J1024 [3] is used for verifying the contribution. The VTM anchor is BMS1.0 configured to VTM with only affine and high precision mv (HPMV) on, and the BMS anchor is BMS1.0 itself. The results are summarized in Table 2. More detailed results are provided in from Table 3 to Table 5.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM anchor, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor.

Table 2 Summary of overall performance in RA/LB configuration over VTM anchor and BMS anchor

| Test a | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.00% | 0.00% | 0.00% | 91% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| LB | 0.00% | 0.00% | 0.00% | 87% | 99% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Test b | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.00% | 0.02% | 0.02% | 99% | 99% | 0.02% | 0.01% | -0.02% | 102% | 100% |
| LB | 0.03% | 0.15% | 0.07% | 99% | 97% | -0.07% | -0.06% | -0.03% | 100% | 99% |
| Test c | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA |  |  |  |  |  |  |  |  |  |  |
| LB |  |  |  |  |  |  |  |  |  |  |

Test a

Table 3 Coding performance of test a

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 91% | 99% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 90% | 100% | 0.00% | 0.00% | 0.00% | 96% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 90% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 92% | 96% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 91% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 92% | 93% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 87% | 101% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 90% | 99% | 0.00% | 0.00% | 0.00% | 95% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 83% | 98% | 0.00% | 0.00% | 0.00% | 90% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 87% | 99% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 91% | 96% | 0.00% | 0.00% | 0.00% | 95% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 4 Coding performance of test b

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 100% | 99% | 0.02% | -0.03% | 0.01% | 102% | 100% |
| Class A2 | 0.00% | 0.00% | -0.01% | 99% | 98% | 0.02% | -0.07% | -0.03% | 102% | 100% |
| Class B | 0.01% | -0.01% | 0.03% | 99% | 98% | 0.00% | 0.01% | 0.01% | 102% | 100% |
| Class C | 0.00% | 0.04% | 0.03% | 99% | 99% | 0.03% | 0.08% | -0.06% | 101% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% | 99% | 99% | 0.02% | 0.01% | -0.02% | 102% | 100% |
| Class D | 0.00% | -0.02% | -0.04% | 99% | 98% | 0.01% | 0.09% | 0.05% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.18% | 0.16% | 99% | 98% | 0.00% | -0.25% | 0.10% | 101% | 99% |
| Class C | 0.01% | 0.26% | -0.09% | 99% | 97% | -0.01% | 0.10% | 0.01% | 101% | 100% |
| Class E | 0.07% | 0.56% | 0.12% | 98% | 97% | -0.30% | 0.03% | -0.32% | 100% | 99% |
| Overall | 0.03% | 0.15% | 0.07% | 99% | 97% | -0.07% | -0.06% | -0.03% | 100% | 99% |
| Class D | 0.04% | 0.06% | 0.04% | 99% | 97% | 0.06% | 0.33% | 0.20% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test c

Table 5 Coding performance of test c

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |  |  |  |  |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

Conclusions

In this contribution, affine prediction in BMS10 is improved by 4 aspects: 1) bit-exact SIMD implementation for affine ME; 2) optimize for adaptive sub-block in affine MC; 3) division free for affine merge candidate derivation; 4) modify the condition check for affine merge mode.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM anchor, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor. For other aspects, experimental results reportedly show on average x%/x% luma coding gain in RA/LB configurations over VTM anchor.

We propose to adopt these improvements into the test model.

References

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).