| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0367-v2 |
| --- | --- |

| Title: | Non-CE4: BMS affine improvements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Huanbang Chen Haitao Yang Jianle Chen | Tel: Email: | +8615017554651 chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, affine prediction in BMS1.0 is improved by 4 aspects: 1) bit-exact SIMD implementation for affine ME; 2) optimize for adaptive sub-block in affine MC; 3) division free for affine merge candidate derivation; 4) modify the condition check for affine merge mode.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM with affine on, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor. For other aspects, experimental results reportedly show on average 0.00%/0.01% luma BD-rate change with 1%/1% encoding time reduction and 2%/2% decoding time reduction in RA/LB configurations over VTM with affine on.

Introduction

Affine prediction in BMS is improved by following aspects:

- bit-exact SIMD implementation for affine ME

For affine ME, the derivations of sobel gradient matrixes and the solving of gradient matrix function are implemented with SSE42 SIMD.

- optimize for adaptive sub-block in affine MC

In BMS1.0, in order to simplify the motion compensation prediction, sub-block based affine transform prediction is applied. The sub-block size M×N is derived as in Equation 1, where MvPre is the motion vector fraction accuracy (1/16), (v2x, v2y) is motion vector of the bottom-left control point, calculated according to 4-parameter motion model. After derived by Equation 1, M and N should be adjusted downward if necessary to make it a divisor of w and h, respectively [1]. The minimum sub-block size of affine block is 4x4 in BMS1.0.

M=clip3(4,w,w×MvPremaxabsv1x-v0x,absv1y-v0y)N=clip3(4,h,h×MvPremaxabsv2x-v0x,absv2y-v0y)	            (1)

The principle of adaptive sub-block size is to generate different motion vectors between two neighbor sub-blocks. Assumes that the width of sub-block is M, then M is the minimum value which results in abs(v1x-v0x)w*M=MvPre or abs(v1y-v0y)w*M=MvPre. In BMS-1.0, the parameter MvPre should be set to 1 because the storage motion vector precision is 1/16, same as the motion vector precision for MC. However, it is set to 1/4, which leads to small sub-block size but without any additional gain.

In this contribution, MvPre is fixed to 1 to align with the principle. And the complicated calculation of formula (1) is refactored with division free operations. Meanwhile, a SPS syntax is added to signal the minimum sub-block size for affine block to control the worst case of affine MC.

- division free for affine merge candidate derivation

Floating point operations and divisions are replaced by fixed point and shits in this stage.

- modify the condition check for affine merge mode

In BMS affine, the control point motion vectors (CPMV) are stored in the top-left, top-right and bottom-left 4x4 sub-blocks in the considered CU. When the affine block size is 4 in width, the top-left CPMV will be rewritten by the top-right CPMV, when the affine block size is 4 in height, the top-left CPMV will be rewritten by the bottom-left CPMV. To avoid such situation, the condition check for affine merge mode is modified from “W*H >= 64” to “W >= 8 && H >=8”.

Test specification

The tests for this contribution are descripted in Table 1.

Table 1 test specification for this contribution

| Test # | Test description |
| --- | --- |
| Test a | Affine SIMD implementation |
| Test b | optimize for adaptive sub-block in affine MC |
| Test c | Aspects 2) to 4) |

Simulation results

The contribution has been implemented on top of BMS1.0 [2]. The test condition descripted in JVET-J1024 [3] is used for verifying the contribution. The VTM anchor is BMS1.0 configured to VTM with only affine and high precision mv (HPMV) on, and the BMS anchor is BMS1.0 itself. The results are summarized in Table 2. More detailed results are provided in from Table 3 to Table 6.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM with affine on, and 3%/7% encoding time reduction in RA/LB configurations over BMS.

For other aspects, experimental results reportedly show on average 0.00%/0.01% luma BD-rate change with 1%/1% encoding time reduction and 2%/2% decoding time reduction in RA/LB configurations over VTM with affine on.

In Table 3, the statistic results for affine sub-block size are provided. The area of different affine sub-block sizes is counted, and then divided by the total area of affine block, as shown in formula (2), where M means the size of sub-block (calculated by width * height). The data is obtained by decoding the RA, VTM configuration bit-streams of test b. From Table 3, it is reportedly shown on average 35% percent of affine blocks are split into a size larger than 4x4.

R= total area of affine block with subblock size Mtotal area of affine blocks*100%	            (2)

Table 2 Summary of overall performance in RA/LB configuration over VTM anchor and BMS anchor

| Test a | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.00% | 0.00% | 0.00% | 91% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| LB | 0.00% | 0.00% | 0.00% | 87% | 99% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Test b | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.00% | 0.02% | 0.02% | 99% | 98% | 0.02% | 0.01% | -0.02% | 101% | 100% |
| LB | 0.03% | 0.15% | 0.07% | 99% | 99% | -0.07% | -0.06% | -0.03% | 100% | 99% |
| Test c | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
| Overall | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| RA | 0.00% | -0.01% | 0.03% | 99% | 98% | 0.02% | -0.01% | -0.03% | 101% | 100% |
| LB | 0.01% | 0.05% | 0.21% | 99% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 3 Statistic results of sub-block size for affine

Test a

Table 3 Coding performance of test a

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 91% | 99% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 90% | 100% | 0.00% | 0.00% | 0.00% | 96% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 90% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 92% | 96% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 91% | 98% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 92% | 93% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 87% | 101% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 90% | 99% | 0.00% | 0.00% | 0.00% | 95% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 83% | 98% | 0.00% | 0.00% | 0.00% | 90% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 87% | 99% | 0.00% | 0.00% | 0.00% | 93% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 91% | 96% | 0.00% | 0.00% | 0.00% | 95% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test b

Table 5 Coding performance of test b

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.05% | 0.04% | 99% | 100% | 0.02% | -0.03% | 0.01% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | -0.01% | 99% | 99% | 0.02% | -0.07% | -0.03% | 101% | 99% |
| Class B | 0.01% | -0.01% | 0.03% | 99% | 98% | 0.00% | 0.01% | 0.01% | 101% | 99% |
| Class C | 0.00% | 0.04% | 0.03% | 99% | 97% | 0.03% | 0.08% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.02% | 99% | 98% | 0.02% | 0.01% | -0.02% | 101% | 100% |
| Class D | 0.00% | -0.02% | -0.04% | 99% | 96% | 0.01% | 0.09% | 0.05% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.03% | -0.18% | 0.16% | 99% | 99% | 0.00% | -0.25% | 0.10% | 100% | 98% |
| Class C | 0.01% | 0.26% | -0.09% | 99% | 99% | -0.01% | 0.10% | 0.01% | 100% | 99% |
| Class E | 0.07% | 0.56% | 0.12% | 98% | 99% | -0.30% | 0.03% | -0.32% | 99% | 99% |
| Overall | 0.03% | 0.15% | 0.07% | 99% | 99% | -0.07% | -0.06% | -0.03% | 100% | 99% |
| Class D | 0.04% | 0.06% | 0.04% | 99% | 98% | 0.06% | 0.33% | 0.20% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Test c

Table 6 Coding performance of test c

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.02% | -0.04% | 100% | 100% | 0.03% | -0.12% | -0.07% | 101% | 100% |
| Class A2 | 0.02% | -0.02% | 0.02% | 100% | 99% | 0.02% | 0.03% | 0.04% | 101% | 99% |
| Class B | 0.00% | -0.03% | 0.06% | 99% | 98% | 0.00% | -0.03% | -0.03% | 101% | 99% |
| Class C | -0.04% | 0.03% | 0.04% | 99% | 95% | 0.03% | 0.07% | -0.06% | 100% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.03% | 99% | 98% | 0.02% | -0.01% | -0.03% | 101% | 100% |
| Class D | 0.01% | -0.12% | -0.13% | 98% | 93% | 0.03% | 0.15% | 0.03% | 100% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM + AFFINE |  |  |  |  | Over BMS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.28% | 0.15% | 99% | 99% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | 0.25% | 0.19% | 99% | 98% | -0.05% | -0.04% | -0.04% | 100% | 99% |
| Class E | 0.01% | 0.32% | 0.32% | 99% | 98% | -0.17% | 0.07% | 0.06% | 100% | 99% |
| Overall | 0.01% | 0.05% | 0.21% | 99% | 98% | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | -0.03% | -0.07% | 99% | 95% | 0.03% | 0.63% | -0.13% | 99% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this contribution, affine prediction in BMS10 is improved by 4 aspects: 1) bit-exact SIMD implementation for affine ME; 2) optimize for adaptive sub-block in affine MC; 3) division free for affine merge candidate derivation; 4) modify the condition check for affine merge mode.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM with affine on, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor. For other aspects, experimental results reportedly show on average 0.00%/0.01% luma BD-rate change with 1%/1% encoding time reduction and 2%/2% decoding time reduction in RA/LB configurations over VTM with affine on.

We propose to adopt these improvements into the test model.

References

- J. Chen, E. Alshina, G. J. Sullivan, J-R. Ohm, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001, July 2017.

- BMS reference software, https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- H. Yang, S. Liu, K. Zhang, “Description of CE4: Inter prediction and motion vector coding”, JVET-J1024, April 2018.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).