| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0368 |
| --- | --- |

| Title: | CE3 3.2.3: Intra mode signaling without non-MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Biao Wang, Zhijie Zhao, Jianle Chen | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This proposal reports the result for intra mode signaling by switching off the macro JVET_B0051_NON_MPM_MODE. JVET_B0051_NON_MPM_MODE proposes to signal the intra mode by dividing the available 67 intra modes into three different lists : MPM list, Selected Modes list, Non-Selected modes list. By disabling JVET_B0051_NON_MPM_MODE, the Intra mode is simply signaled by using an MPM list who size is six and the remaining 61 intra modes are signaled using a fixed length code. Experimental results of the proposed modification is as follows:

Over VTM Anchor (AI, RA): -1.25%, -0.58% Luma coding gain with 114%, 104% encoding time and 102%, 100% decoding time.

Over BMS Anchor (AI, RA): 0.01%, 0.01% Luma coding loss with 100%, 101%, encoding time and 102%, 101% decoding time.

# Introduction

The current proposal mainly checks the effectiveness of the current intra mode coding scheme present in BMS.

# Background

BMS uses a total of 67 intra prediction modes. The intra mode is signaled by dividing the available intra modes into three different lists: MPM modes list, selected modes list and non-selected modes set. The 6 MPM modes are derived from five immediate spatial neighbors as shown in Fig. 1.

Fig. 1 Neighbour intra modes

The order of insertion of intra modes into the MPM list is as follows:  “Left (L)”, “Above (A)”, “Planar”, “DC”, “Below left (BL)”, “above right (AR)”, and “above left (AL)”. The 16 selected modes are chosen from the remaining 61 modes with a fixed interval of value “4”. The rest 45 modes are assigned to the non-selected modes set. The MPM mode is coded with a unary code, the selected mode is signaled with a 4-bit fixed length code, and the non-selected mode is coded with a truncated binary code. The binarization for the syntax element intra prediction mode is specified in Table 1.

| Intra prediction modes | MPM flag | Selected flag | Bin string |
| --- | --- | --- | --- |
| MPM modes (6) | 1 |  | 0 |
|  |  |  | 10 |
|  |  |  | 110 |
|  |  |  | 1110 |
|  |  |  | 11110 |
|  |  |  | 11111 |
| Selected modes (16) | 0 | 1 | 4 bits fixed length code |
| Non-selected modes (45) | 0 | 0 | Truncated binary code |

Table 1. Intra prediction mode coding in BMS

# Proposed Method

This proposal reports the result for intra mode signaling by switching off the macro JVET_B0051_NON_MPM_MODE. JVET_B0051_NON_MPM_MODE proposes to signal the intra mode by dividing the available 67 intra modes into three different lists: MPM list, Selected Modes list, Non-Selected modes list. By disabling JVET_B0051_NON_MPM_MODE, the Intra mode is simply signaled by using an MPM list who size is six and the remaining 61 intra modes are signaled using a fixed length code

# Experimental Results

The table below shows the coding gain and processing time impact of the suggested modifications.

- BMS simulations

  - Anchor: BMS

  - Test: Anchor with Proposal

- VTM simulations

  - Anchor: VTM

  - Test: VTM + Intra65Ang == 1 + Proposal

# Conclusion

The proposal reports the results for intra mode signaling by switching off the Macro JVET_B0051_NON_MPM_MODE.

The suggested modifications results in a luma BD-Rate loss of 0.01% and 0.01% on top of BMS for AI and RA configurations respectively.

# Reference

[1] „Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario“, A. Alshin et. al.,  doc. JVET-J0024, April2018.

[2] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-J1023, April 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. has no pending patent rights relating to the technology described in this contribution