| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0368-r1 |
| --- | --- |

| Title: | CE3 3.2.3: Intra mode signaling without non-MPM list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Biao Wang, Zhijie Zhao, Jianle Chen | Tel: Email: | anand.meher.kotra@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This proposal reports the result for intra mode signaling by switching off the macro JVET_B0051_NON_MPM_MODE. JVET_B0051_NON_MPM_MODE proposes to signal the intra mode by dividing the available 67 intra modes into three different lists : MPM list, Selected Modes list, Non-Selected modes list. By disabling JVET_B0051_NON_MPM_MODE, the Intra mode is simply signaled by using an MPM list who size is six and the remaining 61 intra modes are signaled using a fixed length code. Experimental results of the proposed modification is as follows:

Over VTM Anchor (AI, RA): -1.25%, -0.58% Luma coding gain with 114%, 104% encoding time and 102%, 100% decoding time.

Over BMS Anchor (AI, RA): 0.01%, 0.01% Luma coding loss with 100%, 101%, encoding time and 102%, 101% decoding time.

# Introduction

The current proposal mainly checks the effectiveness of the current intra mode coding scheme present in BMS.

# Background

BMS uses a total of 67 intra prediction modes. The intra mode is signaled by dividing the available intra modes into three different lists: MPM modes list, selected modes list and non-selected modes set. The 6 MPM modes are derived from five immediate spatial neighbors as shown in Fig. 1.

Fig. 1 Neighbour intra modes

The order of insertion of intra modes into the MPM list is as follows:  “Left (L)”, “Above (A)”, “Planar”, “DC”, “Below left (BL)”, “above right (AR)”, and “above left (AL)”. The 16 selected modes are chosen from the remaining 61 modes with a fixed interval of value “4”. The rest 45 modes are assigned to the non-selected modes set. The MPM mode is coded with a unary code, the selected mode is signaled with a 4-bit fixed length code, and the non-selected mode is coded with a truncated binary code. The binarization for the syntax element intra prediction mode is specified in Table 1.

| Intra prediction modes | MPM flag | Selected flag | Bin string |
| --- | --- | --- | --- |
| MPM modes (6) | 1 |  | 0 |
|  |  |  | 10 |
|  |  |  | 110 |
|  |  |  | 1110 |
|  |  |  | 11110 |
|  |  |  | 11111 |
| Selected modes (16) | 0 | 1 | 4 bits fixed length code |
| Non-selected modes (45) | 0 | 0 | Truncated binary code |

Table 1. Intra prediction mode coding in BMS

# Proposed Method

This proposal reports the result for intra mode signaling by switching off the macro JVET_B0051_NON_MPM_MODE. JVET_B0051_NON_MPM_MODE proposes to signal the intra mode by dividing the available 67 intra modes into three different lists: MPM list, Selected Modes list, Non-Selected modes list. By disabling JVET_B0051_NON_MPM_MODE, the Intra mode is simply signaled by using an MPM list who size is six and the remaining 61 intra modes are signaled using a fixed length code

# Experimental Results

The table below shows the coding gain and processing time impact of the suggested modifications.

- BMS simulations

  - Anchor: BMS

  - Test: Anchor with Proposal

- VTM simulations

  - Anchor: VTM

  - Test: VTM + Intra65Ang == 1 + Proposal

# Conclusion

The proposal reports the results for intra mode signaling by switching off the Macro JVET_B0051_NON_MPM_MODE.

The suggested modifications results in a luma BD-Rate loss of 0.01% and 0.01% on top of BMS for AI and RA configurations respectively.

# Reference

[1] „Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario“, A. Alshin et. al.,  doc. JVET-J0024, April2018.

[2] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-J1023, April 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).