| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0369-r1 |
| --- | --- |

| Title: | CE2-related : Longer Tap Deblocking Filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anand Meher Kotra, Biao Wang, Semih Esenlik, Zhijie Zhao, Jianle Chen | Tel: Email: | anand.meher.kotra @huawei.com |
| Source: | Huawei Technologies Co., Ltd. |  |  |

_____________________________

# Abstract

A new longer tap deblocking filter for the luma component is proposed. The proposed filter mainly targets the filtering of blocking artifacts which arise due to the usage of larger size transform units and coding units. The “longer tap” filter introduces new filter condition checks which consider a wider range of spatial activity along the edges. Furthermore, new longer tap filter coefficients are proposed in order to effectively smooth the edges with blocking artifacts belonging to larger blocks.

Moreover, to reduce the line buffer requirements for the “longer tap” filter: For the horizontal edges which overlap with the CTU boundaries, the maximum number of samples used in filter decision and the maximum number of samples used in filter modification from the top block are restricted to be the same as in HEVC deblocking filter. Compared to the deblocking filter used in BMS, the proposed method improves the subjective quality of sequences, especially for the ones which are encoded at lower bitrates. Objective results of the proposed longer tap deblocking filer are as follows:

Over VTM Anchor (AI, RA, LDB): Luma BD-Rate gain of -0.12%, -0.13%, -0.02% is achieved without any increase in EncT and DecT.

Over BMS Anchor (AI, RA, LDB): Luma BD-Rate gain of -0.08%, -0.01%, 0.04% is achieved without any increase in EncT and DecT.

# Introduction

Compared to HEVC, larger transform blocks and coding units are used in VVC. The usage of large block size might introduce significant blocking artifact, particularly in sequences coded at low bitrate. The existing deblocking filter in BMS modifies up to 3 samples perpendicular to edges, resulting in insufficient smoothing for the edges with blocking artifacts. Furthermore, the BMS deblocking filter only consider edges on CU boundaries but not the internal boundaries caused by implicit partition because of constraints in transform block size. Figure 1 shows an area with obvious blocking artifact captured from POC 10 of FoodMarket sequence (encoded at QP 37 using BMS software)

The proposed longer tap filter addresses the necessity of deblocking edges that need more samples to be modified in order to remove the blocking artifact. Meanwhile, it also considers filtering of the implicit TU boundaries within CUs. Furthermore, to ease hardware implementation, the proposed filter restricts the filtering operations for horizontal edges overlapping with CTU boundaries.

Figure 1: Blocking artifacts captured from POC 10 of FoodMarket sequence encoded at QP 37 using BMS software

# Proposed Method

The proposed method considers three aspects. The first aspect defines new “longer tap” filter conditions which determine in which cases the “longer tap” filter should be applied. The second aspect relates to the  new “longer tap” filter coefficients used when the “longer tap” filter conditions are evaluated to true. The “longer tap” filter uses up to eight samples on either side of the edge and modifies a maximum of seven samples on either side of the edge. The third aspect relates to the restriction of number of samples used and modified from the top CTU when a given horizontal edge overlaps with the CTU boundaries.

2.1 New Conditions to determine longer tap filter

Figure 2 illustrates the flow chart which is used to determine if the longer tap filter has to be applied or not. As a pre-requisite, the HEVC strong/weak condition should first be evaluated to true. Additionally two more conditions are also evaluated:

Condition 1:

- for vertical edges, the current  block width and neighboring block width are both >= 16 samples or

- for horizontal edges, the current block height and neighboring block height are both >= 16 samples

Condition 2:

If the condition 1 is valid, then the following condition as shown in Equation 1 is further checked:

p0-p3- p4+p7+q0-q3- q4+q7<3*β32 &&

p0-p2-p3+p5+q0-q2-q3+q5<3*β32

(Equation 1)

where the samples of p0 to p7 and q0 to q7 of an edge are as shown in Figure 3:

Figure 2: Flow chart to determine the new longer tap filter.

q0  q1 q2  q3 q4  q5 q6  q7

p7  p6 p5  p4 p3  p2 p1  p0

Figure 3: P and Q side one row of samples across a vertical edge

As in conventional HEVC strong/weak checking, the condition proposed in Equation 1 is applied for both the first and fourth line of an edge, as shown in Figure 4. Only when the checking of Equation 1 in both the first and fourth lines are valid, then the condition of Equation 1 is considered as valid.

Figure 4: The samples in first and fourth rows (in boldface) used to decide strong/weak filter in HEVC

The proposed new conditions considers the spatial activity across an edge with a wider range, i.e., from p0-p7 (or q0-q7 symmetrically) rather than p0-p3 (or q0-q3 symmetrically) in HEVC, in order to avoid duplicated checking.

2.2 Proposed coefficients of longer tap filter

The proposed filter coefficients are as shown in the table below. The center coefficient is marked in Bold.

| Output pixel | Filter coefficients {p7, p6, p5, p4, p3, p2, p1, p0, q0, q1, q2, q3, q4, q5, q6, q7} | Input pixels |
| --- | --- | --- |
| p6 | {6, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0, 0} | p7 ~ q0 |
| p5 | {5, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0, 0} | p7 ~ q1 |
| p4 | {4, 1, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0, 0} | p7 ~ q2 |
| p3 | {3, 1, 1, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0, 0} | p7 ~ q3 |
| p2 | {2, 1, 1, 1, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0} | p7 ~ q4 |
| p1 | {1, 1, 1, 1, 1, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0} | p7 ~ q5 |
| p0 | {1, 1, 1, 1, 1, 1, 1, 2, 1, 1, 1, 1, 1, 1, 1, 0} | p7 ~ q6 |
| q0 | {0, 1, 1, 1, 1, 1, 1, 1, 2, 1, 1, 1, 1, 1, 1, 1} | p6 ~ q7 |
| q1 | {0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 1, 1, 1, 1} | p5 ~ q7 |
| q2 | {0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 1, 1, 2} | p4 ~ q7 |
| q3 | {0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 1, 3} | p3 ~ q7 |
| q4 | {0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 4} | p2 ~ q7 |
| q5 | {0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 5} | p1 ~ q7 |
| q6 | {0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 6} | p0 ~ q7 |

Table 1 New longer tap filter coefficients

The input samples’ scope is restricted between p7 to q7 (8 samples on each side) in order to avoid overlapping with neighboring blocks’ filtering. Since the width (or height) of both sides for vertical (or horizontal) edges of qualified blocks is at least 16 samples, the input/output samples of each filter operation can be independently processed, enabling parallel filtering for the new longer tap filter.

2.3 Restriction of longer tap filter for horizontal edges which overlap with CTU boundary.

For the design of deblocking filer, line buffer size needs to be considered. The line buffer is used to buffer (N) bottom lines of samples of the neighboring top CTU. Those samples are buffered because the deblocking filtering operation needs to be delayed until (N) top lines of current CTU’s samples are reconstructed. Under the design of above new longer tap deblocking filter, it requires 8 sample lines for the line buffer, which is considered too heavy for hardware implementation.

The line buffer size is restricted by modifying the condition in Equation 1. If current horizontal edge overlaps with the CTU boundary, then the following condition in Equation 2 is evaluated instead of Equation 1.

p3-p2-p1+p0+q0-q3- q4+q7<3*β32 &&

p3-p2-p1+p0+q0-q2-q3+q5<3*β32

(Equation 2)

As seen from the equation, on P side (top block), only samples from p0 to p3 are used whereas on the Q side (bottom block), all samples q0 to q7 are used as these samples belong to the current CTU.

The filter coefficients which are used for the horizontal edges which overlap with the CTU boundaries is shown in Table 2 below.

| Output pixel | Filter coefficients {p7, p6, p5, p4, p3, p2, p1, p0, q0, q1, q2, q3, q4, q5, q6, q7} | Input pixels |
| --- | --- | --- |
| p2 | {0, 0, 0, 0, 6, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0, 0} | p3 ~ q4 |
| p1 | {0, 0, 0, 0, 5, 1, 3, 1, 1, 1, 1, 1, 1, 1, 0, 0} | p3 ~ q5 |
| p0 | {0, 0, 0, 0, 5, 1, 1, 2, 1, 1, 1, 1, 1, 1, 1, 0} | p3 ~ q6 |
| q0 | {0, 0, 0, 0, 4, 1, 1, 1, 2, 1, 1, 1, 1, 1, 1, 1} | p3 ~ q7 |
| q1 | {0, 0, 0, 0, 3, 1, 1, 1, 1, 3, 1, 1, 1, 1, 1, 1} | p3 ~ q7 |
| q2 | {0, 0, 0, 0, 2, 1, 1, 1, 1, 1, 3, 1, 1, 1, 1, 2} | p3 ~ q7 |
| q3 | {0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 1, 3} | p3 ~ q7 |
| q4 | {0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 1, 4} | p2 ~ q7 |
| q5 | {0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 1, 5} | p1 ~ q7 |
| q6 | {0, 0, 0, 0, 0, 0, 0, 1, 1, 1, 1, 1, 1, 1, 3, 6} | p0 ~ q7 |

Table 2: Longer tap filter coefficients for horizontal edges overlapping with CTU borders

# Experimental Results

Figure 5 to 6 show the subjective improvements of the proposed longer tap deblocking filter. Each figure consists of three sub-figures:  a), b), and c). The result presented in sub-figure a) is obtained from BMS 1.0; the result presented in sub-figure b) is obtained from BMS 1.0 with filtering implicit TU boundaries in large CUs; and the result presented in sub-figure c) is obtained from the proposed longer tap filter. As it can been seen, the proposed approach improves subjective quality significantly.

a): BMS 1.0                                                     b): BMS+ filtering implicit TU boundaries

c): proposed approach

Figure 5: subjective quality improvement with the proposed approach, POC 10 of foodMarket QP 37

a): BMS 1.0                                                 b): BMS+ filtering implicit TU boundaries

c): proposed approach

Figure 6: subjective quality improvement with the proposed approach, POC 3 of foodMarket QP 37

## Additional Objective Results

Table 3 shows the coding gain and processing time impact of the proposed longer tap filter with line buffer reduction design.

- VTM simulations

  - Anchor: VTM

  - Test: Anchor with Proposal

- BMS simulations

  - Anchor: BMS

  - Test: Anchor with Proposal

Table 3: objective results of the proposed longer tap filter with line buffer reduction design

Table 4 shows the coding gain and processing time impact of the proposed longer tap filter with line buffer reduction design over an intermediate bug fix version. The intermediate version is a “bug fix” version of  BMS 1.0 which considers the implicit TU boundaries caused by the difference of the largest TU size (64x64 samples) and the largest CU size (128x128 samples). Furthermore, the intermediate version also allows parallel processing that restrict the number of filter samples in order to apply filter independently for each edge segment. The proposed approach is built on top of this intermediate version with two additional features: the first are the new filter condition and coefficients, and the second is the line buffer restriction.

- VTM simulations

  - Anchor: VTM + filtering implicit TU boundaries + parallel filter (intermediate version)

  - Test: Anchor with Proposal

- BMS simulations

  - Anchor: BMS + filtering implicit TU boundaries + parallel filter (intermediate version)

  - Test: Anchor with Proposal

Table 4: objective results of the proposed longer tap filter with line buffer reduction design over the intermediate anchor with intermediate TU boundaries filtered

# Conclusion

A longer tap deblocking filter with line buffer reduction design is proposed. The proposed longer tap filter consists of new filter condition checks and new filter coefficients to effectively reduce the blocking artifacts belonging to larger blocks. To reduce the line buffer requirements: For the horizontal edges which overlap with the CTU boundaries, for the top block, the number of filter samples used in filter condition and filter modification are restricted to the same value as in the deblocking filter of HEVC. Compared to BMS 1.0, significant subjective improvements were observed, especially for sequences encoded with lower bitrates.

Objectively the proposed method brings a BD-Rate gain of -0.08% in AI condition and -0.01% in RA condition on top of BMS. The proposed method is suggested to be studied in a core experiment (CE) along with the other “longer tap” filter designs proposed in the JVET-J (San Diego, April 2018) meeting.

# Reference

[1] “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, A. Alshin et. al.,  doc. JVET-J0024, April, 2018.

[2] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[3] “Description of Core Experiment 2 (CE2):  In-Loop Filters”, L. Zhang, K. Andersson, C.-Y. Chen, doc. JVET-J1022, April 2018

# Patent rights declaration(s)

Huawei Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).