| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0371-v1 |
| --- | --- |

| Title: | CE2.4.1.4: Reduced filter shape size for ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz Nikolay Shlyakhov Nan Hu Vadim Seregin Wei-Jung Chien | Tel: Email: | martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CE2 test 4.1.4 where the largest ALF filter shape is reduced from 9x9 to 7x7, and classifier block size is increased from 2x2 to 4x4. Reduced filter shape results reportedly show 3.10%, 5.10%, and 4.32% luma gain in AI, RA, and LB configurations respectively over VTM-1.0 anchor. Increased classifier block size results in 3.16%, 5.23%, and 4.49% luma gain for the same configurations, and combined both changes show 2.99%, 4.99%, and 4.19% luma gain in AI, RA, and LB configurations respectively over VTM-1.0 anchor.

# Introduction

In the BMS software, the largest ALF filter shape is 9x9 for luma and 5x5 for chroma components, while classifier block size is 2x2.

In the CE2 test 4.1.4 reduced largest filter size to 7x7 and increased filter classifier block size to 4x4 for luma component are tested for the ALF in BMS. In the tests, the total number of ALF fixed filters is reduced to 128 comparing to ALF in BMS, and a mapping table is introduced to map a fixed filter to existed ALF classes. Aligned chroma filter with luma filter size is also tested.

SIMD implementation is provided for ALF classification and filtering and tested in the CE.

# Experimental Results

The discussed changes were implemented into BMS-1.0 software and two anchors are used: BMS software configured in VTM configuration, and BMS configuration with ALF tool disabled. Results are summarized in below tables and can be found in detail in the accompanied Excel tables.

Table 1. Summary of all tests results for VTM configuration (positive numbers is coding gain).

| Test | AI |  |  | RA |  |  | LB |  |  | RA Enc | RA Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. luma 7x7, classifier 2x2, chroma 5x5 | 3.10 | 3.56 | 4.07 | 5.10 | 2.29 | 1.69 | 4.32 | 1.79 | 1.77 | 105% | 135% |
| 2. luma 7x7, classifier 2x2, chroma 7x7 | 3.09 | 4.20 | 4.73 | 5.11 | 3.16 | 2.66 | 4.31 | 2.63 | 2.72 | 104% | 134% |
| 3. luma 7x7, classifier 4x4, chroma 5x5 | 2.99 | 3.56 | 4.07 | 4.99 | 2.24 | 1.66 | 4.19 | 1.64 | 1.85 | 104% | 122% |
| 4. luma 7x7, classifier 4x4, chroma 7x7 | 2.98 | 4.20 | 4.73 | 4.98 | 3.14 | 2.66 | 4.23 | 2.44 | 2.57 | 104% | 122% |
| 5. luma 9x9, classifier 2x2, chroma 5x5 | 3.28 | 3.54 | 4.05 | 5.33 | 2.21 | 1.63 | 4.56 | 1.81 | 2.01 | 108% | 143% |
| 6. luma 9x9, classifier 2x2, chroma 7x7 | 3.27 | 4.19 | 4.71 | 5.35 | 3.11 | 2.57 | 4.63 | 2.61 | 2.88 | 107% | 142% |
| 7. luma 9x9, classifier 4x4, chroma 5x5 | 3.17 | 3.54 | 4.05 | 5.21 | 2.20 | 1.66 | 4.46 | 1.83 | 2.01 | 107% | 129% |
| 8. luma 9x9, classifier 4x4, chroma 7x7 | 3.16 | 4.19 | 4.71 | 5.23 | 3.15 | 2.63 | 4.49 | 2.59 | 2.72 | 108% | 129% |

## ALF luma 7x7, classifier 2x2, chroma 5x5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.28% | -3.32% | -3.50% | 102% | 133% |
| Class A2 | -2.88% | -3.81% | -3.63% | 101% | 124% |
| Class B | -2.67% | -2.32% | -2.99% | 102% | 123% |
| Class C | -3.11% | -2.73% | -3.61% | 101% | 117% |
| Class E | -4.86% | -6.72% | -7.48% | 102% | 129% |
| Overall | -3.10% | -3.56% | -4.07% | 101% | 124% |
| Class D | -2.42% | -2.83% | -3.36% | 101% | 115% |
| Class F (optional) | -1.86% | -1.06% | -1.15% | 102% | 117% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.34% | -2.64% | -1.58% | 104% | 145% |
| Class A2 | -6.23% | -1.46% | -0.33% | 104% | 142% |
| Class B | -5.48% | -3.44% | -2.33% | 106% | 136% |
| Class C | -4.37% | -1.20% | -2.00% | 104% | 123% |
| Class E |  |  |  |  |  |
| Overall | -5.10% | -2.29% | -1.69% | 105% | 135% |
| Class D | -5.12% | -1.96% | -2.34% | 106% | 122% |
| Class F (optional) | -1.97% | -0.90% | -0.99% | 110% | 118% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.15% | -1.67% | -1.35% | 104% | 124% |
| Class C | -3.97% | -1.64% | -1.43% | 103% | 118% |
| Class E | -5.07% | -2.21% | -2.92% | 113% | 113% |
| Overall | -4.32% | -1.79% | -1.77% | 106% | 119% |
| Class D | -3.96% | -1.92% | -1.82% | 104% | 115% |
| Class F (optional) | -3.66% | -2.05% | -1.30% | 112% | 118% |

## ALF luma 7x7, classifier 2x2, chroma 7x7

| All Intra Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -2.28%; -3.56%; -3.77%; 102%; 131%. Class A2; -2.87%; -4.06%; -4.09%; 101%; 121%. Class B; -2.66%; -2.79%; -3.37%; 101%; 122%. Class C; -3.09%; -3.33%; -4.42%; 101%; 118%. Class E; -4.83%; -8.52%; -9.01%; 101%; 129%. Overall; -3.09%; -4.20%; -4.73%; 101%; 123%. Class D; -2.37%; -3.21%; -3.89%; 100%; 115%. Class F (optional); -1.85%; -1.23%; -1.50%; 102%; 117%. Random Access Main 10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -4.34%; -3.26%; -2.01%; 104%; 144%. Class A2; -6.25%; -2.21%; -1.61%; 104%; 141%. Class B; -5.47%; -4.45%; -3.11%; 106%; 135%. Class C; -4.37%; -2.18%; -3.38%; 103%; 122%. Class E. Overall; -5.11%; -3.16%; -2.66%; 104%; 134%. Class D; -5.10%; -2.60%; -3.16%; 105%; 119%. Class F (optional); -1.95%; -1.07%; -1.24%; 110%; 117%. Low delay B Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.17%; -3.30%; -2.38%; 105%; 123%. Class C; -4.02%; -2.80%; -3.24%; 104%; 122%. Class E; -4.93%; -1.28%; -2.58%; 114%; 118%. Overall; -4.31%; -2.63%; -2.72%; 107%; 122%. Class D; -3.97%; -3.84%; -3.88%; 104%; 112%. Class F (optional); -3.69%; -2.12%; -2.39%; 111%; 114%. | All Intra Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -2.03%; -4.10%; -3.00%; 100%; 117%. Class A2; -2.64%; -3.57%; -3.53%; 101%; 115%. Class B; -2.17%; -2.86%; -3.16%; 101%; 117%. Class C; -2.39%; -3.15%; -4.02%; 101%; 112%. Class E; -3.83%; -8.01%; -8.36%; 101%; 121%. Overall; -2.55%; -4.11%; -4.25%; 101%; 116%. Class D; -1.68%; -2.99%; -3.56%; 100%; 112%. Class F (optional); -1.53%; -1.51%; -1.55%; 101%; 118%. Random Access Main 10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -4.04%; -3.37%; -0.91%; 100%; 129%. Class A2; -6.22%; -1.87%; -1.92%; 101%; 126%. Class B; -5.10%; -4.59%; -2.99%; 102%; 121%. Class C; -4.02%; -2.39%; -3.26%; 101%; 115%. Class E. Overall; -4.83%; -3.22%; -2.43%; 101%; 122%. Class D; -4.91%; -2.78%; -3.22%; 101%; 113%. Class F (optional); -1.77%; -1.40%; -1.46%; 102%; 114%. Low delay B Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -3.99%; -3.04%; -2.34%; 99%; 121%. Class C; -3.93%; -3.22%; -3.45%; 100%; 113%. Class E; -4.25%; -0.18%; -0.75%; 104%; 119%. Overall; -4.04%; -2.39%; -2.31%; 101%; 118%. Class D; -4.07%; -3.32%; -3.42%; 100%; 112%. Class F (optional); -3.39%; -2.29%; -2.13%; 104%; 123%. |
| --- | --- |

## ALF luma 7x7, classifier 4x4, chroma 5x5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.30% | -3.32% | -3.50% | 101% | 118% |
| Class A2 | -2.82% | -3.81% | -3.64% | 100% | 114% |
| Class B | -2.59% | -2.32% | -2.99% | 101% | 115% |
| Class C | -2.86% | -2.73% | -3.61% | 100% | 110% |
| Class E | -4.68% | -6.71% | -7.48% | 101% | 114% |
| Overall | -2.99% | -3.56% | -4.07% | 101% | 114% |
| Class D | -2.14% | -2.82% | -3.35% | 101% | 109% |
| Class F (optional) | -1.68% | -1.07% | -1.16% | 101% | 110% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.43% | -2.71% | -1.50% | 104% | 127% |
| Class A2 | -6.15% | -1.42% | -0.37% | 105% | 127% |
| Class B | -5.35% | -3.45% | -2.32% | 105% | 124% |
| Class C | -4.10% | -0.97% | -1.94% | 104% | 112% |
| Class E |  |  |  |  |  |
| Overall | -4.99% | -2.24% | -1.66% | 104% | 122% |
| Class D | -4.86% | -1.95% | -2.38% | 106% | 111% |
| Class F (optional) | -1.80% | -0.87% | -0.96% | 109% | 111% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.07% | -1.74% | -1.31% | 105% | 118% |
| Class C | -3.85% | -1.49% | -1.51% | 104% | 111% |
| Class E | -4.83% | -1.65% | -3.21% | 114% | 112% |
| Overall | -4.19% | -1.64% | -1.85% | 107% | 114% |
| Class D | -3.77% | -1.86% | -1.44% | 104% | 108% |
| Class F (optional) | -3.60% | -2.00% | -0.86% | 112% | 113% |

## ALF luma 7x7, classifier 4x4, chroma 7x7

| All Intra Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -2.30%; -3.56%; -3.77%; 102%; 122%. Class A2; -2.82%; -4.06%; -4.09%; 102%; 112%. Class B; -2.58%; -2.79%; -3.37%; 102%; 115%. Class C; -2.84%; -3.33%; -4.42%; 102%; 110%. Class E; -4.65%; -8.51%; -9.00%; 102%; 115%. Overall; -2.98%; -4.20%; -4.73%; 102%; 114%. Class D; -2.09%; -3.20%; -3.88%; 101%; 108%. Class F (optional); -1.67%; -1.24%; -1.51%; 102%; 110%. Random Access Main 10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -4.41%; -3.22%; -2.06%; 103%; 129%. Class A2; -6.16%; -2.28%; -1.62%; 104%; 127%. Class B; -5.32%; -4.47%; -3.08%; 106%; 123%. Class C; -4.12%; -2.08%; -3.36%; 103%; 111%. Class E. Overall; -4.98%; -3.14%; -2.66%; 104%; 122%. Class D; -4.84%; -2.70%; -3.24%; 105%; 108%. Class F (optional); -1.81%; -1.04%; -1.30%; 110%; 111%. Low delay B Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.09%; -3.09%; -2.41%; 104%; 117%. Class C; -3.89%; -2.84%; -3.09%; 104%; 116%. Class E; -4.93%; -0.81%; -2.16%; 114%; 112%. Overall; -4.23%; -2.44%; -2.57%; 107%; 116%. Class D; -3.85%; -3.56%; -3.68%; 105%; 110%. Class F (optional); -3.61%; -2.27%; -2.32%; 112%; 112%. | All Intra Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -2.07%; -4.10%; -3.00%; 101%; 113%. Class A2; -2.60%; -3.57%; -3.53%; 101%; 110%. Class B; -2.12%; -2.86%; -3.16%; 101%; 111%. Class C; -2.19%; -3.15%; -4.02%; 100%; 107%. Class E; -3.69%; -8.00%; -8.35%; 100%; 114%. Overall; -2.47%; -4.11%; -4.25%; 101%; 111%. Class D; -1.47%; -2.99%; -3.56%; 100%; 106%. Class F (optional); -1.38%; -1.52%; -1.56%; 100%; 111%. Random Access Main 10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -4.14%; -3.39%; -0.95%; 100%; 117%. Class A2; -6.11%; -1.79%; -1.89%; 101%; 117%. Class B; -4.92%; -4.69%; -3.02%; 101%; 114%. Class C; -3.82%; -2.31%; -3.15%; 101%; 108%. Class E. Overall; -4.71%; -3.22%; -2.42%; 101%; 114%. Class D; -4.71%; -2.70%; -3.10%; 101%; 107%. Class F (optional); -1.60%; -1.28%; -1.40%; 102%; 108%. Low delay B Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -3.93%; -3.07%; -2.62%; 99%; 114%. Class C; -3.82%; -3.27%; -3.45%; 100%; 109%. Class E; -4.26%; 0.24%; -1.22%; 103%; 108%. Overall; -3.98%; -2.31%; -2.55%; 101%; 111%. Class D; -3.99%; -3.37%; -3.62%; 100%; 107%. Class F (optional); -3.28%; -2.40%; -2.52%; 103%; 112%. |
| --- | --- |

## ALF luma 9x9, classifier 2x2, chroma 5x5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.45% | -3.30% | -3.48% | 103% | 138% |
| Class A2 | -3.05% | -3.79% | -3.62% | 102% | 131% |
| Class B | -2.79% | -2.29% | -2.96% | 103% | 130% |
| Class C | -3.31% | -2.71% | -3.60% | 102% | 121% |
| Class E | -5.11% | -6.71% | -7.47% | 103% | 132% |
| Overall | -3.28% | -3.54% | -4.05% | 103% | 130% |
| Class D | -2.64% | -2.82% | -3.35% | 103% | 118% |
| Class F (optional) | -1.88% | -1.03% | -1.13% | 103% | 122% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.55% | -2.67% | -1.54% | 106% | 155% |
| Class A2 | -6.47% | -1.33% | -0.25% | 107% | 153% |
| Class B | -5.79% | -3.41% | -2.32% | 109% | 143% |
| Class C | -4.49% | -1.02% | -1.88% | 107% | 128% |
| Class E |  |  |  |  |  |
| Overall | -5.33% | -2.21% | -1.63% | 108% | 143% |
| Class D | -5.39% | -1.99% | -2.50% | 112% | 127% |
| Class F (optional) | -1.94% | -0.90% | -0.99% | 116% | 122% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.54% | -1.51% | -1.56% | 107% | 133% |
| Class C | -4.15% | -1.61% | -1.58% | 106% | 121% |
| Class E | -5.13% | -2.59% | -3.34% | 121% | 117% |
| Overall | -4.56% | -1.81% | -2.01% | 110% | 125% |
| Class D | -4.36% | -1.82% | -2.16% | 109% | 118% |
| Class F (optional) | -3.86% | -1.92% | -1.46% | 118% | 120% |

## ALF luma 9x9, classifier 2x2, chroma 7x7

| All Intra Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -2.45%; -3.54%; -3.75%; 103%; 137%. Class A2; -3.05%; -4.04%; -4.07%; 102%; 126%. Class B; -2.79%; -2.76%; -3.34%; 102%; 129%. Class C; -3.29%; -3.32%; -4.41%; 102%; 121%. Class E; -5.08%; -8.51%; -9.00%; 103%; 134%. Overall; -3.27%; -4.19%; -4.71%; 102%; 129%. Class D; -2.59%; -3.20%; -3.88%; 102%; 119%. Class F (optional); -1.87%; -1.21%; -1.47%; 102%; 123%. Random Access Main 10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -4.56%; -3.25%; -1.97%; 106%; 155%. Class A2; -6.49%; -2.18%; -1.55%; 107%; 154%. Class B; -5.79%; -4.44%; -3.01%; 108%; 143%. Class C; -4.52%; -2.04%; -3.21%; 106%; 125%. Class E. Overall; -5.35%; -3.11%; -2.57%; 107%; 142%. Class D; -5.37%; -2.62%; -3.21%; 112%; 127%. Class F (optional); -1.95%; -1.06%; -1.29%; 115%; 120%. Low delay B Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.58%; -3.36%; -2.79%; 107%; 134%. Class C; -4.24%; -2.81%; -3.26%; 107%; 125%. Class E; -5.25%; -1.10%; -2.53%; 122%; 122%. Overall; -4.63%; -2.61%; -2.88%; 111%; 128%. Class D; -4.44%; -3.59%; -3.35%; 109%; 121%. Class F (optional); -3.85%; -2.46%; -2.14%; 118%; 120%. | All Intra Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -2.20%; -4.09%; -2.98%; 100%; 126%. Class A2; -2.84%; -3.56%; -3.52%; 101%; 122%. Class B; -2.29%; -2.84%; -3.13%; 101%; 122%. Class C; -2.54%; -3.14%; -4.01%; 101%; 115%. Class E; -4.04%; -8.00%; -8.35%; 101%; 127%. Overall; -2.71%; -4.09%; -4.24%; 100%; 122%. Class D; -1.88%; -2.98%; -3.55%; 100%; 114%. Class F (optional); -1.53%; -1.49%; -1.53%; 100%; 118%. Random Access Main 10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -4.28%; -3.29%; -0.94%; 101%; 135%. Class A2; -6.43%; -1.81%; -1.83%; 102%; 134%. Class B; -5.40%; -4.59%; -2.99%; 102%; 126%. Class C; -4.16%; -2.32%; -3.16%; 101%; 118%. Class E. Overall; -5.05%; -3.17%; -2.39%; 101%; 127%. Class D; -5.23%; -2.95%; -3.01%; 103%; 118%. Class F (optional); -1.73%; -1.29%; -1.35%; 103%; 117%. Low delay B Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.40%; -3.20%; -2.40%; 101%; 129%. Class C; -4.24%; -3.37%; -3.72%; 101%; 120%. Class E; -4.43%; -0.12%; -1.16%; 107%; 122%. Overall; -4.35%; -2.48%; -2.53%; 103%; 124%. Class D; -4.75%; -3.35%; -3.77%; 102%; 117%. Class F (optional); -3.42%; -2.56%; -1.89%; 106%; 129%. |
| --- | --- |

## ALF luma 9x9, classifier 4x4, chroma 5x5

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.48% | -3.30% | -3.48% | 102% | 127% |
| Class A2 | -2.99% | -3.79% | -3.62% | 102% | 121% |
| Class B | -2.72% | -2.29% | -2.96% | 103% | 119% |
| Class C | -3.08% | -2.71% | -3.59% | 102% | 112% |
| Class E | -4.92% | -6.70% | -7.47% | 102% | 121% |
| Overall | -3.17% | -3.54% | -4.05% | 102% | 119% |
| Class D | -2.37% | -2.81% | -3.35% | 102% | 111% |
| Class F (optional) | -1.71% | -1.05% | -1.14% | 103% | 114% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.62% | -2.71% | -1.56% | 106% | 137% |
| Class A2 | -6.36% | -1.35% | -0.29% | 107% | 135% |
| Class B | -5.64% | -3.41% | -2.34% | 109% | 131% |
| Class C | -4.26% | -0.96% | -1.91% | 107% | 117% |
| Class E |  |  |  |  |  |
| Overall | -5.21% | -2.20% | -1.66% | 107% | 129% |
| Class D | -5.14% | -2.13% | -2.42% | 112% | 116% |
| Class F (optional) | -1.82% | -0.87% | -0.92% | 115% | 114% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.46% | -1.70% | -1.53% | 107% | 122% |
| Class C | -4.07% | -1.68% | -1.63% | 106% | 114% |
| Class E | -4.99% | -2.24% | -3.34% | 121% | 112% |
| Overall | -4.46% | -1.83% | -2.01% | 110% | 117% |
| Class D | -4.25% | -2.15% | -2.30% | 109% | 111% |
| Class F (optional) | -3.52% | -2.18% | -1.85% | 117% | 112% |

## ALF luma 9x9, classifier 4x4, chroma 7x7

| All Intra Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -2.48%; -3.54%; -3.75%; 103%; 126%. Class A2; -2.99%; -4.04%; -4.07%; 102%; 120%. Class B; -2.71%; -2.76%; -3.34%; 103%; 121%. Class C; -3.06%; -3.31%; -4.41%; 102%; 114%. Class E; -4.89%; -8.50%; -8.99%; 103%; 124%. Overall; -3.16%; -4.19%; -4.71%; 103%; 121%. Class D; -2.32%; -3.19%; -3.88%; 103%; 111%. Class F (optional); -1.70%; -1.23%; -1.49%; 103%; 115%. Random Access Main 10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -4.62%; -3.32%; -2.04%; 106%; 137%. Class A2; -6.41%; -2.16%; -1.58%; 108%; 135%. Class B; -5.64%; -4.49%; -3.07%; 109%; 131%. Class C; -4.27%; -2.08%; -3.31%; 107%; 117%. Class E. Overall; -5.23%; -3.15%; -2.63%; 108%; 129%. Class D; -5.14%; -2.62%; -3.26%; 112%; 116%. Class F (optional); -1.78%; -1.08%; -1.25%; 115%; 114%. Low delay B Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.49%; -3.32%; -2.51%; 108%; 119%. Class C; -4.13%; -2.89%; -3.21%; 107%; 116%. Class E; -4.98%; -0.97%; -2.41%; 122%; 114%. Overall; -4.49%; -2.59%; -2.72%; 111%; 116%. Class D; -4.24%; -3.44%; -3.60%; 109%; 110%. Class F (optional); -3.69%; -2.56%; -2.42%; 118%; 114%. | All Intra Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -2.24%; -4.09%; -2.98%; 101%; 119%. Class A2; -2.80%; -3.56%; -3.52%; 101%; 117%. Class B; -2.23%; -2.84%; -3.13%; 100%; 112%. Class C; -2.37%; -3.14%; -4.00%; 101%; 110%. Class E; -3.88%; -7.99%; -8.35%; 101%; 120%. Overall; -2.63%; -4.09%; -4.23%; 101%; 115%. Class D; -1.68%; -2.98%; -3.55%; 101%; 108%. Class F (optional); -1.39%; -1.51%; -1.54%; 101%; 113%. Random Access Main 10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1; -4.37%; -3.41%; -0.98%; 101%; 125%. Class A2; -6.35%; -1.73%; -1.85%; 102%; 123%. Class B; -5.28%; -4.56%; -2.99%; 102%; 119%. Class C; -3.98%; -2.30%; -3.23%; 102%; 112%. Class E. Overall; -4.96%; -3.16%; -2.42%; 102%; 119%. Class D; -5.03%; -2.66%; -2.98%; 103%; 111%. Class F (optional); -1.65%; -1.33%; -1.43%; 104%; 111%. Low delay B Main10. Over BMS-1.0 ALF=0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.34%; -3.10%; -2.66%; 101%; 121%. Class C; -4.17%; -3.53%; -3.68%; 102%; 115%. Class E; -4.40%; -0.05%; -1.13%; 107%; 116%. Overall; -4.30%; -2.48%; -2.62%; 103%; 118%. Class D; -4.63%; -3.39%; -3.78%; 103%; 110%. Class F (optional); -3.39%; -2.49%; -2.44%; 107%; 117%. |
| --- | --- |

## Deblocking TC offset

In JEM, deblocking TC offset was applied when ALF is enabled, this encoder only change was tested  for BMS ALF with TC offset -6 for AI, and TC offset -2 for RA and LB configurations. Anchor is BMS-1.0. It is suggested to consider TC offset in the future common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -0.72% | 0.69% | 100% | 100% |
| Class A2 | -1.11% | -0.20% | -0.47% | 100% | 101% |
| Class B | -0.99% | -0.64% | 0.00% | 100% | 99% |
| Class C | -0.72% | -1.22% | -1.14% | 101% | 101% |
| Class E | -0.67% | 0.18% | 0.73% | 100% | 100% |
| Overall | -0.87% | -0.57% | -0.10% | 100% | 100% |
| Class D | -0.67% | -0.79% | -0.77% | 100% | 100% |
| Class F (optional) |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.53% | -0.18% | 100% | 99% |
| Class A2 | -0.23% | -0.31% | -0.40% | 100% | 100% |
| Class B | -0.26% | -0.58% | -0.56% | 100% | 101% |
| Class C | -0.18% | -0.70% | -0.73% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.55% | -0.50% | 100% | 100% |
| Class D | -0.22% | -0.73% | -0.70% | 100% | 101% |
| Class F (optional) |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.35% | -0.43% | -0.01% | 100% | 98% |
| Class C | -0.30% | -0.16% | -0.36% | 100% | 100% |
| Class E | -0.76% | -0.38% | 0.11% | 100% | 98% |
| Overall | -0.44% | -0.33% | -0.09% | 100% | 99% |
| Class D | -0.35% | 0.23% | -0.78% | 99% | 97% |
| Class F (optional) |  |  |  |  |  |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).