| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0372-v1 |
| --- | --- |

| Title: | CE2-related: Additional results for CE2.4.1.4 with chroma filter shape aligned with luma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu Vadim Seregin Nikolay Shlyakhov Marta Karczewicz | Tel: Email: | martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents additional test results for CE2 test 4.1.4 where ALF filter shape for chroma is aligned with luma component. In the BMS ALF, luma filter can be switched between 5x5, 7x7, or 9x9 filter shapes, while chroma filter size is always 5x5. The same flexible filter structure for chroma is tested in this contribution. For maximum ALF filter shape size 9x9, test results reportedly show 3.26%, 5.34%, and 4.63% luma gain in AI, RA, and LB configurations respectively over VTM-1.0 anchor.

# Introduction

In the BMS software, largest ALF filter shape is 9x9 for luma and 5x5 for chroma components, while classifier block size is 2x2. In the CE2 test 4.1.4, reduced largest filter size to 7x7 and increased filter classifier block size to 4x4 for luma component are tested.

Fully aligned chroma filter with luma filter sizes is tested in this contribution. The chosen chroma filter shape is signaled per picture as it is done for luma.

# Experimental Results

The discussed changes were implemented on top of CE2.4.1.4 software, which is based on BMS-1.0, and two anchors are used: BMS software configured in VTM configuration, and BMS configuration with ALF tool disabled. Test results are summarized in below tables and can be found in detail in the accompanied Excel tables.

Table 1. Summary of tests results for VTM configuration (positive numbers is coding gain).

| Test | AI |  |  | RA |  |  | LB |  |  | RA Enc | RA Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. Max filter 7x7, classifier 2x2 | 3.09 | 4.20 | 4.73 | 5.11 | 3.15 | 2.65 | 4.34 | 2.74 | 2.67 | 105% | 135% |
| 2. Max filter 7x7, classifier 4x4 | 2.98 | 4.20 | 4.73 | 4.99 | 3.15 | 2.65 | 4.22 | 2.55 | 2.65 | 104% | 122% |
| 3. Max filter 9x9, classifier 2x2 | 3.26 | 4.49 | 5.01 | 5.34 | 3.60 | 3.09 | 4.63 | 3.60 | 3.47 | 107% | 144% |
| 4. Max filter 9x9, classifier 4x4 | 3.15 | 4.49 | 5.01 | 5.23 | 3.61 | 3.11 | 4.53 | 3.50 | 3.61 | 107% | 130% |

## ALF largest filter size 7x7, classifier 2x2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.28% | -3.56% | -3.77% | 101% | 132% |
| Class A2 | -2.87% | -4.06% | -4.09% | 101% | 121% |
| Class B | -2.66% | -2.79% | -3.37% | 102% | 123% |
| Class C | -3.09% | -3.32% | -4.42% | 101% | 117% |
| Class E | -4.83% | -8.51% | -9.01% | 101% | 124% |
| Overall | -3.09% | -4.20% | -4.73% | 101% | 123% |
| Class D | -2.39% | -3.15% | -3.80% | 101% | 114% |
| Class F (optional) | -1.85% | -1.25% | -1.50% | 102% | 115% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.34% | -3.26% | -2.00% | 103% | 145% |
| Class A2 | -6.25% | -2.21% | -1.62% | 104% | 143% |
| Class B | -5.48% | -4.45% | -3.11% | 106% | 135% |
| Class C | -4.37% | -2.15% | -3.35% | 104% | 122% |
| Class E |  |  |  |  |  |
| Overall | -5.11% | -3.15% | -2.65% | 105% | 135% |
| Class D | -5.10% | -2.75% | -3.24% | 106% | 120% |
| Class F (optional) | -1.95% | -1.16% | -1.29% | 110% | 116% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.18% | -3.22% | -2.51% | 104% | 126% |
| Class C | -4.03% | -2.85% | -3.21% | 104% | 120% |
| Class E | -5.02% | -1.80% | -2.22% | 114% | 116% |
| Overall | -4.34% | -2.74% | -2.67% | 107% | 122% |
| Class D | -4.09% | -3.63% | -3.67% | 105% | 115% |
| Class F (optional) | -3.74% | -2.46% | -2.34% | 112% | 117% |

## ALF largest filter size 7x7, classifier 4x4

All Intra Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -2.30%; -3.56%; -3.77%; 100%; 119%. Class A2; -2.82%; -4.06%; -4.09%; 100%; 113%. Class B; -2.58%; -2.79%; -3.37%; 101%; 115%. Class C; -2.85%; -3.32%; -4.42%; 100%; 108%. Class E; -4.65%; -8.51%; -9.00%; 101%; 116%. Overall; -2.98%; -4.20%; -4.73%; 101%; 114%. Class D; -2.11%; -3.14%; -3.79%; 100%; 107%. Class F (optional); -1.67%; -1.26%; -1.51%; 101%; 109%. Random Access Main 10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1; -4.41%; -3.21%; -2.06%; 103%; 128%. Class A2; -6.16%; -2.28%; -1.63%; 104%; 128%. Class B; -5.32%; -4.46%; -3.06%; 106%; 123%. Class C; -4.12%; -2.11%; -3.34%; 103%; 111%. Class E. Overall; -4.99%; -3.15%; -2.65%; 104%; 122%. Class D; -4.85%; -2.86%; -3.33%; 105%; 110%. Class F (optional); -1.79%; -1.11%; -1.34%; 109%; 109%. Low delay B Main10. Over VTM-1.0. Y; U; V; EncT; DecT. Class A1. Class A2. Class B; -4.08%; -3.25%; -2.36%; 104%; 115%. Class C; -3.92%; -2.87%; -3.10%; 103%; 111%. Class E; -4.87%; -0.96%; -2.55%; 113%; 107%. Overall; -4.22%; -2.55%; -2.65%; 106%; 111%. Class D; -3.81%; -3.27%; -3.54%; 104%; 108%. Class F (optional); -3.46%; -2.10%; -1.94%; 112%; 108%.

## ALF largest filter size 9x9, classifier 2x2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.44% | -3.75% | -3.98% | 102% | 140% |
| Class A2 | -3.05% | -4.16% | -4.23% | 101% | 130% |
| Class B | -2.78% | -3.01% | -3.53% | 103% | 130% |
| Class C | -3.28% | -3.69% | -4.73% | 102% | 122% |
| Class E | -5.05% | -9.07% | -9.65% | 102% | 136% |
| Overall | -3.26% | -4.49% | -5.01% | 102% | 131% |
| Class D | -2.60% | -3.23% | -3.91% | 102% | 117% |
| Class F (optional) | -1.86% | -1.38% | -1.61% | 103% | 122% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.57% | -3.45% | -2.09% | 106% | 157% |
| Class A2 | -6.48% | -2.63% | -2.49% | 106% | 157% |
| Class B | -5.79% | -4.99% | -3.48% | 108% | 144% |
| Class C | -4.51% | -2.72% | -3.79% | 106% | 127% |
| Class E |  |  |  |  |  |
| Overall | -5.34% | -3.60% | -3.09% | 107% | 144% |
| Class D | -5.38% | -2.86% | -3.52% | 111% | 125% |
| Class F (optional) | -1.93% | -1.26% | -1.45% | 114% | 119% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.59% | -4.36% | -3.17% | 106% | 129% |
| Class C | -4.27% | -3.43% | -4.29% | 106% | 122% |
| Class E | -5.19% | -2.56% | -2.88% | 122% | 121% |
| Overall | -4.63% | -3.60% | -3.47% | 110% | 125% |
| Class D | -4.46% | -3.70% | -4.03% | 108% | 115% |
| Class F (optional) | -3.77% | -2.74% | -2.34% | 117% | 119% |

## ALF largest filter size 9x9, classifier 4x4

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.47% | -3.75% | -3.98% | 102% | 127% |
| Class A2 | -2.99% | -4.16% | -4.23% | 102% | 122% |
| Class B | -2.70% | -3.01% | -3.53% | 102% | 123% |
| Class C | -3.04% | -3.69% | -4.73% | 102% | 117% |
| Class E | -4.87% | -9.07% | -9.64% | 103% | 126% |
| Overall | -3.15% | -4.49% | -5.01% | 102% | 122% |
| Class D | -2.33% | -3.23% | -3.90% | 102% | 111% |
| Class F (optional) | -1.69% | -1.40% | -1.63% | 103% | 116% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -4.63% | -3.55% | -2.16% | 105% | 139% |
| Class A2 | -6.41% | -2.53% | -2.46% | 106% | 138% |
| Class B | -5.63% | -5.01% | -3.48% | 108% | 132% |
| Class C | -4.28% | -2.72% | -3.84% | 106% | 116% |
| Class E |  |  |  |  |  |
| Overall | -5.23% | -3.61% | -3.11% | 107% | 130% |
| Class D | -5.15% | -2.92% | -3.59% | 111% | 112% |
| Class F (optional) | -1.78% | -1.29% | -1.39% | 114% | 113% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -4.51% | -4.15% | -3.32% | 107% | 122% |
| Class C | -4.17% | -3.52% | -4.05% | 106% | 116% |
| Class E | -5.04% | -2.37% | -3.52% | 121% | 112% |
| Overall | -4.53% | -3.50% | -3.61% | 110% | 118% |
| Class D | -4.29% | -3.63% | -3.99% | 109% | 109% |
| Class F (optional) | -3.64% | -2.57% | -2.39% | 117% | 109% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).