| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0374-v1 |
| --- | --- |

| Title: | CE6.2.2: Hierarchically Structured Matrix-based Transforms for NSST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CE 6.2.2 where hierarchically structured matrix-based transforms (HSMTs) are proposed as alternatives to 8x8 HyGT-based NSST implementations in JEM7. The proposed set of HSMTs reduces memory use in a matrix based NSST scheme by x.x%. The CE results show that

- On top of VTM 1.1, the proposed method provides 6.7%, 3.8% and 2.2% BD-rate gains for luma under AI, RA and LD configurations, respectively.

- On top of BMS 1.1, the proposed method provides 1.3%, 0.7% and 0.3% BD-rate gains for luma under AI, RA and LD configurations, respectively.

The proposed method does not introduce any additional encoding/decoding runtime complexity

# CE Description

In JVET-C0053, a set of secondary transforms, namely HyGT, is proposed to replace the original 4x4 matrix-based NSSTs. Moreover, in JVET-C0063, a set of HyGTs on top of 8x8 NSST is proposed to achieve higher coding performance. This contribution in JVET-J0062 proposes a method to implement NSSTs of dimension 64 using HSMT as an alternative to HyGT [1,2]. In this CE contribution, we test the performance if HSMTs under CTC conditions.

As in HyGT, HSMT implements NSST using multiple passes of smaller transforms for a given block. However, instead of using the pairwise Givens rotations (i.e., butterfly structures) or a full matrix, a hierarchical structure with multiple passes consisting of smaller matrices and permutations are used to define a non-separable transform. Figure 1 illustrates how HSMT is used for transforming 64 samples (i.e., 8×8 top-left subblocks in NSST) defined by a collection of 8×8 matrices and permutations.

The permutations used between passes correspond to block rotations, i.e., they are equivalent to the horizontal-vertical passes in separable transforms, and thus do not need additional memory. A main difference is that in HSMT the 8×8 matrices used to transform each row and each column transform is different, to allow obtaining the non-separable transform.

Figure 1:  Hierarchically Structured Matrix-based Transformation

In this CE contribution, the proposed HSMTs are applied as secondary transforms where the primary transformation is defined as in BMS.

# CE Results

The CE results of the proposed method tested on top of VTM with EMT=3 and EMTFast=3 options and BMS configurations are presented in Tables 1.1 and 1.2, respectively.

Table 1.1: Comparison of the proposed AMT enabled on BMS1.1 with VTM configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -6.47% | -7.22% | -7.87% | 560% | 132% | 6.93% | 37.79% | 32.26% | 64% | 61% |
| Class A2 | -5.58% | -5.19% | -4.85% | 607% | 129% | 5.77% | 16.66% | 11.99% | 63% | 67% |
| Class B | -6.15% | -5.50% | -5.63% | 629% | 126% | 4.18% | 12.33% | 17.23% | 64% | 69% |
| Class C | -7.21% | -4.49% | -5.14% | 701% | 117% | 4.38% | 14.31% | 18.89% | 65% | 74% |
| Class E | -8.22% | -6.30% | -6.42% | 537% | 122% | 6.15% | 12.97% | 16.49% | 64% | 64% |
| Overall | -6.69% | -5.64% | -5.89% | 612% | 125% | 5.28% | 17.84% | 19.11% | 64% | 68% |
| Class D | -6.55% | -3.75% | -4.00% | 784% | 117% | 3.29% | 11.23% | 13.32% | 65% | 76% |
| Class F (optional) | -4.71% | -3.17% | -3.44% | 614% | 112% | 6.37% | 15.94% | 16.94% | 65% | 68% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.68% | -5.35% | -5.90% | 282% | 115% | 15.69% | 39.39% | 40.97% | 70% | 47% |
| Class A2 | -3.09% | -2.21% | -1.66% | 239% | 110% | 20.54% | 26.18% | 18.16% | 67% | 43% |
| Class B | -4.11% | -4.18% | -4.41% | 254% | 109% | 13.60% | 24.22% | 26.23% | 66% | 49% |
| Class C | -4.13% | -2.51% | -3.08% | 263% | 109% | 10.07% | 17.28% | 20.37% | 66% | 54% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.82% | -3.57% | -3.80% | 259% | 111% | 14.46% | 25.80% | 26.00% | 67% | 49% |
| Class D | -3.32% | -1.83% | -2.57% | 261% | 106% | 11.99% | 15.79% | 17.31% | 63% | 52% |
| Class F (optional) | -3.25% | -1.94% | -1.83% | 299% | 109% | 8.83% | 18.29% | 18.27% | 66% | 65% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.44% | -0.76% | -0.93% | 225% | 115% | 9.36% | 11.29% | 11.51% | 60% | 60% |
| Class C | -1.97% | 0.03% | -0.25% | 212% | 110% | 7.97% | 9.52% | 10.46% | 62% | 65% |
| Class E | -2.26% | -0.71% | -0.78% | 165% | 114% | 10.26% | 9.91% | 13.74% | 49% | 68% |
| Overall | -2.24% | -0.49% | -0.67% | 204% | 113% | 9.12% | 10.35% | 11.72% | 58% | 63% |
| Class D | -1.53% | 0.28% | 0.04% | 185% | 104% | 10.73% | 10.08% | 11.23% | 61% | 66% |
| Class F (optional) | -1.73% | -0.77% | -0.30% | 232% | 106% | 9.99% | 15.24% | 15.32% | 61% | 71% |

Table 1.2: Comparison of the proposed AMT enabled on BMS1.1 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.93% | -32.33% | -30.34% | 896% | 215% | -1.55% | -1.15% | -0.87% | 102% | 100% |
| Class A2 | -11.63% | -18.32% | -14.64% | 960% | 192% | -0.92% | -0.28% | 0.02% | 100% | 99% |
| Class B | -10.88% | -16.34% | -20.02% | 1007% | 185% | -1.06% | -0.76% | -0.77% | 102% | 101% |
| Class C | -12.45% | -17.00% | -20.33% | 1105% | 159% | -1.59% | -1.10% | -0.89% | 102% | 101% |
| Class E | -15.00% | -17.65% | -20.29% | 853% | 185% | -1.62% | -0.88% | -0.90% | 101% | 98% |
| Overall | -12.55% | -19.70% | -20.96% | 973% | 185% | -1.33% | -0.84% | -0.70% | 102% | 100% |
| Class D | -10.40% | -13.89% | -15.76% | 1238% | 156% | -0.97% | -0.81% | -0.79% | 103% | 102% |
| Class F (optional) | -11.26% | -16.59% | -17.54% | 964% | 163% | -1.00% | -0.69% | -0.66% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.43% | -31.28% | -33.13% | 414% | 248% | -0.79% | -0.72% | -0.69% | 102% | 101% |
| Class A2 | -19.83% | -21.69% | -16.30% | 362% | 253% | -0.36% | 0.20% | 0.25% | 102% | 100% |
| Class B | -16.12% | -23.01% | -24.38% | 391% | 224% | -0.69% | -0.40% | -0.38% | 102% | 101% |
| Class C | -13.62% | -16.98% | -19.82% | 408% | 203% | -0.83% | -0.24% | -0.57% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.46% | -22.79% | -23.30% | 394% | 228% | -0.68% | -0.30% | -0.37% | 102% | 101% |
| Class D | -13.84% | -15.50% | -17.22% | 424% | 205% | -0.45% | -0.52% | -0.47% | 103% | 102% |
| Class F (optional) | -11.58% | -16.98% | -16.91% | 467% | 170% | -0.58% | -0.41% | -0.22% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.95% | -10.91% | -11.43% | 378% | 193% | -0.22% | -0.25% | -0.40% | 101% | 101% |
| Class C | -9.52% | -8.95% | -10.06% | 345% | 170% | -0.33% | -0.46% | -0.60% | 101% | 100% |
| Class E | -11.59% | -9.19% | -12.29% | 338% | 172% | -0.28% | 0.38% | 0.45% | 100% | 103% |
| Overall | -10.63% | -9.83% | -11.19% | 357% | 179% | -0.27% | -0.16% | -0.25% | 101% | 101% |
| Class D | -10.83% | -9.14% | -9.73% | 302% | 161% | -0.13% | -0.43% | -0.10% | 100% | 101% |
| Class F (optional) | -10.96% | -13.87% | -13.77% | 383% | 146% | -0.42% | -0.22% | -0.44% | 100% | 98% |

Additional results with different EMT, and EMTFast options will be provided in an updated version of this document.

# References

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi, “EE2.7: TU-level non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C0053, 3rd Meeting

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE2.7 related: Improved non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0120, 4th Meeting

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).