| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0374-v2 |
| --- | --- |

| Title: | CE6.2.2: Hierarchically Structured Matrix-based Transforms for NSST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CE 6.2.2 where hierarchically structured matrix-based transforms (HSMTs) are proposed as alternatives to 8x8 HyGT-based NSST implementations in JEM7. The CE results show that

- On top of VTM 1.1, the proposed method by enabling AMT (with EMT=3 and EMTFast=3 options) provides 6.7%, 3.8% and 2.2% average BD-rate gains for AI, RA and LD coding, respectively.

- On top of VTM 1.1, the proposed method with EMT=0 option provides 4.6%, 2.2% and 0.9% average BD-rate gains for AI, RA and LD coding, respectively.

- On top of BMS 1.1, the proposed method provides 1.3%, 0.7% and 0.3% average BD-rate gains for AI, RA and LD coding, respectively.

The proposed method does not introduce any additional encoding/decoding runtime complexity

# CE Description

In JVET-C0053, a set of secondary transforms, namely HyGT, is proposed to replace the original 4x4 matrix-based NSSTs. Moreover, in JVET-C0063, a set of HyGTs on top of 8x8 NSST is proposed to achieve higher coding performance. This contribution in JVET-J0062 proposes a method to implement NSSTs of dimension 64 using HSMT as an alternative to HyGT [1,2]. In this CE contribution, we test the performance if HSMTs under CTC conditions.

As in HyGT, HSMT implements NSST using multiple passes of smaller transforms for a given block. However, instead of using the pairwise Givens rotations (i.e., butterfly structures) or a full matrix, a hierarchical structure with multiple passes consisting of smaller matrices and permutations are used to define a non-separable transform. Figure 1 illustrates how HSMT is used for transforming 64 samples (i.e., 8×8 top-left subblocks in NSST) defined by a collection of 8×8 matrices and permutations.

The permutations used between passes correspond to block rotations, i.e., they are equivalent to the horizontal-vertical passes in separable transforms, and thus do not need additional memory. A main difference is that in HSMT the 8×8 matrices used to transform each row and each column transform is different, to allow obtaining the non-separable transform.

Figure 1:  Hierarchically Structured Matrix-based Transformation

In this CE contribution, the proposed HSMTs are applied as secondary transforms where the primary transformation is defined as in BMS.

# Complexity Metrics

## Operation counts for individual transforms

The estimated operation counts for non-separable secondary forward/inverse transforms in this CE are shown in Table 1.

|  | # of Multiplication | # of Addition | # of Shift |
| --- | --- | --- | --- |
| NSST 4x4 | 256 | 257 | 145 |
| NSST 8x8 | 1664 | 1665 | 81 |

Table 1: Estimated operation counts

Same as for BMS, the NSST of size 4x4 is implemented with HyGT, while for NSST of size 8x8, the proposed Hierarchical structure is applied.

## Memory requirements for storing transform cores

The estimated number of elements and bits needed to store each transform matrix in this CE are shown in Table 2.

|  | Total # of elements | # of bits per elements | Total # of bits |
| --- | --- | --- | --- |
| NSST 4x4 | 512 | 10 | 5120 |
| NSST 8x8 | 161280 | 10 | 1612800 |

Table 2: Estimated number of elements and bits for each transform matrix

The estimated number of elements and bits needed to store the pointers to transforms in this CE are shown in Table 3.

|  | Total # of elements | # of bits per elements | Total # of bits |
| --- | --- | --- | --- |
| NSST 4x4 | 6720 | 8 | 53760 |
| NSST 8x8 | 6720 | 6 | 40320 |

Table 3: Estimated number of elements and bits for pointers to transform

## Specify the minimum bit-precision required by the transforms

Same as for BMS

## Specify if a transform requires multiple iterations

No additional iteration is required from BMS

## Other operations and memory requirements relevant to the proposed tool

None

## Encoder and decoder times

Listed in the experimental results in section 3 and 4.

## List of all combinations of transform and transforms block sizes used for primary transformation

Same as for BMS

## List of all combinations of transforms block size and transform type used for secondary transformation

The secondary transforms are selected based on intra prediction mode and block size, same as for BMS.

The selection between 4x4 and 8x8 blocks are determined based on the following criterion:

| Block width < 8 | 4x4 NSST |
| --- | --- |
| or Block height < 8 |  |
| Block width >= 8 | 8x8 NSST |
| or Block height >= 8 |  |

## Provide analysis of implementation and arithmetic commonalities of proposed transforms

The 4x4 NSST is implemented with HyGT, same as for BMS, while 8x8 NSST is implemented with hierarchical structure proposed in this CE.

# CE Results

The CE results of the proposed method are presented (i) on top of VTM by enabling AMT (with EMT=3 and EMTFast=3 options) (ii) on top of VTM and (iii) BMS configurations in Tables 1.1, 1.2 and 1.3 respectively.

Table 1.1: Results of the proposed method with AMT enabled on VTM over (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -6.47% | -7.22% | -7.87% | 560% | 132% | 6.93% | 37.79% | 32.26% | 64% | 61% |
| Class A2 | -5.58% | -5.19% | -4.85% | 607% | 129% | 5.77% | 16.66% | 11.99% | 63% | 67% |
| Class B | -6.15% | -5.50% | -5.63% | 629% | 126% | 4.18% | 12.33% | 17.23% | 64% | 69% |
| Class C | -7.21% | -4.49% | -5.14% | 701% | 117% | 4.38% | 14.31% | 18.89% | 65% | 74% |
| Class E | -8.22% | -6.30% | -6.42% | 537% | 122% | 6.15% | 12.97% | 16.49% | 64% | 64% |
| Overall | -6.69% | -5.64% | -5.89% | 612% | 125% | 5.28% | 17.84% | 19.11% | 64% | 68% |
| Class D | -6.55% | -3.75% | -4.00% | 784% | 117% | 3.29% | 11.23% | 13.32% | 65% | 76% |
| Class F (optional) | -4.71% | -3.17% | -3.44% | 614% | 112% | 6.37% | 15.94% | 16.94% | 65% | 68% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.68% | -5.35% | -5.90% | 282% | 115% | 15.69% | 39.39% | 40.97% | 70% | 47% |
| Class A2 | -3.09% | -2.21% | -1.66% | 239% | 110% | 20.54% | 26.18% | 18.16% | 67% | 43% |
| Class B | -4.11% | -4.18% | -4.41% | 254% | 109% | 13.60% | 24.22% | 26.23% | 66% | 49% |
| Class C | -4.13% | -2.51% | -3.08% | 263% | 109% | 10.07% | 17.28% | 20.37% | 66% | 54% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -3.82% | -3.57% | -3.80% | 259% | 111% | 14.46% | 25.80% | 26.00% | 67% | 49% |
| Class D | -3.32% | -1.83% | -2.57% | 261% | 106% | 11.99% | 15.79% | 17.31% | 63% | 52% |
| Class F (optional) | -3.25% | -1.94% | -1.83% | 299% | 109% | 8.83% | 18.29% | 18.27% | 66% | 65% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -2.44% | -0.76% | -0.93% | 225% | 115% | 9.36% | 11.29% | 11.51% | 60% | 60% |
| Class C | -1.97% | 0.03% | -0.25% | 212% | 110% | 7.97% | 9.52% | 10.46% | 62% | 65% |
| Class E | -2.26% | -0.71% | -0.78% | 165% | 114% | 10.26% | 9.91% | 13.74% | 49% | 68% |
| Overall | -2.24% | -0.49% | -0.67% | 204% | 113% | 9.12% | 10.35% | 11.72% | 58% | 63% |
| Class D | -1.53% | 0.28% | 0.04% | 185% | 104% | 10.73% | 10.08% | 11.23% | 61% | 66% |
| Class F (optional) | -1.73% | -0.77% | -0.30% | 232% | 106% | 9.99% | 15.24% | 15.32% | 61% | 71% |

Table 1.2: Results of the proposed method VTM over (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -4.64% | -6.36% | -7.08% | 257% | 108% | 9.08% | 39.05% | 33.34% | 29% | 50% |
| Class A2 | -3.61% | -3.94% | -3.97% | 268% | 108% | 8.04% | 18.24% | 13.03% | 28% | 56% |
| Class B | -4.02% | -4.52% | -4.43% | 267% | 104% | 6.58% | 13.53% | 18.78% | 27% | 57% |
| Class C | -5.42% | -4.11% | -4.47% | 285% | 107% | 6.42% | 14.78% | 19.72% | 26% | 68% |
| Class E | -5.49% | -4.25% | -4.30% | 244% | 108% | 9.35% | 15.41% | 19.18% | 29% | 57% |
| Overall | -4.61% | -4.59% | -4.78% | 266% | 107% | 7.66% | 19.16% | 20.52% | 28% | 58% |
| Class D | -4.89% | -3.37% | -3.44% | 307% | 113% | 5.14% | 11.70% | 13.98% | 25% | 74% |
| Class F (optional) | -3.48% | -2.81% | -2.81% | 253% | 104% | 7.77% | 16.39% | 17.64% | 27% | 63% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -2.35% | -5.37% | -5.97% | 136% | 104% | 17.33% | 39.30% | 40.77% | 34% | 42% |
| Class A2 | -1.46% | -2.26% | -2.05% | 126% | 103% | 22.57% | 26.16% | 17.67% | 36% | 40% |
| Class B | -2.22% | -3.84% | -3.90% | 126% | 101% | 15.88% | 24.67% | 26.95% | 33% | 45% |
| Class C | -2.67% | -2.81% | -2.90% | 130% | 102% | 11.76% | 16.95% | 20.59% | 32% | 51% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.21% | -3.56% | -3.68% | 129% | 102% | 16.41% | 25.83% | 26.16% | 33% | 45% |
| Class D | -2.05% | -1.96% | -2.44% | 130% | 102% | 13.51% | 15.62% | 17.48% | 32% | 51% |
| Class F (optional) | -2.14% | -1.72% | -1.60% | 138% | 103% | 10.12% | 18.55% | 18.59% | 31% | 61% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.73% | -1.47% | -1.30% | 116% | 100% | 11.32% | 10.50% | 11.14% | 31% | 52% |
| Class C | -0.80% | -0.96% | -1.12% | 116% | 101% | 9.29% | 8.45% | 9.50% | 34% | 60% |
| Class E | -1.15% | -0.64% | -1.43% | 106% | 100% | 11.53% | 9.99% | 12.89% | 31% | 60% |
| Overall | -0.86% | -1.09% | -1.27% | 114% | 101% | 10.70% | 9.69% | 11.03% | 32% | 57% |
| Class D | -0.49% | -0.46% | -1.13% | 112% | 101% | 11.95% | 9.30% | 9.88% | 37% | 63% |
| Class F (optional) | -0.90% | -0.76% | -1.22% | 120% | 98% | 10.97% | 15.36% | 14.27% | 31% | 66% |

Table 1.3: Results of the proposed method on BMS over (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -13.93% | -32.33% | -30.34% | 896% | 215% | -1.55% | -1.15% | -0.87% | 102% | 100% |
| Class A2 | -11.63% | -18.32% | -14.64% | 960% | 192% | -0.92% | -0.28% | 0.02% | 100% | 99% |
| Class B | -10.88% | -16.34% | -20.02% | 1007% | 185% | -1.06% | -0.76% | -0.77% | 102% | 101% |
| Class C | -12.45% | -17.00% | -20.33% | 1105% | 159% | -1.59% | -1.10% | -0.89% | 102% | 101% |
| Class E | -15.00% | -17.65% | -20.29% | 853% | 185% | -1.62% | -0.88% | -0.90% | 101% | 98% |
| Overall | -12.55% | -19.70% | -20.96% | 973% | 185% | -1.33% | -0.84% | -0.70% | 102% | 100% |
| Class D | -10.40% | -13.89% | -15.76% | 1238% | 156% | -0.97% | -0.81% | -0.79% | 103% | 102% |
| Class F (optional) | -11.26% | -16.59% | -17.54% | 964% | 163% | -1.00% | -0.69% | -0.66% | 103% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -17.43% | -31.28% | -33.13% | 414% | 248% | -0.79% | -0.72% | -0.69% | 102% | 101% |
| Class A2 | -19.83% | -21.69% | -16.30% | 362% | 253% | -0.36% | 0.20% | 0.25% | 102% | 100% |
| Class B | -16.12% | -23.01% | -24.38% | 391% | 224% | -0.69% | -0.40% | -0.38% | 102% | 101% |
| Class C | -13.62% | -16.98% | -19.82% | 408% | 203% | -0.83% | -0.24% | -0.57% | 102% | 101% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.46% | -22.79% | -23.30% | 394% | 228% | -0.68% | -0.30% | -0.37% | 102% | 101% |
| Class D | -13.84% | -15.50% | -17.22% | 424% | 205% | -0.45% | -0.52% | -0.47% | 103% | 102% |
| Class F (optional) | -11.58% | -16.98% | -16.91% | 467% | 170% | -0.58% | -0.41% | -0.22% | 103% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.95% | -10.91% | -11.43% | 378% | 193% | -0.22% | -0.25% | -0.40% | 101% | 101% |
| Class C | -9.52% | -8.95% | -10.06% | 345% | 170% | -0.33% | -0.46% | -0.60% | 101% | 100% |
| Class E | -11.59% | -9.19% | -12.29% | 338% | 172% | -0.28% | 0.38% | 0.45% | 100% | 103% |
| Overall | -10.63% | -9.83% | -11.19% | 357% | 179% | -0.27% | -0.16% | -0.25% | 101% | 101% |
| Class D | -10.83% | -9.14% | -9.73% | 302% | 161% | -0.13% | -0.43% | -0.10% | 100% | 101% |
| Class F (optional) | -10.96% | -13.87% | -13.77% | 383% | 146% | -0.42% | -0.22% | -0.44% | 100% | 98% |

# References

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, R. Joshi, “EE2.7: TU-level non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-C0053, 3rd Meeting

- X. Zhao, A. Said, V. Seregin, M. Karczewicz, J. Chen, “EE2.7 related: Improved non-separable secondary transform”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Doc. JVET-D0120, 4th Meeting

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).