| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0375-v2 |
| --- | --- |

| Title: | CE6.1.1: Extended AMT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CE 6.1.1 where an extension to primary transforms in BMS is tested. In addition to the transforms used in BMS, DST-4 and identity transforms are used to generate more combinations of separable transforms. A look-up table is designed for mapping different set of transforms based on intra-mode and block size information. The CE results show that

- On top of VTM 1.1, the proposed method provides 3.6%, 2.2% and 1.5% average BD-rate gains for luma under AI, RA and LD configurations, respectively.

- On top of BMS 1.1, the proposed method provides 0.3%, 0.1% and 0.1% average BD-rate gains for luma under AI, RA and LD configurations, respectively.

The proposed method does not introduce any additional encoding/decoding time complexity.

This contribution also includes the results for low-complexity implementations using transform adjustment stages (TAS), described in JVET-J0066 and CE 6.1.2 (JVET-K0272). The results show that TAS implementation of the proposed scheme (i) results in less than 0.05% BD-rate loss for AI, RA and LD on VTM and BMS configurations and (ii) provides slight reduction in encoding and decoding time.

# CE Description

In CE 6.1.1, the AMT transforms in BMS are replaced with a new set of transforms. In addition to transforms used in BMS, which are DCT-2, DCT-5, DCT-8, DST-1, and DST-7, two more one-dimensional are added for intra coded blocks. Namely, DST-4 and identity transform are added. It was proposed in JVET-C0022 and JVET-D0126 as well as in the CFP response JVET-J0021, respectively.

For intra coding, the following 7 transform sets are defined, where each set consist of 4 transform pairs:

T0, intra = { (DST-4, DST-4), (DST-7, DST-7), (DST-4, DCT-8), (DCT-8, DST-4) }

T1, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DCT-5) }

T2, intra = { (DST-7, DST-7), (DST-7, DCT-8), (DCT-8, DST-7), (DCT-5, DCT-5) }

T3, intra = { (DST-4, DST-4), (DST-4, DCT-5), (DCT-8, DST-4), (DST-1, DST-7) }

T4, intra = { (DST-4, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T5, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-8, DST-7), (DST-1, DST-7) }

T6, intra = { (DST-7, DST-7), (DST-7, DCT-5), (DCT-5, DST-7), (DST-1, DST-7) }.

The first entry in the transform pair determines vertical transform, and the second entry specifies the horizontal transform.

A look-up table is defined to assign each of the 7 transform sets based on different intra prediction modes and block sizes.

Identity transform is applied for blocks that are not exceeding 16x16 and have intra modes within the proximity of horizontal and vertical intra directions, the proximity is defined by a block size-based threshold. If transform index is equal to 3 and block satisfies the above condition, the horizontal and/or vertical identity transform is applied.

For inter coding, the same transformation scheme in BMS is proposed.

Moreover, the proposed AMTs are implemented using TAS with 6-tap band matrices (see JVET-J0066 and for the details), and corresponding results are presented in Section 3.

# Complexity Metrics

## Operation counts for individual transforms

The estimated operation counts for 1D forward/inverse transforms in this CE are shown in Table 1.

|  | # of Multiplication |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 256 | 1024 | 2048 |
| DCT8 | 8 | 64 | 256 | 1024 | 2048 |
| DST1 | 8 | 64 | 256 | 1024 | 2048 |
| DCT5 | 16 | 64 | 256 | 1024 | 2048 |
| DST4 | 16 | 64 | 256 | 1024 | 2048 |
| Identity | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 8 | 24 | 88 | 344 | 684 |

|  | # of Addition |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 257 | 1025 | 2049 |
| DCT8 | 15 | 65 | 257 | 1025 | 2049 |
| DST1 | 13 | 65 | 257 | 1025 | 2049 |
| DCT5 | 17 | 65 | 257 | 1025 | 2049 |
| DST4 | 17 | 65 | 257 | 1025 | 2049 |
| Identity | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 13 | 37 | 113 | 401 | 807 |

|  | # of Shift |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 17 | 33 | 33 |
| DCT8 | 5 | 9 | 17 | 33 | 33 |
| DST1 | 5 | 9 | 17 | 33 | 33 |
| DCT5 | 5 | 9 | 17 | 33 | 33 |
| DST4 | 5 | 9 | 17 | 33 | 33 |
| Identity | 4 | 8 | 16 | 32 | 33 |
| DCT2 | 5 | 9 | 17 | 33 | 33 |

Table 1: Estimated operation counts for 1D forward/inverse transforms in extended AMT

In this CE, we also include the implementation using TAS with 6-tap band matrices (see JVET-J0066 and JVET-K0272 for the details) for DCT-4, DCT-5, DCT-8, DST-1, and DST-7. Besides, a simplification for implementation of identity matrix multiplication is applied. The estimated operation counts are shown in Table 2. The operation counts include the number of operations needed for implementing DCT-2 family and adjustment stages (multiplication of sparse band matrices)

|  | # of Multiplication |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 182 | 534 | 1750 |
| DCT8 | 8 | 64 | 182 | 534 | 1750 |
| DST1 | 8 | 64 | 182 | 534 | 875 |
| DCT5 | 16 | 64 | 182 | 534 | 875 |
| DST4 | 16 | 64 | 174 | 526 | 1750 |
| Identity | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 8 | 24 | 86 | 342 | 683 |

|  | # of Addition |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 210 | 594 | 1842 |
| DCT8 | 15 | 65 | 210 | 594 | 1842 |
| DST1 | 13 | 65 | 210 | 594 | 1192 |
| DCT5 | 17 | 65 | 210 | 594 | 1192 |
| DST4 | 17 | 65 | 210 | 594 | 1842 |
| Identity | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 13 | 37 | 113 | 401 | 807 |

|  | # of Shift |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 34 | 66 | 98 |
| DCT8 | 5 | 9 | 34 | 66 | 98 |
| DST1 | 5 | 9 | 34 | 66 | 98 |
| DCT5 | 5 | 9 | 34 | 66 | 98 |
| DST4 | 5 | 9 | 34 | 66 | 98 |
| Identity | 4 | 8 | 16 | 32 | 32 |
| DCT2 | 5 | 9 | 17 | 33 | 33 |

Table 2: Estimated operation counts for 1D forward/inverse transforms in extended AMT with TAS approximation

## Memory requirements for storing transform cores

The estimated number of elements and bits needed to store each transform matrix in this CE are shown in Table 3.

|  | # of elements in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| DCT8 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| DST1 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| DCT5 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| DST4 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| Identity | 0 | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 16 | 64 | 256 | 1024 | 4096 | 5456 |
| Summary (for each size) | 96 | 384 | 1536 | 6144 | 24576 | 32736 |

|  | # of bits in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| DCT8 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| DST1 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| DCT5 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| DST4 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| Identity | 0 | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 160 | 640 | 2560 | 10240 | 40960 | 54560 |
| Summary (for each size) | 960 | 3840 | 15360 | 61440 | 245760 | 327360 |

Table 3: Estimated number of elements and bits for each transform matrix

Note that in BMS and this core experiment, the 2x2 matrix for DST-7, DCT-8, DST-1, and DCT-5 are not applied in transform.

With transform adjustment stages (TAS) proposed in JVET-J0066 and the implementation described in JVET-K0272, the estimated number of elements and bits used for transform are shown in Table 4.

|  | # of elements |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT8 | 16 | 64 | 0 | 0 | 0 | 80 |
| DST1 | 16 | 64 | 96 | 192 | 384 | 752 |
| DCT5 | 16 | 64 | 96 | 192 | 384 | 752 |
| DST4 | 16 | 64 | 96 | 192 | 384 | 752 |
| Identity | 0 | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 16 | 64 | 86 | 342 | 1366 | 1874 |
| Summary (for each size) | 96 | 384 | 470 | 1110 | 2902 | 4962 |

|  | # of bits |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 | Summary (for each transform) |
| DST7 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DCT8 | 160 | 640 | 0 | 0 | 0 | 800 |
| DST1 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DCT5 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| DST4 | 160 | 640 | 960 | 1920 | 3840 | 7520 |
| Identity | 0 | 0 | 0 | 0 | 0 | 0 |
| DCT2 | 160 | 640 | 860 | 3420 | 13660 | 18740 |
| Summary (for each size) | 960 | 3840 | 4700 | 11100 | 29020 | 49620 |

Table 4: Estimated number of elements and bits for each transform matrix

Note that the band matrix associated with DCT-8 is a reflection of the band matrix of DCT-7, and therefore no additional memory is needed to store the band matrices.

## Specify the minimum bit-precision required by the transforms

This requirement is the same as in BMS.

## Specify if a transform requires multiple iterations

No additional iteration is required from BMS.

## Other operations and memory requirements relevant to the proposed tool

None.

## Encoder and decoder times

Listed in the experimental results in Sections 3 and 4.

## List of all combinations of transform and transforms block sizes used for primary transformation

This is listed in Section 1.

## Provide analysis of implementation and arithmetic commonalities of proposed transforms

In this CE, except DCT-2, all the transforms with size of 8x8, 16x16, 32x32, and 64x64 are implemented with matrix multiplication. For 4x4 transform, DCT-2, DST-7, DST-1, and DCT-8 are implemented with partial butterflies, while DCT-5, DST-4 and identity matrix are implemented with matrix multiplication. In Table 6, the implementation types described for each transform are summarized.

| Size | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- |
| DST7 |  | PB | MM | MM | MM | MM |
| DCT8 |  | PB | MM | MM | MM | MM |
| DST1 |  | PB | MM | MM | MM | MM |
| DCT5 |  | MM | MM | MM | MM | MM |
| DST4 |  | MM | MM | MM | MM | MM |
| Identity |  | MM | MM | MM | MM | MM |
| DCT2 | PB | PB | PB | PB | PB | PB |

Table 6: Analysis of implementation (PB: partial butterflies, MM: matrix multiplication)

In the additional test with approximation and simplification described in JVET-K0272, the pre-processing and post processing with adjustment stages in encoder and decoder are implemented with transform multiplication (only non-zero elements are involved). All types of DCT/DSTs of size 16x16, 32x32, and 64x64 are implemented with partial butterflies. For block size 8x8, matrix multiplication is used for DST-4, DST-1, DST-7, DCT-5, and DCT-8, while DCT-2 is implemented with partial butterflies. Same as for BMS, 4x4 transforms for DCT-2, DST-1, DST-7, and DCT-8 are implemented with partial butterflies. 4x4 DCT-5 and DST-4 applied in this contribution are implemented with matrix multiplication.  The multiplication of identity matrices is implemented by scaling each value in the input residual blocks. In Table 7, the implementation types described for each transform using method proposed in JVET-K0272 are summarized.

| Size | 2 | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- | --- |
| DST7 |  | PB | MM | PB | PB | PB |
| DCT8 |  | PB | MM | PB | PB | PB |
| DST1 |  | PB | MM | PB | PB | PB |
| DCT5 |  | MM | MM | PB | PB | PB |
| DST4 |  | MM | MM | PB | PB | PB |
| Identity |  | SC | SC | SC | SC | SC |
| DCT2 | PB | PB | PB | PB | PB | PB |

Table 7: Analysis of implementation (PB: partial butterflies, MM: matrix multiplication, SC: scaling)

# CE Results

The CE results of the proposed method tested on top of VTM and BMS configurations are presented in Tables 1.1 and 1.2, respectively.

Table 1.1: Results of the proposed AMT enabled on BMS1.1 with VTM configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.40% | -1.26% | -1.07% | 202% | 142% | 10.39% | 46.25% | 41.60% | 23% | 66% |
| Class A2 | -3.26% | -1.43% | -1.36% | 210% | 129% | 8.39% | 21.22% | 16.07% | 22% | 67% |
| Class B | -3.55% | -1.49% | -1.47% | 215% | 129% | 7.05% | 16.90% | 22.20% | 22% | 70% |
| Class C | -3.39% | -1.11% | -1.43% | 220% | 114% | 8.75% | 18.25% | 23.20% | 20% | 72% |
| Class E | -4.50% | -2.91% | -2.91% | 197% | 126% | 10.44% | 16.89% | 20.64% | 23% | 66% |
| Overall | -3.60% | -1.59% | -1.62% | 210% | 127% | 8.77% | 22.81% | 24.38% | 22% | 69% |
| Class D | -3.38% | -0.87% | -0.91% | 232% | 110% | 6.77% | 14.40% | 16.79% | 19% | 71% |
| Class F (optional) | -2.43% | -1.23% | -1.20% | 211% | 111% | 9.00% | 18.21% | 19.54% | 22% | 68% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.91% | 0.06% | 0.20% | 176% | 119% | 17.81% | 47.35% | 50.02% | 44% | 48% |
| Class A2 | -2.00% | 0.42% | 0.62% | 161% | 111% | 21.91% | 29.70% | 20.97% | 45% | 44% |
| Class B | -2.53% | -0.25% | -0.40% | 163% | 111% | 15.44% | 29.15% | 31.41% | 43% | 50% |
| Class C | -2.15% | 0.08% | -0.19% | 166% | 107% | 12.33% | 20.38% | 23.77% | 42% | 53% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.20% | 0.03% | -0.02% | 166% | 112% | 16.38% | 30.56% | 31.01% | 43% | 49% |
| Class D | -1.96% | 0.34% | 0.01% | 163% | 104% | 13.54% | 18.31% | 20.29% | 39% | 52% |
| Class F (optional) | -1.98% | -0.41% | -0.45% | 174% | 107% | 10.29% | 20.13% | 19.90% | 38% | 64% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.90% | 0.62% | 0.64% | 167% | 114% | 9.95% | 12.77% | 13.25% | 45% | 60% |
| Class C | -1.35% | 0.84% | 1.12% | 160% | 109% | 8.65% | 10.41% | 11.96% | 47% | 64% |
| Class E | -1.04% | 1.55% | 1.08% | 147% | 108% | 11.62% | 12.47% | 15.82% | 44% | 64% |
| Overall | -1.50% | 0.93% | 0.91% | 159% | 111% | 9.93% | 11.91% | 13.46% | 45% | 62% |
| Class D | -1.21% | 0.92% | 0.32% | 149% | 104% | 11.07% | 10.74% | 11.36% | 50% | 65% |
| Class F (optional) | -1.25% | -0.19% | -0.28% | 165% | 108% | 10.52% | 15.99% | 15.15% | 43% | 73% |

Table 1.2: Results of the proposed AMT enabled on BMS1.1 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -12.77% | -31.46% | -29.75% | 874% | 217% | -0.24% | 0.08% | -0.09% | 99% | 101% |
| Class A2 | -10.99% | -18.06% | -14.66% | 953% | 190% | -0.21% | 0.05% | -0.02% | 99% | 99% |
| Class B | -10.11% | -15.77% | -19.50% | 986% | 185% | -0.21% | -0.11% | -0.15% | 100% | 101% |
| Class C | -11.44% | -16.49% | -20.02% | 1086% | 158% | -0.42% | -0.42% | -0.43% | 101% | 100% |
| Class E | -13.98% | -17.25% | -20.03% | 830% | 188% | -0.48% | -0.42% | -0.61% | 98% | 99% |
| Overall | -11.64% | -19.17% | -20.61% | 954% | 185% | -0.31% | -0.17% | -0.26% | 100% | 100% |
| Class D | -9.97% | -13.61% | -15.58% | 1208% | 154% | -0.50% | -0.51% | -0.55% | 100% | 100% |
| Class F (optional) | -11.59% | -17.40% | -18.58% | 955% | 162% | -1.33% | -1.59% | -1.80% | 102% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -16.83% | -30.75% | -32.71% | 403% | 249% | -0.08% | 0.06% | -0.10% | 100% | 101% |
| Class A2 | -19.55% | -21.64% | -16.39% | 354% | 254% | -0.01% | 0.24% | 0.15% | 100% | 100% |
| Class B | -15.61% | -22.63% | -24.02% | 379% | 221% | -0.11% | 0.06% | 0.04% | 99% | 100% |
| Class C | -13.02% | -16.89% | -19.56% | 392% | 200% | -0.15% | -0.12% | -0.27% | 98% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -15.95% | -22.52% | -23.04% | 382% | 227% | -0.09% | 0.05% | -0.05% | 99% | 100% |
| Class D | -13.58% | -15.25% | -16.92% | 407% | 197% | -0.17% | -0.22% | -0.07% | 99% | 98% |
| Class F (optional) | -12.11% | -17.68% | -17.69% | 452% | 167% | -1.14% | -1.22% | -1.10% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.77% | -10.63% | -11.20% | 371% | 189% | -0.02% | 0.05% | -0.14% | 99% | 98% |
| Class C | -9.29% | -8.51% | -9.79% | 339% | 167% | -0.09% | 0.02% | -0.30% | 100% | 98% |
| Class E | -11.51% | -9.76% | -11.65% | 334% | 168% | -0.19% | -0.22% | 1.15% | 99% | 101% |
| Overall | -10.46% | -9.71% | -10.84% | 351% | 176% | -0.09% | -0.03% | 0.13% | 99% | 99% |
| Class D | -10.76% | -9.10% | -9.58% | 300% | 163% | -0.05% | -0.36% | 0.06% | 100% | 103% |
| Class F (optional) | -11.11% | -14.13% | -13.88% | 382% | 152% | -0.59% | -0.59% | -0.59% | 100% | 103% |

# Additional CE Results:TAS implementation of the proposed AMT scheme

The CE results of the TAS implementation of the proposed AMT tested on top of VTM and BMS configurations are presented in Tables 1.3 and 1.4, respectively.

Table 1.3: Results of the proposed AMT based on TAS enabled on BMS1.1 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -3.39% | -1.26% | -1.07% | 202% | 116% | 10.41% | 46.26% | 41.62% | 23% | 54% |
| Class A2 | -3.20% | -1.45% | -1.36% | 209% | 113% | 8.46% | 21.21% | 16.07% | 22% | 58% |
| Class B | -3.50% | -1.46% | -1.45% | 211% | 111% | 7.11% | 16.96% | 22.26% | 21% | 60% |
| Class C | -3.36% | -1.17% | -1.43% | 220% | 107% | 8.78% | 18.19% | 23.18% | 20% | 68% |
| Class E | -4.43% | -2.98% | -2.92% | 197% | 111% | 10.52% | 16.79% | 20.65% | 23% | 59% |
| Overall | -3.56% | -1.61% | -1.61% | 209% | 111% | 8.82% | 22.80% | 24.39% | 22% | 60% |
| Class D | -3.33% | -1.01% | -1.13% | 231% | 106% | 6.82% | 14.29% | 16.58% | 19% | 69% |
| Class F (optional) | -2.41% | -1.19% | -1.23% | 209% | 103% | 9.02% | 18.28% | 19.51% | 22% | 63% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.90% | 0.01% | 0.26% | 161% | 106% | 17.83% | 47.30% | 50.18% | 40% | 43% |
| Class A2 | -1.96% | 0.31% | 0.66% | 151% | 104% | 21.95% | 29.56% | 21.03% | 43% | 41% |
| Class B | -2.48% | -0.17% | -0.37% | 153% | 103% | 15.50% | 29.31% | 31.46% | 40% | 46% |
| Class C | -2.14% | 0.19% | -0.27% | 159% | 103% | 12.34% | 20.51% | 23.74% | 40% | 51% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.17% | 0.06% | -0.01% | 156% | 104% | 16.41% | 30.61% | 31.06% | 40% | 46% |
| Class D | -1.93% | 0.02% | 0.09% | 157% | 103% | 13.57% | 17.91% | 20.45% | 38% | 51% |
| Class F (optional) | -1.96% | -0.32% | -0.56% | 166% | 102% | 10.32% | 20.30% | 19.81% | 37% | 61% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -1.90% | 0.50% | 0.85% | 155% | 104% | 9.95% | 12.62% | 13.51% | 41% | 55% |
| Class C | -1.36% | 0.74% | 0.90% | 153% | 106% | 8.64% | 10.31% | 11.74% | 45% | 62% |
| Class E | -1.13% | 1.31% | 1.74% | 139% | 105% | 11.52% | 12.14% | 16.61% | 41% | 63% |
| Overall | -1.53% | 0.78% | 1.09% | 150% | 105% | 9.91% | 11.73% | 13.69% | 43% | 59% |
| Class D | -1.22% | 0.51% | 0.55% | 145% | 102% | 11.06% | 10.30% | 11.72% | 48% | 64% |
| Class F (optional) | -1.36% | -0.20% | -0.17% | 156% | 102% | 10.39% | 15.96% | 15.31% | 41% | 69% |

Table 1.4: Results of the proposed AMT based on TAS enabled on BMS1.1 with BMS configuration against (left) BMS 1.1 with VTM configuration and (right) BMS1.1 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -12.77% | -31.49% | -29.71% | 867% | 193% | -0.24% | 0.06% | -0.01% | 98% | 89% |
| Class A2 | -10.96% | -18.13% | -14.68% | 926% | 173% | -0.18% | -0.04% | -0.05% | 97% | 89% |
| Class B | -10.09% | -15.70% | -19.43% | 967% | 168% | -0.19% | -0.02% | -0.06% | 98% | 92% |
| Class C | -11.44% | -16.41% | -19.96% | 1074% | 155% | -0.43% | -0.31% | -0.34% | 100% | 98% |
| Class E | -14.00% | -17.20% | -19.92% | 832% | 180% | -0.49% | -0.36% | -0.50% | 99% | 95% |
| Overall | -11.63% | -19.14% | -20.55% | 941% | 172% | -0.30% | -0.13% | -0.19% | 98% | 93% |
| Class D | -9.94% | -13.49% | -15.56% | 1207% | 151% | -0.48% | -0.34% | -0.55% | 100% | 98% |
| Class F (optional) | -11.61% | -17.40% | -18.63% | 950% | 158% | -1.35% | -1.59% | -1.89% | 101% | 96% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -16.83% | -30.71% | -32.65% | 377% | 234% | -0.09% | 0.12% | -0.01% | 93% | 95% |
| Class A2 | -19.53% | -21.68% | -16.36% | 336% | 248% | 0.02% | 0.20% | 0.19% | 95% | 98% |
| Class B | -15.59% | -22.72% | -23.99% | 364% | 217% | -0.08% | -0.04% | 0.07% | 95% | 98% |
| Class C | -13.06% | -16.81% | -19.58% | 385% | 199% | -0.19% | -0.04% | -0.28% | 96% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -15.95% | -22.53% | -23.02% | 366% | 221% | -0.09% | 0.04% | -0.02% | 95% | 97% |
| Class D | -13.64% | -14.98% | -17.00% | 405% | 201% | -0.24% | 0.07% | -0.17% | 98% | 100% |
| Class F (optional) | -12.13% | -17.74% | -17.76% | 444% | 165% | -1.17% | -1.29% | -1.22% | 98% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.1 + vtm cfg. |  |  |  |  | Over BMS-1.1 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.76% | -10.64% | -11.24% | 351% | 180% | -0.01% | 0.05% | -0.19% | 94% | 94% |
| Class C | -9.31% | -8.50% | -9.68% | 328% | 162% | -0.10% | 0.04% | -0.19% | 96% | 96% |
| Class E | -11.49% | -8.90% | -11.90% | 320% | 164% | -0.18% | 0.71% | 0.85% | 95% | 98% |
| Overall | -10.46% | -9.49% | -10.89% | 335% | 170% | -0.08% | 0.21% | 0.07% | 95% | 96% |
| Class D | -10.74% | -8.87% | -9.90% | 293% | 157% | -0.03% | -0.11% | -0.31% | 97% | 99% |
| Class F (optional) | -11.13% | -14.25% | -14.57% | 366% | 145% | -0.61% | -0.72% | -1.33% | 96% | 98% |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-J1026, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, H. Egilmez, V. Seregin, and M. Karczewicz, “Complexity Reduction for Adaptive Multiple Transforms (AMT) using Adjustment Stages”, JVET-J0066, 9th meeting, San Diego, CA, USA, April 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).