| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0376-v1 |
| --- | --- |

| Title: | CE1-2.0.9: Picture Boundary Handling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li Dong Liu | Tel: Email: | +86-181-5651-7881 lytt@mail.ustc.edu.cn dongeliu@ustc.edu.cn |
| Source: | University of Science and Technology of China (USTC) |  |  |

_____________________________

- Abstract

A picture boundary handling method is proposed. In the proposed method, QT and BT partition will used in boundary located CU. Compare to VTM-1.0, the proposed method in CE1 achieves overall -0.05%, -0.31%, and -0.52% luma BD rate saving, respectively for AI, RA and LDB configurations with 101%, 102% and 103% encoding time. Compare to BMS-1.0, the proposed method have overall -0.05%, -0.36%, and -0.68% luma BD rate saving, respectively for AI, RA and LDB configurations, with 101%, 103% and 103% encoding time.

Introduction

The proposal presents a new method of picture boundary handling. The proposed method use QT and BT partition in the boundary located CTU/CU. Horizontal BT and QT are selected for bottom boundary, vertical BT and QT are selected for right boundary, and for corner case, only QT will be used for boundary partition.

Proposed Method

The boundary partition of proposed method in CE1-2.0.9 is descripted as follow:

Fig. 1 signaling tree for boundary partition

- If the size of block is larger than the maximal allowed BT size, only QT is used to perform the boundary partition in the current boundary partition level;

- Otherwise, if the bottom-right sample of current CU is located below the bottom picture boundary, and not extended the right boundary, horizontal BT and QT will be selected based on RD cost for the current boundary partition level, one flag will be signaled as in Fig 1;

- Otherwise, if the bottom-right sample of current CU is located at the right side of the right picture boundary, and not below the bottom boundary, vertical BT and QT will be selected based on RD cost for the current boundary partition level, one flag will be signaled as in Fig 1;

- Otherwise, if the bottom-right sample of current CU is located at the right side of the right picture boundary and below the bottom boundary, only QT is used to perform the partition in the current boundary partition level.

Experiment results of proposed method in CE1

The proposed method is implemented on top of the BMS 1.0 software. The experiment results are to be produced under the common test conditions. Further, the following two configurations are to be used for the experiments, as mention in JVET-J1021 [2]:

- BMS1.0:
VVC software with Benchmark set (BMS) configuration as provided in the repository.

- VTM1.0:
Same software as in BMS1.0 configuration, but with disabling the BMS tools at config level.

Table 1: Experiment results of proposed method in CE1 on top of BMS1.0

Table 2: Experiment results of proposed method in CE1 on top of VTM1.0

Conclusion

A boundary partition method in CE1 is proposed to achieve better performance of the picture boundary handling. Experiment results report that the proposed method saves BD-rate overall  -0.05%, -0.31%, and -0.52% of luma component for AI, RA and LDB configuration compare to VTM anchor; and Compare to BMS anchor, the proposed method saves BD-rate overall -0.05%, -0.36%, and -0.68% of luma component for AI, RA and LDB configuration.

Reference

[1] “Versatile Video Coding (Draft 1)”, B. Bross, doc. JVET-J1001, April 2018

[2] “Description of Core Experiment 1 (CE1): Partitioning”, J. Ma, F. Le Léannec, M. W. Park, doc. JVET-J1021, April 2018

Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).