| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0377-v2 |
| --- | --- |

| Title: | CE3-related: Combining CE3-5.2.5 on using two rows of reference lines for prediction, CE3-2.4.1 on PDPC and 65 angular intra modes |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Steve Keating Karl Sharman Geert Van Der Auwera Adarsh K. Ramasubramonian Vadim Seregin Amir Said Marta Karczewicz | Email: | steve.keating@sony.com karl.sharman@sony.com geertv@qti.qualcomm.com aramasub@qti.qualcomm.com vseregin@qti.qualcomm.com asaid@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Sony Europe, Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes combining tools CE3-2.4.1 (simplified PDPC), CE3-5.2.5 (using two rows of ref. samples for intra prediction) and 65 intra angular modes.  The results are reported as being additive with a BD-rate improvement of -2.59% (Y), -1.75% (U), -1.62% (V) for AI over VTM1.0 with run-times of 134% (Enc) and 114% (Dec).

An additional proposal is to also include the merging of the intra reference sample smoothing filter with the intra interpolation filter as proposed in JVET-K0064. The results for this are reported to give a BD-rate improvement of -3.35% (Y), -2.03% (U), -1.92% (V) for AI with run-times of 149% (Enc) and 121% (Dec).

# Introduction

This document proposes combining tools CE3-5.2.5 (using two rows of reference samples for intra prediction), CE3-2.4.1 (simplified PDPC) and 65 intra angular modes. It additionally proposes combining tools CE3-5.2.5 with simplified PDPC and merging of intra reference sample smoothing with the intra interpolation filter (JVET-K0064) and 65 intra angular modes.

The results for each are presented below.

# Results –VTM All Intra

Combining CE3-2.4.1 and CE3-5.2.5:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.33% | -0.85% | -0.54% | 121% | 116% |
| Class A2 | -1.23% | -0.93% | -0.57% | 120% | 115% |
| Class B | -1.44% | -0.20% | -0.13% | 115% | 112% |
| Class C | -1.20% | 0.25% | 0.20% | 117% | 110% |
| Class E | -1.73% | -0.88% | -0.86% | 115% | 110% |
| Overall | -1.38% | -0.44% | -0.32% | 117% | 112% |
| Class D | -1.02% | 0.45% | 0.87% | 115% | 108% |
| Class F (optional) | 0.91% | 2.08% | 2.90% | 116% | 111% |

Combining CE3-2.4.1 and CE3-5.2.5 with 65 angular modes:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.04% | -1.71% | -1.36% | 141% | 120% |
| Class A2 | -2.29% | -2.13% | -1.72% | 134% | 113% |
| Class B | -2.81% | -1.63% | -1.71% | 133% | 114% |
| Class C | -2.24% | -0.60% | -0.72% | 133% | 111% |
| Class E | -3.52% | -3.15% | -2.86% | 132% | 111% |
| Overall | -2.59% | -1.75% | -1.62% | 134% | 114% |
| Class D | -1.36% | 0.19% | 0.60% | 132% | 112% |
| Class F (optional) | -0.17% | 1.26% | 1.95% | 133% | 113% |

Combining JVET-K0064, CE3-2.4.1 and CE3-5.2.5 with 65 angular modes:

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -2.10% | -1.80% | -1.33% | 151% | 122% |
| Class A2 | -2.45% | -1.92% | -1.53% | 149% | 120% |
| Class B | -3.46% | -1.89% | -1.90% | 150% | 122% |
| Class C | -3.59% | -1.21% | -1.35% | 145% | 117% |
| Class E | -4.98% | -3.73% | -3.67% | 152% | 124% |
| Overall | -3.35% | -2.03% | -1.92% | 149% | 121% |
| Class D | -2.25% | -0.10% | 0.27% | 144% | 122% |
| Class F (optional) | -0.65% | 1.12% | 1.77% | 149% | 119% |

# Comment on results

The gains for the three tools CE3-2.4.1, CE3-5.2.5 and 65 angular modes appear to be more or less additive with a BD-rate improvement of -2.59% (Y), -1.75% (U), -1.62% (V) with run-times of 134% (Enc) and 114% (Dec).

Including the filter merging (JVET-K0064) increases the BD-rate improvement to -3.35% (Y), -2.03% (U), -1.92% (V) with run-times of 149% (Enc) and 121% (Dec).

This implementation of CE3-5.2.5 is without the additional speed-up described in section 4.2 of JVET-K0221.

# Patent rights declarations

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).