| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0378-v1 |
| --- | --- |

| Title: | CE9-Test 2.8: DMVR with bilateral matching and 2 half-pel points search |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li Dong Liu | Email: | lytt@mail.ustc.edu.cn dongeliu@ustc.edu.cn |
| Source: | University of Science and Technology of China (USTC) |  |  |

_____________________________

- Abstract

As part of the CE9 core experiment a decoder side motion vector refinement method is proposed that uses bilateral matching cost function. The proposal is based on the method described in CE 9.2.9. The only difference is application of 2-point half-pel search instead of 4 points.

According to one of the configurations of the proposal (with search range of 2 samples and maximum 11 MRSAD computations), -4.48% luma coding gain with 109%, encoding time and 125% decoding time; over VTM Anchor (RA), and -1.22% luma coding gain with 100%, encoding time and 98% decoding time over BMS Anchor (RA) is achieved.

Background

USTC and Huawei proposed a joint proposal in CE9.2.9. In the proposal the search operation is performed using 6 points in the integer search iterations and 4 points in the half-pel iteration.

The number of SAD search operations directly affect the decoding time and hence the complexity. Therefore in this proposal an alternative half-pel search operation is proposed.

Proposed Method

In the proposed method the proposed method in CE9.2.9 is used with one exception. The half-pel search operation is of CE9.2.9 is replaced with a 2 point search operation which is described below:

Fig. 1: two points half sample search pattern

Half sample precision search is applied only if application of half-pel search does not exceed the search range. In this case only 2 MRSAD calculations are performed, corresponding to points around the central one, which is chosen as the best during the integer precision search. The location of 2 half-pel points is chosen by the gradient of costs, obtained during integer pel search. (Fig. 1). At the end the refined motion vector pair is output that correspond to the minimal cost point.

Experiment results

Results for 2 sample search range

The table below show the experimental results for application of the proposed method, using 2 integer iterations and 1 half-pel iteration. According to the proposal a total of maximum 11 MRSAD computations are performed (9 integer and 2 half-pel point MRSAD calculations).

Table 1: Result for 2 integer iterations and one half pel iteration.

Results for 1 sample search range

The table below show the experimental results for application of the proposed method, using 1 integer iterations and 1 half-pel iteration. According to the proposal a total of maximum 8 MRSAD computations are performed (6 integer and 2 half-pel point MRSAD calculations).

Table 2: Result for 1 integer iterations and one half pel iteration.

Comparison to CE9.2.9

The proposed method achieves 5% and 7% lower decoding time compared to the 4-point half-pel search method proposed in CE9.2.9. The tables below compare the results reported for CE9.2.8 and CE9.2.9.

| VTM |  |  |  |  | BMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Max # of MRSAD calculation | Max. Search Range | Half Pel |
| -3.68% | -3.17% | -3.33% | 106% | 124% | -0.68% | -0.48% | -0.52% | 98% | 98% | 10 | 1 sample | On |
| -4.59% | -3.97% | -4.14% | 107% | 132% | -1.29% | -1.02% | -1.10% | 98% | 101% | 13 | 2 samples | On |

Table 3: The results reported for the proposal CE9.2.9

| VTM |  |  |  |  | BMS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Max # of MRSAD calculation | Max. Search Range | Half Pel |
| -3.61% | -3.15% | -3.27% | 107% | 119% | -0.64% | -0.46% | -0.54% | 99% | 97% | 8 | 1 sample | On |
| -4.48% | -3.92% | -4.11% | 109% | 125% | -1.22% | -0.98% | -1.05% | 100% | 98% | 11 | 2 samples | On |

Table 4: The results reported for this. The decoding time is reduced 5% for 1 integer iteration case whereas it is reduced 7% for 2 integer iteration case.

Conclusion

A bilateral matching based DMVR method using adaptive search pattern and 2 points half-pel sample precision search in CE9.2.8 is proposed to achieve better performance and less complexity in DMVR process.

According to one of the configurations of the proposal (with search range of 2 pixel samples and maximum 13 MRSAD computations), -4.48% luma coding gain with 109%, encoding time and 125% decoding time; over VTM Anchor (RA), and -1.22% luma coding gain with 100%, encoding time and 98% decoding time over BMS Anchor (RA) is achieved. Compared to the 4 point half-pel search that is presented in CE9.2.9, 7% and 3% decoding time reduction is achieved according to VTM and BMS configurations respectively. It is proposed to be adopted in the VVC.

References

[1] JVET common test conditions and software reference configurations, Jill Boyce, doc. JVET-J1010-v1, April 2018

[2] Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation, Semih Esenlik, doc. JVET-J1021, April 2018

Patent rights declaration(s)

University of Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).