| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0379-v1 |
| --- | --- |

| Title: | CE5: CABAC probability initialization from previous inter frames (test C1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi Egilmez, Yung-Hsuan Chao, Marta Karczewicz, Vadim Seregin 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CABAC probability initialization from previous inter coded (B and P) slices, where the same method in JEM7 is proposed. The CE results show that

- On top of VTM 1.1, the proposed method provides 0.1% BD-rate gains for luma under RA and LD configurations, respectively.

- On top of BMS 1.1, the proposed method provides 0.2% and 0.4% BD-rate gains for luma under RA and LD configurations, respectively.

# CE Description

In JVET-J0021, for inter B and P slices, probability initialization can be made based on a previous B or P slice by using the probability estimate obtained after coding certain number of CTUs. As in JEM7, JVET-J0021 proposes a location-based method where the probability estimate is obtained after coding CTUs up to a pre-defined position in the slice. Specifically, the following formula is used to determine the number of CTUs in B/P slices coded:

CTU location = min ((number of CTUs in a CTU line + number of CTUs in  a frame)/2 + 1, number    of CTUs in  a frame).

# CE Results

The CE results of the proposed method tested on top of VTM and BMS configurations are presented in Tables 1.1 and 1.2, respectively.

Table 1.1: Comparison of the proposed method enabled on BMS1.0 with VTM configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% | 14.39% | 47.79% | 42.98% | 11% | 46% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 101% | 12.14% | 22.97% | 17.65% | 11% | 53% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 98% | 11.01% | 18.71% | 24.11% | 10% | 54% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 12.61% | 19.62% | 25.05% | 9% | 64% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% | 15.70% | 20.36% | 24.30% | 12% | 54% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 12.90% | 24.74% | 26.42% | 11% | 54% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% | 10.53% | 15.46% | 17.92% | 8% | 66% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 102% | 102% | 11.76% | 19.66% | 21.00% | 11% | 62% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.11% | -0.07% | 101% | 100% | 20.05% | 46.71% | 49.52% | 25% | 41% |
| Class A2 | -0.04% | -0.09% | -0.06% | 101% | 99% | 24.36% | 28.86% | 20.13% | 29% | 40% |
| Class B | -0.11% | -0.02% | 0.06% | 101% | 100% | 18.36% | 29.51% | 32.06% | 26% | 45% |
| Class C | -0.16% | -0.04% | -0.06% | 100% | 99% | 14.67% | 20.28% | 24.00% | 25% | 50% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.11% | -0.05% | -0.02% | 101% | 100% | 18.91% | 30.36% | 31.02% | 26% | 44% |
| Class D | -0.36% | -0.01% | 0.02% | 100% | 100% | 15.43% | 17.88% | 20.42% | 24% | 49% |
| Class F (optional) | -0.10% | 0.00% | -0.05% | 102% | 103% | 12.47% | 20.58% | 20.39% | 23% | 61% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.09% | -0.42% | 0.07% | 99% | 102% | 12.02% | 11.35% | 12.95% | 27% | 53% |
| Class C | -0.09% | -0.16% | -0.16% | 100% | 101% | 10.08% | 9.37% | 10.53% | 30% | 60% |
| Class E | -0.09% | 0.04% | -0.04% | 100% | 101% | 12.63% | 10.57% | 13.89% | 30% | 61% |
| Overall | -0.09% | -0.22% | -0.03% | 99% | 101% | 11.53% | 10.49% | 12.38% | 28% | 57% |
| Class D | -0.44% | -0.77% | -0.69% | 99% | 98% | 11.96% | 9.36% | 10.05% | 33% | 62% |
| Class F (optional) | -0.18% | -0.22% | -0.16% | 103% | 104% | 11.92% | 15.77% | 15.82% | 27% | 68% |

Table 1.2: Comparison of the proposed method enabled on BMS1.0 with BMS configuration against (left) BMS 1.0 with VTM configuration and (right) BMS1.0 with BMS configuration.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -12.56% | -31.49% | -29.68% | 878% | 212% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | -10.81% | -18.06% | -14.63% | 956% | 192% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class B | -9.91% | -15.68% | -19.37% | 986% | 185% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class C | -11.05% | -16.14% | -19.67% | 1071% | 156% | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E | -13.57% | -16.91% | -19.53% | 832% | 186% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Overall | -11.37% | -19.02% | -20.39% | 953% | 183% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | -9.50% | -13.18% | -15.09% | 1199% | 152% | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F (optional) | -10.39% | -16.05% | -17.08% | 938% | 167% | 0.00% | 0.00% | 0.00% | 101% | 102% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -16.86% | -30.91% | -32.81% | 401% | 242% | -0.11% | -0.25% | -0.24% | 101% | 99% |
| Class A2 | -19.61% | -21.90% | -16.61% | 351% | 249% | -0.07% | -0.19% | -0.12% | 103% | 102% |
| Class B | -15.68% | -22.78% | -24.22% | 389% | 226% | -0.18% | -0.12% | -0.23% | 101% | 101% |
| Class C | -13.14% | -17.02% | -19.63% | 395% | 199% | -0.26% | -0.28% | -0.36% | 100% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -16.03% | -22.69% | -23.19% | 385% | 226% | -0.17% | -0.20% | -0.24% | 101% | 100% |
| Class D | -13.86% | -15.39% | -17.20% | 414% | 203% | -0.48% | -0.41% | -0.38% | 100% | 100% |
| Class F (optional) | -11.25% | -16.79% | -16.89% | 460% | 172% | -0.18% | -0.16% | -0.16% | 102% | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over BMS-1.0 + vtm cfg. |  |  |  |  | Over BMS-1.0 + bms cfg. |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -10.88% | -11.31% | -11.39% | 373% | 189% | -0.15% | -0.93% | -0.09% | 101% | 98% |
| Class C | -9.48% | -8.73% | -9.88% | 339% | 167% | -0.28% | -0.19% | -0.44% | 100% | 99% |
| Class E | -12.08% | -8.59% | -12.38% | 333% | 169% | -0.91% | 0.86% | -0.25% | 100% | 102% |
| Overall | -10.71% | -9.77% | -11.14% | 351% | 176% | -0.39% | -0.24% | -0.25% | 100% | 99% |
| Class D | -11.21% | -9.63% | -9.89% | 301% | 160% | -0.60% | -0.57% | -0.53% | 101% | 101% |
| Class F (optional) | -10.87% | -13.65% | -13.68% | 389% | 157% | -0.19% | -0.12% | -0.06% | 102% | 104% |

# References

- J. Chen and E. Alshina, “Algorithm description for Versatile Video Coding and Test Model 1 (VTM1),” JVET-J1002, 9th meeting, San Diego, CA, USA, April 2018.

- T. Nguyen and A. Said, “Core Experiment 5: Arithmetic Coding Engine,” JVET-J1025, 9th meeting, San Diego, CA, USA, April 2018.

- J.Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, 9th meeting, San Diego, CA, USA, April 2018.

- A. Said, M. Karczewicz, H. Egilmez, V. Seregin, L. Zhang, and X. Zhao, “EE2: Arithmetic coding with context-dependent double-window adaptation response,” JVET-H0061, 8th Meeting, Macao, China, Oct. 2017.

- A. Said, M. Karczewicz, L. Zhang, V. Seregin, and X. Zhao, “Arithmetic coding with context-dependent double-window adaptation response,” JVET-G0112, 7th Meeting, Torino, Italy, July 2017.

- A. Alshin, E. Alshina, and J.-H. Park, “High precision probability estimation for CABAC,” in Proc. IEEE Visual Commun. Image Process. Conf., Kuching, Malaysia, Nov. 2013.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).